# CUSC - SECTION 14

CHARGING METHODOLOGIES

CONTENTS

14.1	Introduction

Part I -The Statement of the Connection Charging Methodology

14.2	Principles

14.3	The Calculation of the Basic Annual Connection Charge for an Asset

14.4 	Other Charges

14.5 	Connection Agreements

14.6	Termination Charges

14.7	Contestability

14.8	Asset Replacement

14.9	Data Requirements

14.10	Applications

14.11	Illustrative Connection Charges

14.12	Examples of Connection Charge Calculations

14.13	Nominally Over Equipped Connection Sites

Part 2 -The Statement of the Use of System Charging Methodology

Section 1 – The Statement of the Transmission Use of System Charging 	Methodology;

14.14	Principles

14.15	Derivation of Transmission Network Use of System Tariff

14.16	Derivation of the Transmission Network Use of System Energy Consumption Tariff and Short Term Capacity Tariffs

14.17	Demand Charges

14.18	Generation Charges

14.19	Data Requirements

14.20	Applications

14.21	Transport Model Example

14.22 	Illustrative Calculation of Boundary Sharing Factors (BSFs) and Shared / Not-Shared incremental km

14.23	Example: Calculation of Zonal Generation Tariffs and Charges

14.24	Example: Calculation of Zonal Demand Tariff

14.25	Reconciliation of Demand Related Transmission Network Use of System Charges

14.26	Classification of parties for charging purposes

14.27	Transmission Network Use of System Charging Flowcharts

14.28	Example: Determination of The Company’s Forecast for Demand Charge Purposes

14.29	Stability & Predictability of TNUoS tariffs

Section 2 – The Statement of the Balancing Services Use of System Charging Methodology

14.30	Principles

14.31	Calculation of the Daily Balancing Services Use of System Charges

14.32	Settlement of BSUoS

Schedule 1 - Calculation of charges that fall within the Connection Exclusion EU Regulation 838/2010

# CUSC - SECTION 14

CHARGING METHODOLOGIES

Introduction

This section of the CUSC sets out the statement of the Connection Charging Methodology and the Statement of the Use of System Methodology

Part 1 - The Statement of the Connection Charging Methodology

# 14.2 Principles

## Costs and their Allocation

- Connection charges enable The Company to recover, with a reasonable rate of return, the costs involved in providing the assets that afford connection to the National Electricity Transmission System.

- Connection charges relate to the costs of assets installed solely for and only capable of use by an individual User.  These costs may include civil costs, engineering costs, and land clearance and preparation costs associated with the connection assets, but for the avoidance of doubt no land purchase costs will be included.

- Connection charges are designed not to discriminate between Users or classes of User.  The methodology is applied to both connections that were in existence at Vesting (30 March 1990) and those that have been provided since.

## Connection/Use of System Boundary

- The first step in setting charges is to define the boundary between connection assets and transmission system infrastructure assets.

- In general, connection assets are defined as those assets solely required to connect an individual User to the National Electricity Transmission System, which are not and would not normally be used by any other connected party (i.e. “single user assets”).  For the purposes of this Statement, all connection assets at a given location shall together form a connection site.

- Connection assets are defined as all those single user assets which:

- for Double Busbar type connections, are those single user assets connecting the User’s assets and the first  transmission licensee owned substation, up to and including the Double Busbar Bay;

- for teed or mesh connections, are those single user assets from the User’s assets up to, but not including, the HV disconnector or the equivalent point of isolation;

- for cable and overhead lines at a transmission voltage, are those single user connection circuits connected at a transmission voltage equal to or less than 2km in length that are not potentially shareable.

- Shared assets at a banked connection arrangement will not normally be classed as connection assets except where both legs of the banking are single user assets under the same Bilateral Connection Agreement.

- Where customer choice influences the application of standard rules to the connection boundary, affected assets will be classed as connection assets.  For example, in England & Wales NGET does not normally own busbars below 275kV, where The Company and the customer agree that NGET will own the busbars at a low voltage substation, the assets at that substation will be classed as connection assets and will not automatically be transferred into infrastructure.

- The design of some connection sites may not be compatible with the basic boundary definitions in 14.2.6 above.  In these instances, a connection boundary consistent with the principles described above will be applied.

# 14.3 The Calculation of the Basic Annual Connection Charge for an Asset

## Pre and Post Vesting Connections

- Post Vesting connection assets are those connection assets that have been commissioned since 30 March 1990.  Pre Vesting connection assets are those that were commissioned on or before the 30 March 1990.

- The basic connection charge has two components.  A non-capital component, for which both pre and post vesting assets are treated in the same way and a capital component for which there are slightly different options available for pre and post vesting assets.  These are detailed below.

## Calculation of the Gross Asset Value (GAV)

- The GAV represents the initial total cost of an asset to the transmission licensee.  For a new asset it will be the costs incurred by the transmission licensee in the provision of that asset.  Typically, the GAV is made up of the following components:

Construction Costs - Costs of bought in services

Engineering - Allocated equipment and direct engineering cost

Interest During Construction – Financing cost

Liquidated Damages Premiums - Premium required to cover Liquidated Damages if applicable.

Some of these elements may be optional at the User’s request and are a matter of discussion and agreement at the time the connection agreement is entered into.

- The GAV of an asset is re-valued each year normally using one of two methods.  For ease of calculation, April is used as the base month.

- In the Modern Equivalent Asset (MEA) revaluation method, the GAV is indexed each year with reference to the prevailing price level for an asset that performs the same function as the original asset;

- In the Transmission Owner Price Index (TOPI) revaluation method, the original cost of an asset is indexed each year by the TOPI formula set out in paragraph 14.3.6.  For Pre Vesting connection assets commissioned on or before 30 March 1990, the original cost is the 1996/97 charging GAV (MEA re-valued from vesting). The original costs of Post Vesting assets are calculated based on historical cost information provided by the transmission licensee’s.

- In the MEA revaluation method, the MEA value is based on a typical asset.  An MEA ratio is calculated to account for specific site conditions, as follows:

- The outturn GAV (as calculated in paragraph 14.3.4 above) is re-indexed by  TOPI to the April of the Financial Year the Charging Date falls within;

- This April figure is compared with the MEA value of the asset in the Financial Year the Charging Date falls within and a ratio calculated;

- If the asset was commissioned at a Connection Site where, due to specific conditions, the asset cost more than the standard MEA value, the ratio would be greater than 1. For example, if an asset cost 10% more to construct and commission than the typical asset the MEA ratio would be 1.1.  If, however, the asset was found only to cost 90% of the typical MEA value the ratio would be 0.9;

- The MEA ratio is then used in all future revaluations of the asset.  The April GAV of the asset in any year is thus the current MEA value of the asset multiplied by the ratio calculated for the Financial Year the Charging Date falls within.

- The TOPI revaluation method is as follows:

- The outturn GAV (as calculated in paragraph 14.3.4 above) is re-indexed by TOPI to the April of the Financial Year the Charging Date falls within. This April GAV is thus known as the Base Amount;

- The Base Amount GAV is then indexed to the following April by using the TOPI  formula used in  the Transmission Owner’s Price Control.  April GAVs for subsequent Financial years are found using the same process of indexing by TOPI.

i.e. GAVn = GAVn-1 * TOPIn

- TOPI calculation for year n is as follows:

## Calculation of Net Asset Value

- The Net Asset Value (NAV) of each asset for year n, used for charge calculation, is the average (mid year) depreciated GAV of the asset.  The following formula calculates the NAV of an asset, where An is the age of the asset (number of completed Financial Years old) in year n:

- In constant price terms an asset with an initial GAV of £1m and a depreciation period of 40 years will normally have a NAV in the year of its commissioning of £0.9875m (i.e. a reduction of 1.25%) and in its second year of £0.9625m (i.e. a further reduction of 2.5% or one fortieth of the initial GAV).  This process will continue with an annual reduction of 2.5% for each year of the asset's life.

## Capital Components of the Connection charge for Post Vesting Connection Assets

- The standard terms for a connection offer will be:

- 40 year life (with straight line depreciation);

- TOPI indexation

- In addition a number of options exist:

- a capital contribution based on the allocated GAV at the time of commissioning will reduce capital. Typically a capital contribution made in advance of or at the time of commissioning  will include costs to cover the elements outlined below and charges are calculated as set out in the equations below;

- Construction costs

- Engineering costs (Engineering Charge x job hours)

- Interest During Construction (IDC)

- Return element (6%)

- Liquidated Damages Premium (LD) (if applicable)

General Formula:

Capital Contribution Charge = (Construction Costs + Engineering Charges) x (1+Return %) + IDC + LD Premium

- The MEA and TOPI revaluation methods are described further in 14.3.21. As an example, we will assume MEA revaluation is  a 7.5% rate of return, AND 6% on the TOPI revaluation basis;

- annual charges based on depreciation periods other than 40 years;

- annuity based charging;

- indexation of GAVs based on principles other than MEA revaluation and TOPI indexation.  No alternative forms of indexation have been employed to date.

- For new connection assets, should a User wish to agree to one or more of the options detailed above, instead of the standard connection terms, the return elements charged by the transmission licensee may also vary to reflect the re-balancing of risk between the transmission licensee and the User.  For example, if Users choose a different indexation method, an appropriate rate of return for such indexation method will be derived.

- A User can choose to make a capital contribution based on the allocated and depreciated NAV of a commissioned asset. For a capital contribution to take account at the start of Financial Year n, the User may, at most once per year, make a full or partial capital contribution of at least 10% of the NAV prevailing as of 31st March in year n-1. The User shall notify The Company of the capital contribution amount no later than 1st September in year n-1, and pay the capital contribution 45 days prior to the start of Financial Year n which will be applied to the NAV prevailing at the start of year n. As the capital component of the connection charge for year n will reduce as a result of the capital contribution, a reduced rate of return element will be payable and a lower security requirement will be required in Financial Year n and subsequent years.

## Capital Components of the Connection charge for Pre Vesting Connection Assets

- The basis of connection charges for GB assets commissioned on or before 30 March 1990 is broadly the same as the standard terms for connections made since 30 March 1990.  Specifically charges for pre vesting connection assets are based on the following principles:

- The GAV is the 1996/97 charging GAV (MEA re-valued from vesting) subsequently indexed by the same measure of TOPI as used in the Transmission Owner’s Price Control;

- 40 year life (with straight line depreciation);

- 6% rate of return

- Pre-vesting 1996 MEA GAVs for Users’ connection sites are available from The Company on request from the Charging Team.

## Non-Capital Components - Charging for Maintenance and Transmission Running Costs

- The non-capital component of the connection charge is divided into two parts, as set out below.  Both of these non-capital elements will normally be identified in the charging appendices of relevant Bilateral Agreements.

## Part A: Site Specific Maintenance Charges

- This is a maintenance only component that recovers a proportion of the costs and overheads associated with the maintenance activities conducted on a site-specific basis for connection assets of the transmission licensees.

- Site-specific maintenance charges will be calculated each year based on the forecast total site specific maintenance for NETS divided by the total GAV of the transmission licensees NETS connection assets, to arrive at a percentage of total GAV.  For 2010/11 this will be 0.52%.  For the avoidance of doubt, there will be no reconciliation of the site-specific maintenance charge.

## Part B: Transmission Running Costs

- The Transmission Running Cost (TRC) factor is calculated at the beginning of each price control to reflect the appropriate amount of other Transmission Running Costs (rates, operation, indirect overheads) incurred by the transmission licensees that should be attributed to connection assets.

- The TRC factor is calculated by taking a proportion of the forecast Transmission Running Costs for the transmission licensees (based on operational expenditure figures from the latest price control) that corresponds with the proportion of the transmission licensees’ total connection assets as a function of their total business GAV.  This cost factor is therefore expressed as a percentage of an asset's GAV and will be fixed for the entirety of the price control period.  The currently applicable TRC factor, calculated as above, is detailed in the Statement of Use of System Charges which is available from the Website.

- To illustrate the calculation, the following example uses the average operating expenditure from the published price control and the connection assets of each transmission licensee expressed as a percentage of their total system GAV to arrive at the 2010/11 GB TRC value of 1.45%:

Example:

Connection assets as a percentage of total system GAV for each TO:

| Scottish Power Transmission Ltd | 15.1% |
| --- | --- |
| Scottish Hydro Transmission Ltd | 8.6% |
| NGET | 12.5% |

Published current price control average annual operating expenditure (£m):

| Scottish Power Transmission Ltd | 29.1 |
| --- | --- |
| Scottish Hydro Transmission Ltd | 11.3 |
| NGET | 295.2 |

Total GB Connection GAV = £2.12bn

GB TRC Factor = (15.1% x £29.1m + 8.6% x £11.3m + 12.5% x £295.2m) / £2.12bn

GB TRC Factor = 1.99%

Net GB TRC Factor = Gross GB TRC Factor – Site Specific Maintenance Factor*

Net GB TRC Factor = 1.99% - 0.54% = 1.45%

* Note – the Site Specific Maintenance Factor used to calculate the TRC Factor is that which applies for the first year of the price control period or in this example, is the 2007/8 Site Specific Maintenance Factor of 0.54%.

## The Basic Annual Connection Charge Formula

- The charge for each connection asset in Financial Year n can be derived from the general formula below. This is illustrated more fully by the examples in Appendix 2: Examples of Connection Charge Calculations.

Annual Connection Chargen = Dn (GAVn) + Rn (NAVn) + SSFn (TOPIGAVn) + TCn (GAVn)

Where:

For n	=	year to which charge relates within the Depreciation Period

n 		=	year to which charge relates

GAVn	= 	GAV for year n re-valued by relevant indexation method

TOPIGAVn 	=	GAV for year n re-valued by TOPI indexation

NAVn 	=	NAV for year n based on re-valued GAVn

Dn		             = Depreciation rate as percentage (equal to 1/Depreciation 	Period(typically 1/40 = 2.5% of GAV)

Rn	=	For assets subject to TOPI indexation, the real pre-tax Weighted Average Cost of Capital for the Relevant Transmission Licensee for year n (WACCn).

For asset subject to MEA indexation, the real pre-tax Weighted Average Cost of Capital for the Relevant Transmission Licensee for year n (WACCn) plus 1.5 percentage points.

Where for the year n:

WACCn=real post tax cost of equity1-corporation tax rate × 1-notional gearing % + real cost of debt ×notional gearing %

And where for the calculation of WACCn: The real post-tax cost of equity, notional gearing %, real cost of debt and the corporation tax rate, are as specified in the latest published Ofgem Price Control Financial Model (PCFM) relating to year n, or should Ofgem fail to publish or cease to publish a PCFM, those specified in the latest public regulatory determination(s) or decision(s) should be used.

SSFn	=	Site Specific Factor for year n as a % (equal to the Site Specific Cost/Total Site GAV)

TCn		=	Transmission Running Cost component for year n (other Transmission Owner Activity costs).

For n	=	year to which charge relates beyond the Depreciation Period

n 		=	year to which charge relates

GAVn	=		GAV for year n re-valued by relevant indexation method

TOPIGAVn 	=	GAV for year n re-valued by TOPI indexation

NAVn 	=	0

Dn		=	0

Rn		=	0

SSFn	=	Site Specific Factor for year n as a % (equal to the Site Specific Cost/Total Site GAV)

TCn		=	Transmission Running cost component for year n (other Transmission Owner Activity costs).

- Note that, for the purposes of deriving asset specific charges for site-specific maintenance, the TOPI re-valued GAV is used.  This is to ensure that the exact site charges are recovered from the assets at the site.  The site costs are apportioned to the assets on the basis of the ratio of the asset GAV to total Site GAV.

## Adjustment for Capital Contributions

- If a User chooses to make a 100% capital contribution (either pre-commissioning or post-commissioning) to The Company towards their allocation of a connection asset then no capital charges will be payable and hence the connection charges for that asset would be calculated as follows:

Annual Connection Chargen = SSFn (TOPIGAVn) + TCn (GAVn)

- If a User chooses to make a partial capital contribution(s) (either pre-commissioning or post-commissioning) to The Company towards their allocation of a connection asset, for example PCCF = 50%, then the connection charges for that asset would be calculated as follows:

Annual Connection Chargen = Dn (GAVn*PCCF) + Rn (NAVn*PCCF) + SSFn (TOPIGAVn) + TCn (GAVn)

PCCF = Partial Capital Contribution Factor taking into account a capital contribution made pre-commissioning compared to the GAV (as outlined in 14.3.10), and any capital contributions made post-commissioning compared to the appropriate NAV (as outlined in 14.3.12) as appropriate.

Modification of Connection Assets

- Where a modification to an existing connection occurs at the User’s request or due to developments to the transmission system, their annual connection charges will reflect any additional connection assets that are necessary to meet the User's requirements. Charges will continue to be levied for existing assets that remain in service.  Termination charges as described in Chapter 5 below will be charged for any existing connection assets made redundant as a result of the modification.

# 14.4 Other Charges

- In addition to the basic annual connection charges set out above, the User may pay The Company for certain other costs related to their connection.  These will be set out in the Bilateral and Construction Agreements where appropriate and are described below.

## One-off Works-

-

- To provide or modify a connection, the transmission licensee may be required to carry out works on the transmission system that, although directly attributable to the connection, may not give rise to additional connection assets.  These works are defined as “one-offs”.  Liability for one-off charges is established with reference to the principles laid out below:

- Where a cost cannot be capitalised into either a connection or infrastructure asset, typically a revenue cost

- Where a non-standard incremental cost is incurred as a result of a User's request, irrespective of whether the cost can be capitalised

- Termination Charges associated with the write-off of connection assets at the connection site.

Consistent with these principles and in accordance with Connection Charging Methodology modification GB ECM-01, which was implemented on 1 December 2005, a one-off charge will be levied for a Category 1 Intertripping Scheme or a Category 3 Intertripping Scheme. A one-off charge will not be levied for a Category 2 Intertripping Scheme or a Category 4 Intertripping Scheme.

- The one-off charge is a charge equal to the cost of the works involved, together with a reasonable return, as shown in 14.4.4 below.

- For information, the general formula for the calculation of the one-off charge for works is outlined below.

One-off Charge = (Construction Costs + Engineering Charges) x (1 + Return %)

+ IDC + LD Premium

Where:		Engineering Charges = “Engineering Charge” x job hours

Return % = 6%

IDC = Interest During Construction

LD Premium = The Company Liquidated Damages Premium (if applicable)

- The calculation of the one-off charge for write-off of assets is outlined below:

Write-off Charge = 100% of remaining NAV of redundant assets

- One-offs are normally paid on an agreed date, which is usually upon completion of the works.  However, arrangements may be agreed between the transmission licensee and the User to pay the charge over a longer period.  If a one-off is paid over a longer period it is termed a Transmission Charge. It is usually a depreciating finance charge or annuity based charge with a rate of return element and may include agreement on a schedule of termination payments if the agreement is terminated before the end of the annuity period.   The charge is usually inflated annually by the same TOPI figure that is used to inflate GAVs, though Users can request alternative indexation methods.

- Where an infrastructure asset has been subject to One-off Works, and a User has paid a relating charge calculated in accordance with paragraph 14.4.4, The Company may adjust the treatment of the assets within the TNUoS transport model as set out in paragraphs 14.15.15 to 14.15.22.

## Miscellaneous Charges

- Other contract specific charges may be payable by the User, these will be set out in the Bilateral and Construction Agreements where appropriate.

## Rental sites

- Where The Company owns a site that is embedded within a distribution network, the connection charge to the User is based on the capital costs and overheads but does not include maintenance charges.

## Final Metering Scheme (FMS)/Energy Metering Systems

- Charges for FMS metering are paid by the registrant of the FMS metering at the connection site.  It is charged on a similar basis as other Connection Assets.  The electronic components of the FMS metering have a replacement and depreciation period in line with those advised by the transmission licensees, whilst the non-electronic components normally retain a 40 year replacement and depreciation period (or a User specified depreciation period as appropriate).

# 14.5 Connection Agreements

## Indicative Agreement

- The standard connection agreement offered by The Company is an indicative price agreement.  From the Charging Date as set out in the User's Bilateral Connection Agreement, the User's initial connection charge is based on a fair and reasonable estimate of the expected costs of the connection.

## Outturning the Indicative Agreement

- Once the works required to provide a new or modified connection are completed and the costs finalised, the connection scheme is "outturned".  The Company reconciles the monies paid by the User on the indicative charge basis against the charges that would have been payable based on the actual costs incurred in delivering the project together with any relevant interest.  This process involves agreeing a new charging GAV (The Base Amount) with the User in line with the elements stated in paragraph 14.3.3 and then calculating connection charges with this GAV.

- In addition, for Users that have chosen MEA revaluation their MEA ratios are agreed at outturn and this ratio is used for MEA revaluation in subsequent years.

- In the case of connection asset replacement where there is no initiating User, the outturn is agreed with the User at the site.

## Firm Price Agreement

- In addition to the options stated in paragraph 14.3.10 above, firm price agreements are also available.  Typically with this option the charges to be incurred, and any indexation, are agreed between The Company and the User and connection charges are not recalculated once outturn costs are known.  A typical example of a firm price agreement is:

- Capital Contribution

- Firm Price GAV

- Running Costs (based on a firm price GAV)

- Fixed Schedule of Termination Amounts

- When a User selects a firm price agreement some or all of the above elements can be made firm. Any elements of the agreement that have not been made firm will be charged on an indicative basis in accordance with this statement.

- Final Sums and Consents costs are never made firm in a Firm Price Agreement.  Details of both are set out in the Construction Agreement.

## Monthly Connection Charges

- The connection charge is an annual charge payable monthly.

- If the initial Charging Date does not fall within the current Financial Year being charged for and there are no revisions to charges during the year, the monthly connection charge will equal the annual connection charge divided by twelve.

- For the Financial Year in which the Charging Date occurs (as set out in the User's Bilateral Agreement) or for any Financial Year in which a revision to charges has occurred during the Financial Year, for each complete calendar month from the Charging Date (or effective date of any charge revision) to the end of the Financial Year in which the Charging Date (or charge revision) occurs, the monthly connection charge shall be equal to the annual connection charge divided by twelve.

- For each part of a calendar month, the charge will be calculated as one twelfth of the annual connection charge prorated by the ratio of the number of days from and including the Charging Date to the end of the month that the Charging Date falls in and the number of days in that month.

- For example, say the annual connection charge for Financial Year 2010/11 is £1.2m and the Charging Date falls on the 15th November 2010, the monthly charges for the Financial Year 2010/11 would be as follows:

- November = £1,200,000/12 * (16/30)	=	£53,333.33

- Dec 10, Jan 11, Feb 11, Mar 11 	= 	£1,200,000/12

= 	£100,000.00

- The above treatment does not apply to elements such as Miscellaneous Charges (as defined in 14.4.8) and Transmission Charges (annuitised one-offs, as defined in 14.4.6).  If the Charging Date falls within a Financial Year, then the full annual charge will remain payable and will be spread evenly over the remaining months.   This is because these payments are an annuitisation of charges that would normally be paid up-front as one-off payments.

# 14.6 Termination Charges

## Charges Liable

- Where a User wholly or partially disconnects from the transmission system they will pay a termination charge.  The termination charge will be calculated as follows:

- Where the connection assets are made redundant as a result of the termination or modification of a Bilateral Connection Agreement, the User will be liable to pay an amount equal to the NAV of such assets as at the end of the Financial Year in which termination or modification occurs, plus:

- The reasonable costs of removing such assets.  These costs being inclusive of the costs of making good the condition of the connection site

- If a connection asset is terminated before the end of a Financial Year, the connection charge for the full year remains payable. Any remaining Use of System Charges (TNUoS and BSUoS) also remain payable

- For assets where it has been determined to replace upon the expiry of the relevant Replacement Period in accordance with the provisions set out in the CUSC and in respect of which a notice to Disconnect or terminate has been served in respect of the Connection Site at which the assets were located; and due to the timing of the replacement of such assets, no Connection Charges will have become payable in respect of such assets by the User by the date of termination; the termination charges will include the reasonable costs incurred by  the transmission licensee in connection with the installation of such assets

- Previous capital contributions paid to The Company will be taken into account

- The Calculation of Termination amounts for Financial Year n is as follows:

Termination Chargen = UoSn + Cn + NAVan + R - CC

Where:

UoSn	= Outstanding Use of System Charge for year (TNUoS and BSUoS)

Cn	= Outstanding Connection Charge for year

NAVan	= NAV of Type A assets as at 31 March of Financial Year n

R	= Reasonable costs of removal of redundant assets and making good

CC	= An allowance for previously paid capital contributions

- Examples of reasonable costs of removal for terminated assets and making good the condition of the site include the following:

- If a circuit breaker is terminated as a result of a User leaving a site, this may require modifications to the protection systems.

- If an asset were terminated and its associated civils had been removed to 1m below ground then the levels would have to be made up.  This is a common condition of planning consent.

## Repayment on Re-Use of Assets

- If any assets in respect of which a termination charge was made to The Company are re-used at the same site or elsewhere on the system, including use as infrastructure assets, The Company will make a payment to the original terminating User to reflect the fact that the assets are being reused.

- The arrangements for such repayments for re-use of Assets are that The Company will pay the User a sum equal to the lower of:

- the Termination Amount paid in respect of such  Assets; or

- the NAV attributed to such  Assets for charging purposes upon their re-use

less any reasonable costs incurred  in respect of the storage of those assets.

- The definition of re-use is set out in the CUSC.  Where The Company decides to dispose of a terminated asset where it is capable of re-use, The Company shall pay the User an appropriate proportion of the sale proceeds received.

## Valuation of Assets that are re-used as connection assets or existing infrastructure assets re-allocated to connection

- If an asset is reused following termination or allocated to connection when it has previously been allocated to TNUoS, a value needs to be determined for the purposes of connection charges.  In both instances the connection charge will be based on the standard formula set out in paragraph 14.3.21. The Gross Asset Value will be based on the original construction costs and indexed by TOPI. Where original costs are not known a reasonable value will be agreed between The Company and the User based on similar types of asset in use. The Net Asset Value will be calculated as if the asset had been in continuous service as a connection asset from its original commissioning date taking into account the depreciation period.

- Where an asset has been refurbished or updated to bring it back into service a new value and an appropriate replacement period will be agreed between The Company and the User. This will be based on the value of similar types of asset in service and the costs of the refurbishment.

# 14.7 Contestability

- Some connection activities may be undertaken by the User.  The activities are the provision, or construction, of connection assets, the financing of connection assets and the ongoing maintenance of those assets.   While some Users have been keen to see contestability wherever possible, contestability should not prejudice system integrity, security and safety.  These concerns have shaped the terms that are offered for contestability in construction and maintenance.

## Contestability in Construction

- Users have the option to provide (construct) connection assets if they wish.   Formal arrangements for Users exercising this choice are available and further information on User choice in construction can be obtained from The Company.

# 14.8 Asset Replacement

- Appendix A of a User's Bilateral Connection Agreement specifies the age (number of complete Financial Years old), for charging purposes, of each of the NETS connection assets at the Connection Site for the corresponding Financial Year. Connection charges are calculated on the assumption that the assets will not need to be replaced until the charging age has reached the duration of the asset’s Replacement Period.

If a connection asset is to be replaced, The Company will enter into an agreement for the replacement with the User.  Where replacement occurs before the original asset’s charging age has reached the duration of its Replacement Period, The Company will continue to charge for the original asset and make no charge to the existing User for the new asset until the original asset’s charging age has reached the duration of its Replacement Period.

Where the replacement occurs after the original asset’s charging age has reached the duration of its Replacement Period, The Company will charge on the basis of the original asset until replaced and on the basis of the new asset on completion of the works.

- When the original asset’s charging age has reached the duration of its Replacement Period the User’s charge will be calculated on the then Net Asset Value of the new asset. The new asset begins depreciating for charging purposes upon completion of the asset replacement.

The Basic Annual Connection Charge Formulae are set out in Chapter 2: The Basic Annual Connection Charge Formula.

## Asset Replacement that includes a change of Voltage

- There are a number of situations where an asset replacement scheme may involve a change in the voltage level of a User's connection assets.  These replacement schemes can take place over a number of years and may involve a long transitory period in which connection assets are operational at both voltage levels.

- These situations are inevitably different from case to case and hence further charging principles will need to be developed over time as more experience is gained.  Set out below, are some generic principles.  This methodology will be updated as experience develops.

- The general principles used to date are to ensure that, in the transitory period of an asset replacement scheme, the User does not pay for two full transmission voltage substations and that the charges levied reflect the Replacement Period of the original connection assets.  In addition, in line with paragraph 14.8.1 above, charges will only be levied for the new assets once the original assets would have required replacement.

- For example, a transmission licensee in investing to meet a future Security Standard need on the main transmission system, may require the asset replacement of an existing 275kV substation with a 400kV substation prior to the expiry of the original assets’ Replacement Period.   In this case, The Company will seek to recover the connection asset component via connection charges when the assets replaced were due for asset replacement.  Prior to this, the User should not see an increase in charges and therefore the investment costs would be recovered through TNUoS charges.

In addition, if in the interim stage the User has, say, one transformer connected to the 275kV substation and one transformer connected to the 400kV substation, the charge will comprise an appropriate proportion of the HV assets at each site and not the full costs of the two substations. Note that the treatment described above is only made for transitory asset replacement and not enduring configurations where a User has connection assets connected to two different voltage substations.

# 14.9 Data Requirements

- Under the connection charging methodology no data is required from Users in order to calculate the connection charges payable by the User.

# 14.10 Applications

- Application fees are payable in respect of applications for new connection agreements and modifications to existing agreements based on the reasonable costs transmission licensees incur in processing these applications.  Users can opt to pay a fixed price application fee in respect of their application or pay the actual costs incurred.  The fixed price fees for applications are detailed in the Statement of Use of System Charges.

- If a User chooses not to pay the fixed fee, the application fee will be based on an advance of transmission licensees’ Engineering and out-of pocket expenses and will vary according to the size of the scheme and the amount of work involved. Once the associated offer has been signed or lapses, a reconciliation will be undertaken. Where actual expenses exceed the advance, The Company will issue an invoice for the excess. Conversely, where The Company does not use the whole of the advance, the balance will be refunded.

- The Company will refund the first application fee paid (the fixed fee or the amount post-reconciliation) made under the Construction Agreement for new or modified existing agreements.  The refund shall be made either on commissioning or against the charges payable in the first three years of the new or modified agreement.  The refund will be net of external costs.

- The Company will not refund application fees for applications to modify a new agreement or modified existing agreement at the User’s request before any charges become payable.  For example, The Company will not refund an application fee to delay the provision of a new connection if this is made prior to charges becoming payable.

# 14.11 Illustrative Connection Charges

## From 2021/22 First Year Connection Charges based on the TOPI Method (6% rate of return used as an example)

- The following table provides an indication of typical charges for new connection assets.  Before using the table, it is important to read through the notes below as they explain the assumptions used in calculating the figures.

Calculation of Gross Asset Value (GAV)

- The GAV figures in the following table were calculated using the following assumptions:

- Each asset is new

- The GAV includes estimated costs of construction, engineering, Interest During Construction and Liquidated Damages premiums

For details of the Calculation of the Gross Asset Value, see Chapter 2 of this Statement.

Calculation of first year connection charge

- The first year connection charges in the following table were calculated using the following assumptions:

- The assets are new

- The assets are depreciated over 40 years

- The rate of return is assumed to be 6% for TOPI indexation

- The connection charges include maintenance costs at a rate of 0.52% of the GAV

- The connection charges include Transmission Running Costs at a rate of 1.45% of the GAV

For details of the Basic Annual Connection Charge Formula, see Chapter 2 of this Statement.

Please note that the actual charges will depend on the specific assets at a site.  Agreement specific NAVs and GAVs for each User will be made available on request.

Notes on Assets

- The charges for Double and Single Busbar Bays include electrical and civil costs.

- Transformer cable ratings are based on winter soil conditions.

- In this example, transformer charges include civil costs of plinth and noise enclosure and estimated transport costs, but not costs of oil dump tank and fire trap moat.  Transport costs do not include hiring heavy load sea transportation or roll-on roll-off ships.

|  | £000’s |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 400kV |  | 275kV |  | 132kV |  |
|  | GAV | Charge | GAV | Charge | GAV | Charge |
| Double Busbar Bay | 2300 | 239 | 1890 | 197 | 630 | 65 |
| Single Busbar Bay | 1830 | 190 |  |  | 460 | 50 |
| Transformer Cables 100m (incl. Cable sealing ends) |  |  |  |  |  |  |
| 120MVA |  |  | 970 | 100 | 310 | 30 |
| 180MVA | 1480 | 150 | 970 | 101 | 320 | 30 |
| 240MVA | 1520 | 158 | 980 | 102 | 355 | 37 |
| 750MVA | 1540 | 160 | 1135 | 118 |  |  |
| Transformers |  |  |  |  |  |  |
| 45MVA 132/66kV |  |  |  |  | 1060 | 110 |
| 90MVA 132/33kV |  |  |  |  | 102 0 | 106 |
| 120MVA 275/33kV |  |  | 2110 | 219 |  |  |
| 180MVA 275/66kV |  |  | 2560 | 266 |  |  |
| 180MVA 275/132kV |  |  | 2180 | 227 |  |  |
| 240MVA 275/132kV |  |  | 2630 | 273 |  |  |
| 240MVA 400/132kV | 3180 | 340 |  |  |  |  |

## Connection Examples

Example 1

Example 2

Example 3

# 14.12 Examples of Connection Charge Calculations

The following examples of connection charge calculations are intended as general illustrations.

## Example 1

- This example illustrates the method of calculating the first year connection charge for a given asset value. This method of calculation is applicable to indicative price agreements for new connections, utilising the TOPI method of charging, and assuming:

i)	the asset is commissioned on 1 April 2010

ii)	there is no inflation from year to year i.e. GAV remains constant

iii)	the site specific maintenance charge component remains constant throughout the 40 years at 0.52% of GAV

iv)	the Transmission Running Cost component remains constant throughout the 40 years at 1.45% of GAV

v)	the asset is depreciated over 40 years

vi)	the rate of return charge remains constant at 6% for the 40 year life of the asset

vii)	the asset is terminated at the end of its 40 year life

For the purpose of this example, the asset on which charges are based has a Gross Asset Value of £3,000,000 on 1 April 2010.

| - Charge | - Calculation |  |
| --- | --- | --- |
| Site Specific Maintenance Charge (0.52% of GAV) | 3,000,000 x 0.52% | £15,600 |
| Transmission Running Cost - (1.45% of GAV) | 3,000,000 x 1.45% | £43,500 |
| Capital charge (40 year depreciation 2.5% of GAV) | 3,000,000 x 2.5% | £75,000 |
| Return on mid-year NAV (6%) | 2,962,500 x 6% | £177,750 |
| TOTAL |  | £311,850 |

The first year charge of £311,850 would reduce in subsequent years as the NAV of the asset is reduced on a straight-line basis.

This gives the following annual charges over time (assuming no inflation):

Year	Charge

1	£311,850

2	£307,350

10	£271,350

40	£136,350

Based on this example, charges of this form would be payable until 31 March 2050.

## Example 2

- The previous example assumes that the asset is commissioned on 1 April 2010.  If it is assumed that the asset is commissioned on 1 July 2010, the first year charge would equal 9/12th of the first year annual connection charge i.e. £233,887.50

This gives the following annual charges over time:

Year	Charge

1	£233,887.50 (connection charge for period July to March)

2	£307,350

10	£271,350

40	£136,350

## Example 3

- In the case of a firm price agreement, there will be two elements in the connection charge, a finance component and a running cost component.  These encompass the four elements set out in the examples above.  Using exactly the same assumptions as those in example 1 above, the total annual connection charges will be the same as those presented.  These charges will not change as a result of the adoption of a different charging methodology by The Company, providing that the connection boundary does not change.

## Example 4

- If a User has chosen a 20-year depreciation period for their Post Vesting connection assets and subsequently remains connected at the site beyond the twentieth year their charges are calculated as follows.

For years 21-40 they will pay a connection charge based on the following formula:

Annual Connection Chargen = SSFn  (TOPIGAVn)+ TCn (GAVn)

The NAV will be zero and the asset will be fully depreciated so there will be no rate of return or depreciation element to the charge.

# 14.13 Nominally Over Equipped Connection Sites

- This chapter outlines examples of ways in which a connection site can be considered as having connection assets that exceed the strict, theoretical needs of the individual Users at the connection site.  These can be described as:

## Historical

- This is where the connection assets at the connection site were installed to meet a requirement of the Users for connection capacity that no longer exists.  An example would be where a User, at one time, had a requirement for, say, 270 MW. This would allocate three 240 MVA 400/132kV transformers to the User.  Due to reconfiguration of that User’s network only 200 MW is now required from the connection site. The lower requirement would only allocate two transformers, but all the transformers are kept in service.  The connection assets will continue to be assigned to the User’s connection, and charged for as connection, until the User makes a Modification Application to reduce the historical requirement. In some cases the Modified requirement will mean that Termination Payments will have to be made on some connection assets.

## Early Construction

- If a User has a multi-phase project, it may be necessary to install connection assets for the latter phases at the time of the first phase.  These connection assets could be charged from the first phase charging date.

## Connection site Specific Technical or Economic Conditions

- In circumstances where the transmission licensee has identified a wider requirement for development of the transmission system, it may elect to install connection assets of greater size and capacity than the practicable minimum scheme required for a particular connection. In these circumstances, however, connection charges for the party seeking connection will normally be based on the level of connection assets consistent with the practicable minimum scheme needed to meet the applicant's requirements.

- There may be cases where there are specific conditions such that the practicable minimum scheme at a site has to be greater than the strict, theoretical interpretation of the standards.  In these cases all assets will still be assigned to connection and connection charges levied.

- A practicable minimum scheme is considered in terms of the system as a whole and may include a change in voltage level.

# Part 2 - The Statement of the Use of System Charging Methodology

# Section 1 – The Statement of the Transmission Use of System Charging Methodology

# 14.14 Principles

- Transmission Network Use of System Charges reflect the cost of installing, operating and maintaining the Transmission System for the Transmission Owner (TO) (including Competitively Appointed Transmission Owners (CATOs)) activity functions of the Transmission Businesses of each Relevant Transmission Licensee. These activities are undertaken to the standards prescribed by the ESO Licence and the Transmission Licences, to provide the capability to allow the flow of bulk transfers of power between Connection Sites and to provide transmission system security.

- The allowed revenue defined for these activities agreed with the Authority at the time of the Transmission Owners’ price control review for the succeeding price control period. The allowed revenue can be adjusted during the price control period. Transmission Network Use of System Charges are set to recover the allowed revenue as set by the price control (where necessary, allowing for any Kt adjustment for under or over recovery in a previous year net of the income recovered through pre-vesting connection charges).

14.14.2a The payments made to Competitively Appointed Transmission Owners (CATOs) are not set via a price control of the same form as incumbent Transmission Owners; instead, the payment to each CATO takes the form of a Tender Revenue Stream (TRS).  The method for determining the TRS for a CATO will be prescribed within its licence.  Transmission Network Use of System Charges are set to recover the Allowed Revenue which is determined in accordance with the terms of the CATO’s licence, such Allowed Revenue may include amongst other things the TRS, adjustments for indexation and incentivisation, and other payments provided for under its licence.

- The basis of charging to recover the allowed revenue is the Investment Cost Related Pricing (ICRP) methodology, which was initially introduced by The Company in 1993/94 for England and Wales.  The principles and methods underlying the ICRP methodology were set out in The Company document "Transmission Use of System Charges Review: Proposed Investment Cost Related Pricing for Use of System (30 June 1992)".

- In December 2003, The Company published the Initial Thoughts consultation for a GB methodology using the England and Wales methodology as the basis for consultation. The Initial Methodologies consultation published by The Company in May 2004 proposed two options for a GB charging methodology with a Final Methodologies consultation published in August 2004 detailing The Company’s response to the Industry with a recommendation for the GB charging methodology. In December 2004, The Company published a Revised Proposals consultation in response to the Authority’s invitation for further review on certain areas in The Company’s recommended GB charging methodology.

- In April 2004 The Company introduced a DC Loadflow (DCLF) ICRP based transport model for the England and Wales charging methodology. The DCLF model has been extended to incorporate Scottish network data with existing England and Wales network data to form the GB network in the model. In April 2005, the GB charging methodology implemented certain proposals which have been further expanded so that the model now includes the following:

- The application of multi-voltage circuit expansion factors with a forward-looking Expansion Constant that does not include substation costs in its derivation.

- The application of locational security costs, by applying a multiplier to the Expansion Constant reflecting the difference in cost incurred on a secure network as opposed to an unsecured network.

- The application of a de-minimus level demand charge of £0/kW for Half Hourly and £0/kWh for Non-Half Hourly metered demand and £0/KWh for Unmetered Supplies and £0/site/day for Transmission Demand Residual Tariffs, to avoid the application of negative demand charges.

- The application of 132kV expansion factor on a Transmission Owner basis reflecting the regional variations in network upgrade plans.

- The Company will set tariffs in a manner so that the locationally varying element, as established by the DCLF ICRP model and, where appropriate, local substation and local circuit charges, are levied on all Generator and Demand Users. Any remaining Transmission Owner revenues will be recovered from demand only in a non-locational manner through the Transmission Demand Residual Tariffs

- For the purpose of compliance with the Limiting Regulation in the context of setting limits on the annual charges paid by generation The Company will exclude Charges for Physical Assets Required for Connection when calculating the total amount to be recovered from Generators (GCharge (Forecast)).

- If having applied the exclusion of Charges for Physical Assets Required for Connection The Company identifies that an adjustment to TNUoS Charges is required to remain compliant with the Limiting Regulation then an Adjustment Tariff will be applied to all Generators in the following circumstances.

a) The Adjustment Tariff will be applied if The Company identifies that either:

1. Annual average TNUoS charges payable by Generator Users will fall below €0/MWh

OR

2. Annual average TNUoS charges payable by Generator Users will exceed €2.50/MWh adjusted by a risk margin to allow for error in tariff setting.

b) Where annual average TNUoS charges to Generators are positive under the GCharge (Forecast) the Adjustment Tariff will be applied if the Adjustment Revenue is less than £0. The Adjustment Revenue is expressed as:

𝐴𝑑𝑗𝑅𝑒𝑣𝑒𝑛𝑢𝑒 = (𝐺𝑂 ∗ ((𝐶𝑎𝑝𝐸𝐶 ∗ (1 − 𝑦)) ∗ 𝐸𝑅)) –

𝐺𝐶ℎ𝑎𝑟𝑔𝑒(𝐹𝑜𝑟𝑒𝑐𝑎𝑠𝑡)

- Where annual average TNUoS charges to Generators are negative under the GCharge (Forecast) the Adjustment Revenue will be the difference between £0 and the total recovered from Generators. The Adjustment Revenue will be expressed as:

𝐴𝑑𝑗𝑅𝑒𝑣𝑒𝑛𝑢𝑒 = 0 − 𝐺𝐶ℎ𝑎𝑟𝑔𝑒(𝐹𝑜𝑟𝑒𝑐𝑎𝑠𝑡)

- The total adjusted revenue expected to be recovered from Generators (AdjGenRev) through TNUoS tariffs can therefore be expressed as:

𝐴𝑑𝑗𝐺𝑒𝑛𝑅𝑒𝑣 = 𝐺𝐶ℎ𝑎𝑟𝑔𝑒(𝐹𝑜𝑟𝑒𝑐𝑎𝑠𝑡) + 𝐴𝑑𝑗𝑅𝑒𝑣𝑒𝑛𝑢𝑒

- The error margin used in calculating TNUoS tariffs for the Financial Year is expressed as:

y = (1+ ErrorGenRev) / (1 - ErrorGO) -1

- Where:

y = error margin expressed in %.

ErrorGenRev = the highest absolute percentage error in generation revenue collection, adjusted by systemic error, from the past 5 full years (year t-6 to t-2 inclusive). Systemic error is the average of %error in generation revenue collection for the past 5 full years. Systemic error can be positive or negative.

ErrorGO = the highest absolute percentage error in generation TWh outputs, from the past 5 full years (year t-6 to t-2 inclusive).

- The Company will use the latest OBR Forecast of £/€ exchange rate published prior to the 31st October in the year preceding the relevant Financial Year to convert average annual TNUoS charges payable by Generators in the GCharge (Forecast) to a comparable value for the purposes of assessing compliance with the Limiting Regulation.

- The Adjustment Tariff used in the calculation will be either:

1.  a negative £/kW tariff that reduces annual average TNUoS charges to Generators to below the risk adjusted upper limit of the Limiting Regulation in accordance with 14.14.5 (f).

OR

2. a positive £/kW tariff that increases annual average TNUoS charges to Generators to above the lower limit of the Limiting Regulation in accordance with 14.14.5 (f).

Expressed in either case as:

AdjTariff=AdjRevenueChargeable Capacity

Where;

CapEC	=	Upper limit of the range specified in the Limiting Regulation

y	=	Error margin built in to adjust CapEC

GO	=	Forecast GB Generation Output for generation liable for Transmission charges (i.e. energy injected into the transmission network in MWh) for the Financial Year

ER	=	The latest OBR Forecast €/£ Exchange Rate published prior to the 31st October in the year preceding the relevant Financial Year

GCharge (Forecast)	=	The total forecast TNUoS revenue to be recovered from Generators in the Financial Year minus Charges for Physical Assets Required for Connection.

AdjRevenue	=	Adjustment Revenue

Chargeable Capacity	=	as per paragraph 14.18.6

AdjTariff	=	Any Adjustment Tariff required to remain compliant with the Limiting Regulation.

- The currently applicable number of generation zones, determined in accordance with 14.15.37 and using the criteria outlined in paragraph 14.15.42, is detailed in the Statement of Use of System Charges which is available from the Website.

- The number of demand zones has been determined as 14, corresponding to the 14 GSP groups.

- The underlying rationale behind Transmission Network Use of System charges is that efficient economic signals are provided to Users when services are priced to reflect the incremental costs of supplying them.  Therefore, charges should reflect the impact that Users of the transmission system at different locations would have on the Transmission Owner's costs, if they were to increase or decrease their use of the respective systems.  These costs are primarily defined as the investment costs in the transmission system, maintenance of the transmission system and maintaining a system capable of providing a secure bulk supply of energy.

The ESO Licence requires The Company to operate the National Electricity Transmission System to specified standards. In addition The Company and  transmission licensees are required to plan and develop the National Electricity Transmission System to meet these standards.  These requirements mean that the system must conform to a particular Security Standard and capital investment requirements are largely driven by the need to conform to both the deterministic and supporting cost benefit analysis aspects of this standard.  It is this obligation, which provides the underlying rationale for the ICRP approach, i.e. for any changes in generation and demand on the system, The Company must ensure that it satisfies the requirements of the Security Standard.

- The Security Standard identifies requirements on the capacity of component sections of the system given the expected generation and demand at each node, such that demand can be met and Generators’ output over the course of a year (capped at their Transmission Entry Capacity, TEC) can be accommodated in the most economic and efficient manner.  The derivation of the incremental investment costs at different points on the system is therefore determined against the requirements of the system both at the time of peak demand and across the remainder of the year. The Security Standard uses a Demand Security Criterion and an Economy Criterion to assess capacity requirements.  The charging methodology therefore recognises both these elements in its rationale.

- The Demand Security Criterion requires sufficient transmission system capacity such that peak demand can be met through generation sources as defined in the Security Standard, whilst the Economy Criterion requires sufficient transmission system capacity to accommodate all types of generation in order to meet varying levels of demand efficiently. The latter is achieved through a set of deterministic parameters that have been derived from a generic Cost Benefit Analysis (CBA) seeking to identify an appropriate balance between constraint costs and the costs of transmission reinforcements.

- The TNUoS charging methodology seeks to reflect these arrangements through the use of dual backgrounds in the Transport Model, namely a Peak Security background representative of the Demand Security Criterion and a Year Round background representative of the Economy Criterion.

- To recognise that various types of generation will have a different impact on incremental investment costs the charging methodology uses a generator’s TEC, Peak Security flag, and Annual Load Factor (ALF) when determining Transmission Network Use of System charges relating to the Peak Security and Year Round backgrounds respectively.  For the Year Round background the diversity of the plant mix (i.e the proportion of low carbon and carbon generation) in each charging zone is also taken into account.

- In setting and reviewing these charges The Company has a number of further objectives. These are to:

- offer clarity of principles and transparency of the methodology;

- inform existing Users and potential new entrants with accurate and stable cost messages;

- charge on the basis of services provided and on the basis of incremental rather than average costs, and so promote the optimal use of and investment in the transmission system; and

- be implementable within practical cost parameters and time-scales.

- The Company will typically calculate TNUoS tariffs annually, publishing final tariffs in respect of a Financial Year by the end of the preceding January. However The Company may update the tariffs part way through a Financial Year.

# 14.15 Derivation of the Transmission Network Use of System Tariff

- The Transmission Network Use of System (TNUoS) Tariff comprises two separate elements. Firstly, a locationally varying element derived from the DCLF ICRP transport model to reflect the costs of capital investment in, and the maintenance and operation of, a transmission system to provide bulk transport of power to and from different locations. Secondly, a non-locationally varying element related to the provision of residual revenue recovery from demand only.

- For generation TNUoS tariffs the locational element itself is comprised of five  separate components.  Three wider  components –

- Wider Peak Security Component

- Wider Year Round Not-shared component

- Wider Year Round component

These components reflect the costs of the wider network under the different generation backgrounds set out in the Demand Security Criterion (for Peak Security component) and Economy Criterion (for both Year Round components) of the Security Standard.  The two Year Round components reflect the unshared and shared costs of the wider network based on the diversity of generation plant types.

Two local components –

- Local substation, and

- Local circuit

These components reflect the costs of the local network.

Accordingly, the wider tariff represents the combined effect of the three wider locational tariff components and the local tariff represents the combination of the two local locational tariff components.

Finally, an Adjustment Tariff component may also be charged to Generators as per paragraph 14.14.5.

- The process for calculating the TNUoS tariff is described below.

## The Transport Model

### Model Inputs

- The DCLF ICRP transport model calculates the marginal costs of investment in the transmission system which would be required as a consequence of an increase in demand or generation at each connection point or node on the transmission system, based on a study of peak demand conditions using both Peak Security and Year Round generation backgrounds on the transmission system.  One measure of the investment costs is in terms of MWkm.  This is the concept that ICRP uses to calculate marginal costs of investment.  Hence, marginal costs are estimated initially in terms of increases or decreases in units of kilometres (km) of the transmission system for a 1 MW injection to the system.

- The transport model requires a set of inputs representative of the Demand Security and Economy Criterion set out in the Security Standards. These conditions on the transmission system are represented in the Peak Security and Year Round background respectively as follows:

- Nodal generation information per node (TEC, plant type and SQSS scaling factors)

- Nodal net demand information

- Transmission circuits between these nodes

- The associated lengths of these routes, the proportion of which is overhead line or cable and the respective voltage level

- The cost ratio of each of 132kV overhead line, 132kV underground cable, 275kV overhead line, 275kV underground cable and 400kV underground cable to 400kV overhead line to give circuit expansion factors

- The cost ratio of each separate sub-sea AC circuit and HVDC circuit to 400kV overhead line to give circuit expansion factors

- 132kV overhead circuit capacity and single/double route construction information is used in the calculation of a generator’s local charge.

- Offshore transmission cost and circuit/substation data

- For a given Financial Year "t", the nodal generation TEC figure and generation plant types at each node will be based on the Applicable Value for year "t" in the NETS Electricity Ten Year Statement (ETYS) in year "t-1" plus updates to the October of year "t-1". The contracted TECs and generation plant types in the NETS Electricity Ten Year Statement (ETYS) include all plant belonging to Generators who have a Bilateral Agreement with the TOs.  For example, for 2010/11 charges, the nodal generation data is based on the forecast for 2010/11 in the 2009 NETS Electricity Ten Year Statement (ETYS) plus any data included in the quarterly updates in October 2009.

- Scaling factors for different generation plant types are applied on their aggregated capacity for both Peak Security and Year Round backgrounds.  The scaling is either Fixed or Variable (depending on the generation plant type as referenced in the table below) and based on the factors used in the Security Standard, which are referenced in the same table.

A 10% floor for the Year Round Background variable scaling factor is used for charging purposes. If the methodology set out in the SQSS for calculating the variable scaling factor results in a lower number, the variable factor will be set at 10%, and fixed scaling factors adjusted by a uniform amount. This adjustment will be based on the SQSS principle that the total scaled output of all generation plant shall equal the ACS (Annual Average Cold Spell) peak demand minus total imports from external systems, and can be calculated using the following equation.

Adjustment= ACSPeak Demand-TEC×10%Variable PlantTEC×Year Round Scaling FactorFixed Plant

Adjusted Fixed Scaling Factor=Adjustment×Scaling Factor

| Generation Plant Type | Peak Security Background | Year Round Background |
| --- | --- | --- |
| Intermittent | Fixed (0%) | Fixed (70%) |
| Nuclear & CCS | Variable | Fixed (85%) |
| Interconnectors | Fixed (0%) | Fixed (100%) |
| Hydro | Variable | Variable (>10%) |
| Pumped Storage | Variable | Fixed (50%) |
| Peaking | Variable | Fixed (0%) |
| Other (Conventional) | Variable | Variable (>10%) |

14.21 includes an example of adjusting scaling factors to ensure compliance with the floor.

The base scaling factors and generation plant types are set out in the Security Standard. These may be reviewed from time to time. The latest version will be used in the calculation of TNUoS tariffs and is published in the Statement of Use of System Charges.

- The Company will categorise plant based on the categorisations described in the Security Standard. Peaking plant will include oil and OCGT technologies and Other (Conv.) represents all remaining conventional plant not explicitly stated elsewhere in the table. In the event that a power station is made up of more than one technology type, the type of the higher Transmission Entry Capacity (TEC) would apply.

- Nodal net demand data for the transport model will be based upon the GSP net demand that Users have forecast to occur at the time of Annual Average Cold Spell (ACS) Demand for year "t" in the April Electricity Ten Year Statement (ETYS) for year "t-1" plus updates to the October of year "t-1".

- Subject to paragraphs 14.15.15 to 14.15.22, Transmission circuits for Financial Year "t" will be defined as those with existing wayleaves for the year "t" with the associated lengths based on the circuit lengths indicated for year "t" in the April NETS Electricity Ten Year Statement (ETYS) for year "t-1" plus updates to October of year "t-1".  If certain circuit information is not explicitly contained in the NETS  Electricity Ten Year Statement (ETYS), The Company will use the best information available.

- The circuit lengths included in the transport model are solely those, which relate to assets defined as 'Use of System' assets.

- For HVDC circuits, the impedance will be calculated to provide flows based on a ratio of the capacity provided by the HVDC link relative to the capacities on all major transmission system boundaries that it parallels.

- The transport model employs the use of circuit expansion factors to reflect the difference in cost between (i) AC Circuits and HVDC circuits, (ii) underground and sub-sea circuits, (iii) cabled circuits and overhead line circuits, (iv) 132kV and 275kV circuits, (v) 275kV circuits and 400kV circuits, and (vi) uses 400kV overhead line (i.e. the 400kV overhead line expansion factor is 1).  As the transport model expresses cost as marginal km (irrespective of cables or overhead lines), some account needs to be made of the fact that investment in these other types of circuit (specifically HVDC and sub-sea cables of various voltages, 400kV underground cable, 275kV overhead line, 275kV underground cable, 132kV overhead line and 132kV underground cable) is more expensive than for 400kV overhead line. This is done by effectively 'expanding' these more expensive circuits by the relevant circuit expansion factor, thereby producing a larger marginal kilometre to reflect the additional cost of investing in these circuits compared to 400kV overhead line.  When calculating the local circuit tariff for a generator, alternative 132kV and offshore expansion factors to those used in the remainder of the tariff calculation are applied to the generator’s local circuits.

- The circuit expansion factors for HVDC circuits and AC subsea cables are determined on a case by case basis using the costs which are specific to individual projects containing HVDC or AC subsea circuits.

Adjustments to Model Inputs associated with One-off Works

- Where, following the implementation of CUSC Modification CMP203, a User has paid a One-Off Charge that related to One-off Works carried out on an onshore circuit, and such One-off Works would affect the value of a TNUoS tariff paid by the User, the transport model inputs associated with the onshore circuit shall be adjusted by The Company to reflect the asset value that would have been modelled if the works had been undertaken on the basis of the original asset design rather than the One-off Works.

- Subject to paragraphs 14.15.17 to 14.15.19, where, prior to the implementation of CUSC Modification CMP203, a User has paid a One-Off Charge (or has paid a charge to the relevant TO prior to 1st April 2005 on the same principles as a One-Off Charge) that related to works equivalent to those described under paragraph 14.15.15, an adjustment equivalent to that under paragraph 14.15.15 shall be made to the transport model inputs as follows.

- Such adjustment shall be made following a User’s request, which must be received by The Company no later than the second occurrence of 31st December following the implementation of CUSC Modification CMP203.

- The Company shall only make an adjustment to the transport model inputs, under paragraph 14.15.16 where the charge was paid to the relevant TO prior to 1st April 2005 where evidence has been provided by the User that satisfies The Company that works equivalent to those under paragraph 14.15.15 were funded by the User.

- Where a User has sufficient reason to believe that adjustments under paragraph 14.15.18 should be made in relation to specific assets that affect a TNUoS tariff that applies to one of its sites and outlines its reasoning to The Company, The Company shall (upon the User’s request and subject to the User’s payment of reasonable costs incurred by The Company in doing so) use its reasonable endeavours to assist the User in obtaining any evidence The Company or a TO may have to support its position.

- Where a request is made under paragraph 14.15.16 on or prior to 31st December in a Financial Year, and The Company is satisfied based on the accompanying evidence provided to The Company under paragraph 14.15.17 that it is a valid request, the transport model inputs shall be adjusted accordingly and taken into account in the calculation of TNUoS tariffs effective from the year commencing on the 1st April following this and otherwise from the next subsequent 1st April.

- The following table provides examples of works for which adjustments to transport model inputs would typically apply:

| Ref | Description of works | Adjustments |
| --- | --- | --- |
| 1 | Undergrounding - A User requests to underground an overhead line at a greater cost. | As the cable cost will be more expensive than the overhead line (OHL) equivalent, the circuit will be modelled as an OHL. |
| 2 | Substation Siting Decision - A User requests to move the existing or a planned substation location to a place that means that the works cannot be justified as economic by the TO. | As the revised substation location may result in circuits being extended. If this is the case, the originally designed circuit lengths (as per the originally designed substation location) would be used in the transport model. |
| 3 | Circuit Routing Decision - A User asks to move an existing or a planned circuit route in a way in which the works cannot be justified as economic by the TO. | As any circuit route changes that extend circuits are likely to result in a greater TNUoS tariff, the originally designed circuit lengths would be used in the transport model. |
| 4 | Building circuits at lower voltages - A User requests lower tower height and therefore a different voltage. | As lower voltage circuits result in a higher expansion factor being used, the circuits would be modelled at the originally designed higher voltage. |

- The following table provides examples of works for which adjustments to transport model typically would not apply:

| Ref | Description of works | Reasoning |
| --- | --- | --- |
| 1 | Undergrounding - A User chooses to have a cable installed via a tunnel rather than buried. | Cable expansion factors are applied in the transport model regardless of whether a cable is tunnelled and buried, so there is no increased TNUoS cost. |
| 2 | Additional circuit route works - A User asks for screening to be provided around a new or existing circuit route. | Circuit expansion factors are applied in the transport model irrespective of these works, so there is no increased TNUoS cost. |
| 3 | Additional circuit route works - A User requests that a planned overhead line route is built using alternative transmission tower designs. | Circuit expansion factors are applied in the transport model irrespective of these works, so there is no increased TNUoS cost. |
| 4 | Additional substation works - A User asks for screening to be provided around a new or existing substation. | The additional substation works will not affect the User’s TNUoS charge as there is no effect on power flows or circuit costs within the transport model. |
| 5 | Additional substation works - Changes to connection assets (e.g. HV-LV transformers and associated switchgear), metering, additional LV supplies, additional protection equipment, additional building works, etc. | The additional substation works will not affect the User’s TNUoS charge as there is no effect on power flows or circuit costs within the transport model. |
| 6 | Diversion - A User asks to temporarily move an existing or a planned circuit route in a way in which the works cannot be justified as economic by the TO. | The temporary circuit changes will not be incorporated into the transport model. |
| 7 | Connection Entry Capacity (CEC) before Transmission Entry Capacity (TEC). A User asks for a connection in a year prior to the relating TEC; i.e. physical connection without capacity. | No additional works are being undertaken, works are simply being completed well in advance of the generator commissioning. The One-Off Charge reflects the depreciated value of the assets prior to commissioning (and any TNUoS being charged). |
| 8 | Early asset replacement - An asset is replaced prior to the end of its expected life. | As the asset is simply replaced, no data in the transport model is expected to change. |
| 9 | Additional Engineering/ Mobilisation costs - A User requests changes to the planned works, that results in additional operational costs. | The data in the transport model is unaffected. |
| 10 | Offshore (Generator Build) - Any of the works described above or under paragraph 14.15.18. | The value of the works will not form part of the asset transfer value therefore will not be used as part of the offshore tariff calculation. |
| 11 | Offshore (Offshore Transmission Owner (OFTO) Build) - Any of the works described above or under paragraph 14.15.18. | As part of determining the TNUoS revenue associated with each asset, the value of the One-Off Works would be excluded when pro-rating the OFTO’s allowed revenue against assets by asset value. |

- The Company shall publish any adjusted transport model inputs that it intends to use in the calculation of TNUoS tariffs effective from the year commencing on the following 1st April in the NETS Electricity Ten Year Statement (ETYS) October Update. Any further adjustments that The Company makes shall be published by The Company upon the publication of the final TNUoS tariffs for the year concerned.

### Model Outputs

- The transport model takes the inputs described above and carries out the following steps individually for Peak Security and Year Round backgrounds.

- Depending on the background, the TEC of the relevant generation plant types are scaled by a percentage as described in 14.15.7, above. The TEC of the remaining generation plant types in each background are uniformly scaled such that total national generation (scaled sum of contracted TECs) equals total national ACS Demand.

- For each background, the model then uses a DCLF ICRP transport algorithm to derive the resultant pattern of flows based on the network impedance required to meet the nodal net demand using the scaled nodal generation, assuming every circuit has infinite capacity. Flows on individual transmission circuits are compared for both backgrounds and the background giving rise to the highest flow is considered as the triggering criterion for future investment of that circuit for the purposes of the charging methodology. Therefore all circuits will be tagged as Peak Security or Year Round depending upon the background resulting in the highest flow. In the event that both backgrounds result in the same flow, the circuit will be tagged as Peak Security. Then it calculates the resultant total network Peak Security MWkm and Year Round MWkm, using the relevant circuit expansion factors as appropriate.

- Using these baseline networks for Peak Security and Year Round backgrounds, the model then calculates for a given injection of 1MW of generation at each node, with a corresponding 1MW offtake (net demand) distributed across all demand nodes in the network, the increase or decrease in total MWkm of the whole Peak Security and Year Round networks. The proportion of the 1MW offtake allocated to any given demand node will be based on total background nodal net demand in the model. For example, with a total net GB demand of 60GW in the model, a node with a net demand of 600MW would contain 1% of the offtake i.e. 0.01MW.

- Given the assumption of a 1MW injection, for simplicity the marginal costs are expressed solely in km.  This gives a Peak Security marginal km cost and a Year Round marginal km cost for generation at each node (although not that used to calculate generation tariffs which considers local and wider cost components).  The Peak Security and Year Round marginal km costs for demand at each node are equal and opposite to the Peak Security and Year Round nodal marginal km respectively for generation and this is used to calculate demand tariffs.  Note the marginal km costs can be positive or negative depending on the impact the injection of 1MW of generation has on the total circuit km.

- Using a similar methodology as described above in 14.15.27, the local and wider marginal km costs used to determine generation TNUoS tariffs are calculated by injecting 1MW of generation against the node(s) the generator is modelled at and increasing by 1MW the offtake across the distributed reference node.  It should be noted that although the wider marginal km costs are calculated for both Peak Security and Year Round backgrounds, the local marginal km costs are calculated on the Year Round background.

- In addition, any circuits in the model, identified as local assets to a node will have the local circuit expansion factors which are applied in calculating that particular node’s marginal km. Any remaining circuits will have the TO specific wider circuit expansion factors applied.

- An example is contained in 14.21 Transport Model Example.

## Calculation of local nodal marginal km

- In order to ensure assets local to generation are charged in a cost reflective manner, a generation local circuit tariff is calculated. The nodal specific charge provides a financial signal reflecting the security and construction of the infrastructure circuits that connect the node to the transmission system.

- Main Interconnected Transmission System (MITS) nodes are defined as:

- Grid Supply Point connections with 2 or more transmission circuits connecting at the site; or

- connections with more than 4 transmission circuits connecting at the site.

- Where a Grid Supply Point is defined as a point of supply from the National Electricity Transmission System to network operators or non-embedded customers excluding generator or interconnector load alone. For the avoidance of doubt, generator or interconnector load would be subject to the circuit component of its Local Charge. A transmission circuit is part of the National Electricity Transmission System between two or more circuit-breakers which includes transformers, cables and overhead lines but excludes busbars and generation circuits.

- Generators directly connected to a MITS node will have a zero local circuit tariff.

- Generators not connected to a MITS node will have a local circuit tariff derived from the local nodal marginal km for the generation node i.e. the increase or decrease in marginal km along the transmission circuits connecting it to all adjacent MITS nodes (local assets).

## Calculation of zonal marginal km

- Given the requirement for relatively stable cost messages through the ICRP methodology and administrative simplicity, nodes are assigned to zones. The currently applicable number of generation zones is detailed in the Statement of Use of System Charges which is available from the Website.

- Demand zone boundaries have been fixed and relate to the GSP Groups used for energy market settlement purposes.

- The nodal marginal km are amalgamated into zones by weighting them by their relevant generation or demand capacity.

- Generators will have zonal tariffs derived from both, the wider Peak Security nodal marginal km; and the wider Year Round nodal marginal km for the generation node calculated as the increase or decrease in marginal km along all transmission circuits except those classified as local assets.

The zonal Peak Security marginal km for generation is calculated as:

Where

Gi	=	Generation zone

j	=	Node

NMkmPS	=	Peak Security Wider nodal marginal km from transport model

WNMkmPS	=	Peak Security Weighted nodal marginal km

ZMkmPS	=	Peak Security Zonal Marginal km

Gen	=	Nodal Generation (scaled by the appropriate Peak Security Scaling factor) from the transport model

Similarly, the zonal Year Round marginal km for generation is calculated as

Where

NMkmYR	=	Year Round Wider nodal marginal km from transport model

WNMkmYR	=	Year Round Weighted nodal marginal km

ZMkmYR	=	Year Round Zonal Marginal km

Gen                =	Nodal Generation (scaled by the appropriate Year Round Scaling factor) from the transport model

- The zonal Peak Security marginal km for demand zones are calculated as follows. If Nodal Demand from a node is less than 0 (Exporting) the nodal demand will be set to zero and therefore not contribute to the Zonal marginal km

Where:

Di	=	Demand zone

Dem	=	Positive Nodal Net Demand from transport model

Similarly, the zonal Year Round marginal km for demand zones are calculated as follows:

- The number of generation zones will be fixed to 27 zones and the assignment of existing relevant nodes to these 27 generation zones will be fixed to those that are effective as of 31st March 2021 based on methodology in effect during the 2020/21. Relevant nodes are considered to be those with generation connected to them.  Financial Year. Any newly created relevant nodes will be assigned to one of the 27 generation zones.

- Not Used

- Not Used

- Not Used

Accounting for Sharing of Transmission by Generators

- A proportion of the marginal km costs for generation are shared incremental km reflecting the ability of differing generation technologies to share transmission investment.  This is reflected in charges through the splitting of Year Round marginal km costs for generation  into Year Round Shared marginal km costs and Year Round Not-Shared marginal km which are then used in the calculation of the wider £/kW generation tariff.

- The sharing between different generation types is accounted for by (a) using transmission network boundaries between generation zones set by connectivity between generation charging zones, and (b) the proportion of Low Carbon and Carbon generation behind these boundaries.

- The zonal incremental km for each generation charging zone is split into each boundary component by considering the difference between it and the neighbouring generation charging zone using the formula below;

Where;

BIkmab	=	boundary incremental km between generation charging zone A and generation charging zone B

ZIkm	=	generation charging zone incremental km.

- The table below shows the categorisation of Low Carbon and Carbon generation. This table will be updated by The Company in the Statement of Use of System Charges as new generation technologies are developed.

| Carbon | Low Carbon |
| --- | --- |
| Coal | Wind |
| Gas | Hydro (excl. Pumped Storage) |
| Biomass | Nuclear |
| Oil | Marine |
| Pumped Storage | Tidal |
| Interconnectors |  |

Determination of Connectivity

- Connectivity is based on the existence of electrical circuits between TNUoS generation charging zones that are represented in the Transport model.  Where such paths exist, generation charging zones will be effectively linked via an incremental km transmission boundary length.  These paths will be simplified through in the case of;

- Parallel paths – the longest path will be taken.  An illustrative example is shown below with x, y and z representing the incremental km between zones.

- Parallel zones – parallel zones will be amalgamated with the incremental km immediately beyond the amalgamated zones being the greater of those existing prior to the amalgamation.  An illustrative example is shown below with a, b, c, and d representing the initial incremental km between zones, and x and y representing the final incremental km following zonal amalgamation.

- An illustrative Connectivity diagram is shown below:

The arrows connecting generation charging zones and amalgamated generation charging zones represent the incremental km transmission boundary lengths towards the notional centre of the system.  Generation located in charging zones behind arrows is considered to share based on the ratio of Low Carbon to Carbon cumulative generation TEC within those zones.

- The Company will review Connectivity at the beginning of a new price control period, and under exceptional circumstances such as major system reconfigurations.  If any such reassessment is required, it will be undertaken against a background of minimal change to existing Connectivity and in line with the notification process set out in the ESO Licence, the Transmission Licence and the CUSC.

Calculation of Boundary Sharing Factors

- Boundary sharing factors (BSFs) are derived from the comparison of the cumulative proportion of Low Carbon and Carbon generation TEC behind each of the incremental MWkm boundary lengths using the following formulae –

If , then all Year round marginal km costs are shared i.e. the BSF is 100%.

Where:

LC	=	Cumulative Low Carbon generation TEC behind the relevant transmission boundary

C	=	Cumulative Carbon generation TEC behind the relevant transmission boundary

If   then the BSF is calculated using the following formula: -

Where:

BSF	=	boundary sharing factor.

- The shared incremental km for each boundary are derived from the multiplication of the boundary sharing factor by the incremental km for that boundary;

Where;

SBIkmab	=	shared boundary incremental km between generation charging zone A and generation charging zone B

BSFab	=	generation charging zone boundary sharing factor.

- The shared incremental km is discounted from the incremental km for that boundary to establish the not-shared boundary incremental km.  The not-shared boundary incremental km reflects the cost of transmission investment on that boundary accounting for the sharing of power stations behind that boundary.

Where;

NSBIkmab	=	not shared boundary incremental km between generation charging zone A and generation charging zone B.

- The shared incremental km for a generation charging zone is the sum of the appropriate shared boundary incremental km for that generation charging zone as derived from the connectivity diagram.

Where;

ZMkmnYRS	=	Year Round Shared Zonal Marginal km for generation charging zone n.

- The not-shared incremental km for a generation charging zone is the sum of the appropriate not-shared boundary incremental km for that generation charging zone as derived from the connectivity diagram.

Where;

ZMkmnYRNS	=	Year Round Not-Shared Zonal Marginal km for generation zone n.

## Deriving the Final Local £/kW Tariff and the Wider £/kW Tariff

- The zonal marginal km (ZMkmGi) are converted into costs and hence a tariff by multiplying by the Expansion Constant and the Locational Security Factor (see below). The nodal local marginal km (NLMkmL) are converted into costs and hence a tariff by multiplying by the Expansion Constant and a Local Security Factor.

### The Expansion Constant

- The expansion constant, expressed in £/MWkm, represents the annuitised value of the transmission infrastructure capital investment required to transport 1 MW over 1 km.  Its magnitude is derived from the projected cost of 400kV overhead line, including an estimate of the cost of capital, to provide for future system expansion.

- In the methodology, the expansion constant is used to convert the marginal km figure derived from the transport model into a £/MW signal. The tariff model performs this calculation, in accordance with 14.15.96 – 14.15.121, and also then calculates the residual element of the overall tariff (to ensure correct revenue recovery in accordance with the price control), in accordance with 14.15.137.

- The transmission infrastructure capital costs used in the calculation of the expansion constant are provided via an externally audited process. They also include information provided from all onshore Transmission Owners (TOs). They are based on historic costs and tender valuations adjusted by a number of indices (e.g. global price of steel, labour, inflation, etc.). The objective of these adjustments is to make the costs reflect current prices, making the tariffs as forward looking as possible.  This cost data represents The Company’s best view; however it is considered as commercially sensitive and is therefore treated as confidential. The calculation of the expansion constant also relies on a significant amount of transmission asset information, much of which is provided in the  Electricity Ten Year Statement (ETYS).

- For each circuit type and voltage used onshore, an individual calculation is carried out to establish a £/MWkm figure, normalised against the 400KV overhead line (OHL) figure, these provide the basis of the onshore circuit expansion factors discussed in 14.15.70 – 14.15.77.  In order to simplify the calculation a unity power factor is assumed, converting £/MVAkm to £/MWkm. This reflects that the fact tariffs and charges are based on real power.

- The table below shows the first stage in calculating the onshore expansion constant.  A range of overhead line types is used and the types are weighted by recent usage on the transmission system. This is a simplified calculation for 400kV OHL using example data:

| 400kV OHL expansion constant calculation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MW | Type | £(000)/km | Circuit km* | £/MWkm | Weight |
| A | B | C | D | E = C/A | F=E*D |
| 6500 | La | 700 | 500 | 107.69 | 53846 |
| 6500 | Lb | 780 | 0 | 120.00 | 0 |
| 3500 | La/b | 600 | 200 | 171.43 | 34286 |
| 3600 | Lc | 400 | 300 | 111.11 | 33333 |
| 4000 | Lc/a | 450 | 1100 | 112.50 | 123750 |
| 5000 | Ld | 500 | 300 | 100.00 | 30000 |
| 5400 | Ld/a | 550 | 100 | 101.85 | 10185 |
| Sum |  |  | 2500 (G) |  | 285400 (H) |
|  |  |  |  | Weighted Average (J= H/G): | 114.160 (J) |
| *These are circuit km of types that have been provided in the previous 10 years. If no information is available for a particular category the best forecast will be used. |  |  |  |  |  |

- The weighted average £/MWkm (J in the example above) is then converted in to an annual figure by multiplying it by an annuity factor. The formula used to calculate of the annuity factor is shown below:

- The Weighted Average Cost of Capital (WACC) and asset life are established at the start of a price control and remain constant throughout a price control period. The WACC used in the calculation of the annuity factor is NGET’s regulated rate of return, this assumes that it will be reasonably representative of all licensees. The asset life used in the calculation is 50 years; the appropriateness of this is reviewed when the annuity factor is recalculated at the start of a price control period.  These assumptions, applied in accordance with 14.15.64, provide a current annuity factor, as set out in the Statement of Use of System Charges which is available from the Website.

- The final step in calculating the expansion constant is to add a share of the annual transmission overheads (maintenance, rates etc). This is done by multiplying the average weighted cost (J) by an ‘overhead factor’. The ‘overhead factor’ represents the total business overhead in any year divided by the total Gross Asset Value (GAV) of the transmission system. This is recalculated at the start of each price control period. The currently applicable overhead factor used in the calculation of the current expansion constant is, calculated as above, and detailed in the Statement of Use of System Charges which is available from the Website. The overhead and annuitised costs are then added to give the expansion constant.

- Using the previous example, and the 2009/10 values for the annuity factor (6.6%) and overhead factor (1.8%), the final steps in establishing the expansion constant are demonstrated below:

| 400kV OHL expansion constant calculation | Ave £/MWkm |
| --- | --- |
| OHL | 114.160 |
| Annuitised | 7.535 |
| Overhead | 2.055 |
| Final | 9.589 |

- This process is carried out for each voltage onshore, along with other adjustments to take account of upgrade options, see 14.15.73, and normalised against the 400KV overhead line cost (the expansion constant) the resulting ratios provide the basis of the onshore expansion factors.  The process used to derive circuit expansion factors for Offshore Transmission Owner networks is described in 14.15.80.

- This process of calculating the incremental cost of capacity for a 400kV OHL, along with calculating the onshore expansion factors is carried out for the first year of the price control and is increased by inflation, TOPI, (May–October average increase, as defined in the ESO Licence and/or Transmission Licence) each subsequent year of the price control period.  The currently applicable expansion constant is detailed in the Statement of Use of System Charges which is available from the Website.

14.15.69A 	Notwithstanding Paragraph 14.15.69 from the first year of (and during) the T2 price control (which starts on 1st April 2021), until a further change is made, the Expansion Constant will be that used in the 2020/21 Financial Year inflated in accordance with TOPI as per paragraph 14.15.69; and plus inflation as defined in the ESO License and/or Transmission Licence for each subsequent year of the T2 price control.

Onshore Wider Circuit Expansion Factors

- Base onshore expansion factors are calculated by deriving individual expansion constants for the various types of circuit, following the same principles used to calculate the 400kV overhead line expansion constant. The factors are then derived by dividing the calculated expansion constant by the 400kV overhead line expansion constant. The factors will be fixed for each respective price control period.

- In calculating the onshore underground cable factors, the forecast costs are weighted equally between urban and rural installation, and direct burial has been assumed. The operating costs for cable are aligned with those for overhead line. An allowance for overhead costs has also been included in the calculations.

- The 132kV onshore circuit expansion factor is applied on a TO basis. This is to reflect the regional variation of plans to rebuild circuits at a lower voltage capacity to 400kV. The 132kV cable and line factor is calculated on the proportion of 132kV circuits likely to be uprated to 400kV. The 132kV expansion factor is then calculated by weighting the 132kV cable and overhead line costs with the relevant 400kV expansion factor, based on the proportion of 132kV circuitry to be uprated to 400kV. For example, in the TO areas of NGET and Scottish Power where there are no plans to uprate any 132kV circuits, the full cable and overhead line costs of 132kV circuit are reflected in the 132kV expansion factor calculation.

- The 275kV onshore circuit expansion factor is applied on a GB basis and includes a weighting of 83% of the relevant 400kV cable and overhead line factor. This is to reflect the averaged proportion of circuits across all three Onshore Transmission Licensees which are likely to be uprated from 275kV to 400kV across GB within a price control period.

- The 400kV onshore circuit expansion factor is applied on a GB basis and reflects the full costs for 400kV cable and overhead lines.

- AC sub-sea cable and HVDC circuit expansion factors (Specific Circuit Expansion Factors) are calculated on a case by case basis using actual project costs net of any cost adjustments.

- Calculation of Specific Circuit Expansion Factors shall include only: the cost of the converters (where applicable); and the cost of the cable; and a percentage of the total overhead project costs, defined as the combined costs of the cables and converters (as relevant) divided by the total capital cost of the project minus a percentage of the cost adjustment, defined as the combined costs of the cables, converters (as relevant) and appropriate overhead costs, as calculated above, all divided by the total capital cost of the project.

Specific Circuit Expansion Factors in RIIO-T3

14.15.76A)  The Specific Circuit Expansion Factors calculated as of 1 April 2025 and used in the 2025/26 Financial Year will remain fixed and be used in subsequent Financial Years until a further change is made. For clarity, the Specific Expansion Factors calculated and applied after 1st April 2025 will remain fixed once calculated, until a further change is made.

- The TO specific onshore circuit expansion factors which are currently applicable, are detailed in the Statement of Use of System Charges which is available from the Website.

Onshore Local Circuit Expansion Factors

- The local onshore circuit tariff is calculated using local onshore circuit expansion factors. These expansion factors are calculated using the same methodology as the onshore wider expansion factor but without taking into account the proportion of circuit kms that are planned to be uprated.

- In addition, the 132kV onshore overhead line circuit expansion factor is sub divided into four more specific expansion factors. This is based upon maximum (winter) circuit continuous rating (MVA) and route construction whether double or single circuit. The 132kV onshore overhead line circuit expansion factors which are currently applicable, are detailed in the Statement of Use of System Charges which is available from the Website.

### Onshore Expansion Factors in RIIO-T2

### Notwithstanding Paragraph 14.15.69, the previous paragraphs and following the same intent as adopted at Paragraph 14.15.69A, from the first year of (and during) the T2 price control (which starts on 1st April 2021), until a further change is made, the  Onshore expansion factors (being the Onshore local circuit factors and the Onshore wider circuit expansion factors, except those used for HVDC circuits and sub-sea AC cable) will be the value used in the 2020/21 Financial Year. For clarity HVDC circuits and sub-sea AC cable will continue to be calculated in accordance with 14.15.75.

### Offshore Circuit Expansion Factors

- Offshore expansion factors (£/MWkm) are derived from information provided by Offshore Transmission Owners for each offshore circuit.  Offshore expansion factors are Offshore Transmission Owner and circuit specific.  Each Offshore Transmission Owner will periodically provide, via the STC, information to derive an annual circuit revenue requirement.  The offshore circuit revenue shall include revenues associated with the Offshore Transmission Owner’s reactive compensation equipment, harmonic filtering equipment, asset spares and HVDC converter stations.

- In the year that the offshore transmission assets are transferred to the Offshore Transmission Owner, the offshore circuit expansion factor would be calculated as follows:

Where:

CRevOFTO1	=	The offshore circuit revenue in £ for Year 1

L	=	The total circuit length in km of the offshore circuit

CircRat	=	The continuous rating of the offshore circuit

- In all subsequent years, the offshore circuit expansion factor would be calculated as follows:

Where:

AvCRevOFTO	=	The annual offshore circuit revenue averaged over the remaining years of the onshore National Electricity Transmission System Operator (NETSO) price control

L	=	The total circuit length in km of the offshore circuit

CircRat	=	The continuous rating of the offshore circuit

- For the avoidance of doubt, the offshore circuit revenue values, CRevOFTO1 and AvCRevOFTO shall be determined using asset values after the removal of any One-Off Charges.

- Prevailing Offshore Transmission Owner specific expansion factors will be published in the Statement of Use of System Charges which is available from the Website. These shall be recalculated for the start of each price control period using the formula in paragraph 14.15.82. For each subsequent year within the price control period, these expansion factors will be adjusted by the annual Offshore Transmission Owner specific indexation factor, OFTOInd, calculated as follows;

where:

OFTOIndt,f	=	the indexation factor for Offshore Transmission Owner f in respect of Financial Year t;

OFTORevIndt,f  	=	the indexation rate applied to the revenue of Offshore Transmission Owner f under the terms of its transmission licence in respect of Financial Year t; and

TOPIt 	=	the indexation rate applied to the expansion

constant in respect of Financial Year t.

Offshore Interlinks

14.15.85 	The revenue associated with an Offshore Interlink shall be divided entirely between those Generators benefiting from the installation of that Offshore Interlink. Each of these Users will be responsible for their charge from their charging date, meaning that a proportion of the Offshore Interlink revenue may be socialised prior to all relevant Users being chargeable. The proportion associated with each User will be based on the Measure of Capacity to the MITS using the Offshore Interlink(s) in the event of a single circuit fault on the User’s circuit from their offshore substation towards the shore, compared to the Measure of Capacity of the other Users.

Where:

An Offshore Interlink is a circuit which connects two offshore substations that are connected to a Single Common Substation. It is held in open standby until there is a transmission fault that limits the User’s ability to export power to the Single Common Substation. In the Transport Model, they are to be modelled in open standby.

A Single Common Substation is a substation where:

i. 	each substation that is connected by an Offshore Interlink is connected via at least one circuit without passing through another substation; and

ii. 	all routes connecting each substation that is connected by an Offshore Interlink to the MITS pass through.

The Measure of Capacity to the MITS for each Offshore substation is the result of the following formula or zero whichever is larger. For the situation with only one interlink, all terms relating to C should be set to zero:

For Substation A:

min { CapIAB, ILFA × TECA - RCapA, CapB - ILFB × TECB + min (CapIBC, CapC - ILFC × TECC ) }

For Substation B:

min { ILFB × TECB - RCapB, min (CapIAB, CapA - ILFA × TECA )

+ min ( CapIBC, CapC - ILFC × TECC) }

For Substation C:

min { CapIBC, ILFC × TECC - - RCapC, CapB - ILFB × TECB + min (CapIAB, CapA – ILFA × TECA ) }

and

CapIAB 	= 	total capacity of the Offshore Interlink between substations A and B

CapIBC 	= 	total capacity of the Offshore Interlink between substations B and C

CapX    = 	total capacity of the circuit between offshore substation X and the Single Common Substation, where X is A, B or C.

RCapX  =	remaining capacity of the circuit between offshore substation X and the Single Common Substation in the event of a single cable fault, where X is A, B or C.

TECX  = 	the sum of the TEC for the Users connected, or contracted to connect, to offshore substation X, where X is A, B or C, where the value of TEC will be the maximum TEC that each User has held since the initial charging date, or is contracted to hold if prior to the initial charging date.

ILFX	=	Offshore Interlink Load Factor, where X is A, B or C.

The Offshore Interlink Load Factor (ILF) is based on the Annual Load Factor (ALF).  Until all the Users connected to a Single Common Substation have a station specific Annual Load Factor based on five years of data, the generic ALF for the fuel type will be used as the ILF for all stations.  When all Users have a station specific ALF, the value of the ALF in the first such year will be used as the ILF in the calculation for all subsequent Financial Years.

14.15.86 The apportionment of revenue associated with Offshore Interlink(s) in 14.15.85 applies in situations where the Offshore Interlink was included in the design phase, or if one or more User(s) has already financially committed or been commissioned then only where that User(s) agrees to the Offshore Interlink.

14.15.87  Alternatively to the formula specified in 14.15.85 the proportion of the OFTO revenue associated with the Offshore Interlink allocated to each generator benefiting from the installation of an Offshore Interlink may be agreed between these Users. In this event:

a. 	All relevant Users shall notify The Company of its respective proportions three months prior the OTSDUW asset transfer in the case of a generator build, or the charging date of the first generator, in the case of an OFTO build.

b. 	All relevant Users may agree to vary the proportions notified under (a)   by each writing to The Company three months prior to the charges being set for a given Financial Year.

c. 	Once a set of proportions of the OFTO revenue associated with the Offshore Interlink has been provided to The Company, these will apply for the next and future Financial Years unless and until The Company is informed otherwise in accordance with (b) by all of the relevant Users.

d.	If all relevant Users are unable to reach agreement on the proportioning of the OFTO revenue associated with the Offshore Interlink they can raise a dispute. Any dispute between two or more Users as to the proportioning of such revenue shall be managed in accordance with CUSC Section 7 Paragraph 7.4.1 but the reference to the ‘London Court of International Arbitration’ shall instead be to the ‘Authority’ and the Authority’s determination of such dispute shall, without prejudice to apply for judicial review of any determination, be final and binding on the Users.

### The Locational Onshore Security Factor

- The locational onshore security factor for everything other than Identified Onshore Circuits is derived by running a secure DCLF ICRP transport study of the network excluding local circuits and Identified Onshore Circuits based on the same market background as used for Zoning in the DCLF ICRP transport model. This calculates the nodal marginal costs where peak net demand can be met despite the Security and Quality of Supply Standard contingencies (simulating single and double circuit faults) on the network. Essentially the calculation of secured nodal marginal costs is identical to the process outlined above except that the secure DCLF study additionally calculates a nodal marginal cost taking into account the requirement to be secure against a set of worse case contingencies in terms of maximum flow for each circuit.

- For the purposes of 14.15.88 the secured nodal cost differential is compared to that produced by the DCLF ICRP transport model and the resultant ratio of the two determines the locational security factor using the Least Squares Fit method. Further information may be obtained from the Website.

- For the purposes of 14.15.88 the locational onshore security factor, derived in accordance with paragraphs 14.15.88 and 14.15.89 and expressed to two decimal places, is based on an average from a number of studies conducted by The Company to account for future network developments. This security factor is reviewed for each price control period and fixed for the duration. The locational onshore security factor which is currently applicable, is detailed in the Statement of Use of System Charges.

- An Identified Onshore Circuit shall be defined as a single transmission HVDC subsea circuit or a single transmission AC subsea circuit between two MITS Nodes where there is only one route for the power to flow between the two MITS Nodes. The expansion factors for Identified Onshore Circuits are adjusted by dividing the applicable expansion factor for the Identified Onshore Circuits, calculated as per Sections 14.15.70 to 14.15.77, by the locational onshore security factor calculated in 14.15.90. When the locational onshore security factor is applied as per Section 14.15.94 and 14.15.95, this would result in an effective locational onshore security factor for Identified Onshore Circuits of 1.0.

Local Security Factors

- Local onshore security factors are generator specific and are applied to a generator’s local onshore circuits.  If the loss of any one of the local circuits prevents the export of power from the generator to the MITS then a local security factor of 1.0 is applied. For generation with circuit redundancy, a local security factor is applied that is equal to the locational security factor, derived in accordance with paragraphs 14.15.88 and 14.15.90.

- Where a Transmission Owner has designed a local onshore circuit (or otherwise that circuit once built) to a capacity lower than the aggregated TEC of the generation using that circuit, then the local security factor of 1.0 will be multiplied by a Counter Correlation Factor (CCF) as described in the formula below;

Where; Dmin = minimum annual net demand (MW) supplied via that circuit in the absence of that generation using the circuit

Tcap = transmission capacity built (MVA)

Gcap = aggregated TEC of generation using that circuit

CCF cannot be greater than 1.0.

- A specific offshore local security factor (LocalSF) will be calculated for each offshore connection using the following methodology:

Where:

NetworkExportCapacity =	the total export capacity of the network disregarding any Offshore Interlinks

k	    = 	the generation connected to the offshore network

14.15.94	The local offshore security factor for single circuits with a single cable will be 1.0 and for multiple circuit connections will be capped at the locational onshore security factor, derived in accordance with 14.15.88-14.15.90.

14.15.95 	The offshore local security factor for configurations with one or more Offshore Interlinks is updated so that the offshore circuit tariff will include the proportion of revenue associated with the Offshore Interlink(s). The specific offshore local security factor for configurations involving an Offshore Interlink, which may be greater than the locational onshore security factor, will be calculated for each offshore connection using the following methodology:

Where:

IRevOFTO =  The appropriate proportion of the Offshore Interlink(s) revenue in £ associated with the offshore connection calculated in 14.15.85

CRevOFTO =  The offshore circuit revenue in £ associated with the circuit(s) from the offshore substation to the Single Common Substation.

LocalSFinitial =    Initial Local Security Factor calculated in 14.15.93 and 14.15.94

And other definitions as in 14.15.93.

### Initial Transport Tariff

- First an Initial Transport Tariff (ITT) must be calculated for both Peak Security and Year Round backgrounds. For Generation, the Peak Security zonal marginal km (ZMkmPS), Year Round Not-Shared zonal marginal km (ZMkmYRNS) and Year Round Shared zonal marginal km (ZMkmYRS) are simply multiplied by the expansion constant and the locational security factor to give the Peak Security ITT, Year Round Not-Shared ITT and Year Round Shared ITT respectively:

Where

ZMkmGiPS	=	Peak Security Zonal Marginal km for each generation zone		ZMkmGiYRNS	=	Year Round Not-Shared Zonal Marginal km for each generation     charging zone

ZMkmGiYRS	=	Year Round Shared Zonal Marginal km for each generation charging zone

EC		=	Expansion Constant

LSF		=	Locational Security Factor

ITTGiPS	=	Peak Security Initial Transport Tariff (£/MW) for each generation zone

ITTGiYRNS	=	Year Round Not-Shared Initial Transport Tariff (£/MW) for each generation charging zone

ITTGiYRS	=	Year Round Shared Initial Transport Tariff (£/MW) for each generation charging zone.

- Similarly, for demand the Peak Security zonal marginal km ( ZMkmPS) and Year Round zonal marginal km (ZMkmYR) are simply multiplied by the expansion constant and the locational security factor to give the Peak Security ITT and Year Round ITT respectively:

Where

ZMkmDiPS	=	Peak Security	Zonal Marginal km for each demand zone

ZMkmDiYR	=	Year Round Zonal Marginal km for each demand zone

ITTDiPS		=	Peak Security Initial Transport Tariff (£/MW) for each demand zone

ITTDiYR	=	Year Round Initial Transport Tariff (£/MW) for each demand zone

- The next step is to multiply these ITTs by the expected metered triad gross GSP group demand and generation capacity to gain an estimate of the initial revenue recovery for both Peak Security and Year Round backgrounds. The metered triad gross GSP group demand and generation capacity are based on analysis of forecasts provided by Users and are confidential.

Metered triad gross GSP group demand is net demand for all GSP groups less embedded exports for all GSP groups.

Where

ITRRG	=	Initial Transport Revenue Recovery for generation

GGi	=	Total forecast Generation for each generation zone (based on analysis of confidential User forecasts)

ITRRD	=	Initial Transport Revenue Recovery for gross GSP group demand

DDi	=	Total forecast Metered Triad gross GSP group Demand for each demand zone (based on analysis of confidential User forecasts)

In addition, the initial tariffs for generation are also multiplied by the Peak Security flag when calculating the initial revenue recovery component for the Peak Security background. When calculating the initial revenue recovery for the Shared component of the Year Round background, the initial tariffs are multiplied by the Annual Load Factor (see below). When calculating the initial revenue recovery for the Not Shared component of the Year Round background, the initial tariffs are multiplied by the Year Round Not Shared Flag.

Peak Security (PS) Flag

- The revenue from a specific generator due to the Peak Security locational       tariff needs to be multiplied by the appropriate Peak Security (PS) flag. The PS flags indicate the extent to which a generation plant type contributes to the need for transmission network investment at peak demand conditions. The PS flag is derived from the contribution of differing generation sources to the demand security criterion as described in the Security Standard. In the event of a significant change to the demand security assumptions in the Security Standard, The Company will review the use of the PS flag.

| Generation Plant Type | PS flag |
| --- | --- |
| Intermittent | 0 |
| Other | 1 |

Year Round Not Shared (YRNS) Flag

- The revenue from a specific generator due to the Year Round Not Shared locational tariff needs to be multiplied by the appropriate Year Round Not Shared (YRNS) flag. The YRNS flag indicates the extent to which a generation plant type contributes to the need for transmission network investment at year round demand conditions in areas of the System where the proportion of Low Carbon generation exceeds Carbon generation as defined in 14.15.49.

| Generation Plant Type | YRNS flag |
| --- | --- |
| Non Conventional Carbon | 1 |
| Conventional Carbon | ALF |

Annual Load Factor (ALF)

- The ALF for each individual Power Station is calculated using the relevant TEC (MW) and corresponding output data. Where output data is not available for a Power Station, including for new Power Stations and emerging Power Station technologies, generic data for the appropriate generation plant type will be used.

- For a given Financial Year “t” the Power Station ALF will be based on information from the previous five Financial Years, calculated for each Financial Year as set out below.

Where:

GMWhp is the maximum of FPN or actual metered output in a Settlement Period related to the power station TEC (MW); and

TECp is the TEC (MW) applicable to that Power Station for that Settlement Period including any STTEC and LDTEC, accounting for any trading of TEC.

- The appropriate output (FPN or actual metered) figure is derived from BM Unit data available to The Company and relates to the total TEC of the Power Station.

- Once all five Financial Year ALFs have been calculated for the individual Power Station they are compared, and the highest and lowest figures are discarded. The final ALF, to be used for transmission charging purposes, is calculated as the average of the remaining three ALFs.

- In the event that only four Financial Years of complete output (FPN or actual metered) data are available for an individual Power Station then the higher three Financial Years ALF would be used in the calculation of the final ALF. In the event that only three Financial Years of complete output (FPN or actual metered) data are available then these three Financial Years would be used.

- Due to the aggregation of output (FPN or actual metered) data for dispersed generation (e.g. cascade hydro schemes), where a single generator BMU consists of geographically separated power stations, the ALF would be calculated based on the total output of the BMU and the overall TEC of those Power Stations.

- In the event that there are not three full Financial Years of an individual power station’s output available, missing output (FPN or actual metered) data would be replaced by generic data for that generation plant type to ensure three Financial Years of information are available for the Power Station. The derivation of the generic data is described in paragraphs 14.15.111-14.15.114.

- Users will receive draft ALFs before 25th December of the Financial Year (t-1) for the Financial Year (t) and will have a period of 15 Business Days

- from date of publishing to notify The Company of any errors. Failure to agree changes relating to errors will be treated as a charging dispute under the CUSC.

- The ALFs used in the setting of final tariffs will be published in the annual Statement of Use of System Charges. Changes to ALFs after this publication will not result in changes to published tariffs (e.g. following dispute resolution).

Derivation of Generic ALFs

- The generic ALF is derived from the average annual output of the ten most recently commissioned GB generation of a particular generation plant type that have at least five Financial Years’ data, using an identical methodology to that used for the Power Station specific calculation described above. Where less than ten GB Generators of a particular generation plant type exist, then data from all existing Generators of that particular generation plant type will be used. Example generation plant type categories are listed below;

Fuel Type

Biomass

Coal

Gas

Hydro

Nuclear (by reactor type)

Oil & OCGTs

Pumped Storage

Onshore Wind

Offshore Wind

CHP

- The Company will keep these categories under review and update as necessary. Where within a category there is a significant locational difference consideration will be given to zonal generic factors. The factors used will be published in the Statement of Use of System Charges and will be reviewed annually.

- If a User can demonstrate that the generation plant type of a Power Station has changed, consideration will be given to the use of relevant generic ALF information in the calculation of their charges until sufficient specific data is available.

- For new and emerging generation plant types, where insufficient data is available to allow a generic ALF to be developed, The Company will use the  best information available e.g. from manufactuers and data from use of similar technologies outside GB. The factor will be agreed with the relevant Generator. In the event of a disagreement the standard provisions for dispute in the CUSC will apply.

### TNUoS Embedded Export Tariff

- Embedded exports are exports measured on a half-hourly basis by Metering Systems, in accordance with the BSC, that are not subject to generation TNUoS.

- The embedded export tariff will be applied to the metered Triad volumes of Embedded Exports for each demand zone as follows:

EETDi=ITTDiPS+ITTDiYR+EX

Where

ITTDiPS =	Peak Security Initial Transport Tariff for the demand zone;

ITTDiYR =	Year Round Initial Transport Tariff for the demand zone, and

EX:

First Financial Year following the implementation date of CMP 264/265:

=23XP-AGIC+AGIC

Second Financial Year following the implementation date of CMP 264/265:

=13XP-AGIC+AGIC

Third Financial Year following the implementation date of CMP 264/265 and every subsequent Financial Year:

= AGIC

Where

XP	=	Value of demand residual in Financial Year prior to implementation

AGIC =	The Avoided GSP Infrastructure Credit (AGIC) which represents the unit 
cost of infrastructure reinforcement at GSPs which is avoided as a consequence of embedded generation connected to the distribution networks served by those GSPs. It is calculated from the average annuitised cost of that infrastructure reinforcement divided by the average capacity delivered by a supergrid transformer.

The Avoided GSP Infrastructure Credit is calculated at the beginning of each price control period and in the first applicable Financial Year following the implementation date of CMP264/265 using data submitted by onshore TSOs as part of the price control process. The data used is from the most recent [20] schemes submitted under the price  control process and indexed each year by the TOPI formula set out in 14.3.6 until the end  of the price control. For the avoidance of doubt, this approach does not include the cost of the supergrid transformers or any other connection assets as they are paid for by the relevant DNOs thorough their connection charges.

The Value of EETDi will be floored at zero, so that EETDi is always zero or positive.

The embedded export tariff applicable where a User directly connected to the National Electricity Transmission System is located at a Grid Supply Point that connects to more than one demand zone, will be derived as follows:

ITTDcPS  =	average (ITTDaPS ,ITTDbPS ITTDdPS …)

ITTDcYR  =	average (ITTDaYR ,ITTDbYR ITTDdYR …)

ITTDcPS =	average of Peak Security Initial Transport Tariff attributable to each demand zone (ITTDaPS ,ITTDbPS ITTDdPS …); and

ITTDcYR =	average of Year Round Initial Transport Tariff attributable to each demand zone (ITTDaYR ,ITTDbYR ITTDdYR …).

Where C is demand directly connected to the National Electricity Transmission System at a Grid Supply Point that connects to more than one demand zone, and a, b, d… are the relevant GSP Groups.

### Initial Revenue Recovery

### For the Peak Security background the initial tariff for generation is multiplied by the total forecast generation capacity and the PS flag to give the initial revenue recovery:

Gi=1nITTGiPS×GGi×FPS=ITRRGPS

Where:

ITRRGPS	=	Peak Security Initial Transport Revenue Recovery for generation

GGi	=	Total forecast Generation for each generation zone (based on analysis of confidential User forecasts)

FPS	=	Peak Security flag appropriate to that generator type

n	=	number of generation zones

The initial revenue recovery for gross GSP group demand for the Peak Security background is calculated by multiplying the initial tariff by the total forecast metered triad gross GSP group demand:

Di=114ITTDiPS×DDi=ITRRDPS

Where:

ITRRDPS	=	Peak Security Initial Transport Revenue Recovery for gross GSP group demand

DDi	=	Total forecast Metered Triad gross GSP group Demand for each demand zone (based on analysis of confidential User forecasts)

### For the Year Round background, the initial tariff for generation is multiplied by the total forecast generation capacity whilst calculating Initial Recovery for the Not-Shared component from Non Conventional Carbon. For Conventional Carbon the initial tariff for the Not Shared component is multiplied by both, the total forecast generation capacity and the ALF to give the initial revenue recovery. The initial tariff for the Shared component is multiplied by both, the total forecast generation capacity and the ALF to give the initial revenue recovery:

Gi=1nITTGiYRNSNCC×GGi=ITRRGYRNSNCC

Gi=1nITTGiYRNSCC×GGi×ALF=ITRRGYRNSCC

ITRRGYRNS=ITRRGYRNSNCC+ITRRGYRNSCC

Where:

ITRRGYRNSNCC	=	Year Round Not-Shared Initial Transport Revenue Recovery for Non Conventional Carbon generation

ITRRGYRNSCC	=	Year Round Not-Shared Initial Transport Revenue Recovery for Conventional Carbon generation

ITRRGYRNS	=	Year Round Not-Shared Initial Transport Revenue Recovery for generation

ITRRGYRS	=	Year Round Shared Initial Transport Revenue Recovery for  generation

ALF	=	Annual Load Factor appropriate to that generator.

- Similar to the Peak Security background, the initial revenue recovery for gross  GSP group demand for the Year Round background is calculated by multiplying the initial tariff by the total forecast metered triad gross GSP group demand:

Where:

ITRRDYR	=	Year Round Initial Transport Revenue Recovery for gross GSP

group demand

- The initial revenue recovery for Embedded Exports is the Embedded Export Tariff multiplied by the total forecast volume of Embedded Export at triad:

ITRREE=Di=114EETDi×EEVDi

Where

ITRREE		=	Initial Revenue impact for Embedded Exports

EEVDi		=	Forecast Embedded Export metered volume at Triad (MW)

For the avoidance of doubt, the initial revenue recovery for embedded exports

can be positive or negative.

## Deriving the Final Local Tariff (£/kW)

## Local Circuit Tariff

- Generation with a local circuit tariff is calculated by multiplying the Year Round nodal marginal km along the local circuit by the expansion constant and the relevant local security factor (whether onshore or offshore) and summing across local circuits to give the local circuit tariff:

Where

k		=	Local circuit k for generator

NLMkmGjL	=	Year Round Nodal marginal km along local circuit k using local circuit expansion factor.

EC		=	Expansion Constant

LocalSFk	=	Local Security Factor for circuit k

CLTGi		=	Circuit Local Tariff (£/kW)

### Onshore Local Substation Tariff

- All chargeable generation is subject to the local substation tariff component. This is determined by assessing the generation substation type for the substation at the connection charging boundary against three cost determining factors:

(a)	HV connection voltage – the voltage at the boundary between the User’s connection assets and the transmission system;

(b)	Sum of TEC at the generation substation – the combined TEC of all generation at the connecting substation; and

(c)	The level of redundancy at the generation substation – single busbar / single switch mesh connections are examples of no redundancy connections, whereas examples of connections with redundancy include double busbar and mesh sub station designs.

- Using the above factors, the corresponding £/kW tariffs that are currently applicable, are detailed in the Statement of Use of System Charges.

- The process for calculating Local Substation Tariffs will be carried out for the first year of the price control and will subsequently be indexed by TOPI for each subsequent year of the price control period.

- The effective Local Tariff (£/kW) is calculated as the sum of the circuit and substation onshore and/or offshore components:

Where

ELTGi 		=	Effective Local Tariff (£/kW)

SLTGi		=	Substation Local Tariff (£/kW)

- Where tariffs do not change mid way through a Financial Year, final local tariffs will be the same as the effective tariffs:

ELTGi 		=	LTGi

Where

LTGi 		=	Final Local Tariff (£/kW)

- Where tariffs are changed part way through the year, the final tariffs will be calculated by scaling the effective tariffs to reflect that the tariffs are only applicable for part of the year and parties may have already incurred TNUoS liability.

and

Where:

b = number of months the revised tariff is applicable for

FLL = Forecast local liability incurred over the period that the original tariff is applicable for

- For the purposes of charge setting, the total local charge revenue is calculated by:

Where

LCRRG		=	Local Charge Revenue Recovery

Gj	=	Forecast chargeable Generation or Transmission Entry Capacity in kW (as applicable) for each generator (based on analysis of confidential information received from Users)

### Offshore substation local tariff

- All offshore chargeable generation is subject to an offshore substation tariff.  The offshore substation tariff shall be the sum of transformer, switchgear and platform components.

- Each tariff component, expressed in £/kW, shall be the ratio of the Offshore Transmission Owner revenue (£) and rating associated with the transformers, switchgear or platform (kW) at each offshore substation.  The Offshore Transmission Owner revenue of each tariff component shall include that associated with asset spares.  In the case of the platform component, the relevant rating shall be the lower of the transformer or switchgear ratings.  As with the offshore circuit expansion factors, the Offshore Transmission Owner revenue associated with each tariff component shall be averaged over the remaining years of the NETSO price control.

- Offshore Transmission Owner revenue associated with interest during construction and project development overheads will be attributed to the relevant asset category with which it is associated.  If these or any other costs included in the Offshore Transmission Owner revenue are not readily attributable to a given asset category, they will be pro-rated across the various asset categories based on their relative cost.

- A discount shall be provided to the offshore substation tariff to reflect the average cost of civil engineering for onshore substations. The currently applicable discount is detailed in the Statement of Use of System Charges. This will be inflated by TOPI each year and reviewed every price control period.

- Offshore substation tariffs shall be reviewed at the start of every onshore price control period. For each subsequent year within the price control period, these shall be inflated in the same manner as the associated Offshore Transmission Owner Revenue.

- The revenue from the offshore substation local tariff is calculated by:

Where:

SLTk	= 	the offshore substation tariff for substation k

Genk	=	the generation connected to offshore substation k

### The Residual Tariff

### 14.15.135	The total revenue to be recovered through TNUoS charges is determined each year with reference to the Transmission Licensees’ Price Control formulas less the costs expected to be recovered through Pre-Vesting connection charges.  Hence in any given year t, a target revenue figure for TNUoS charges (TRRt) is set  as follows:

### TRRt = Rt - PVCt

Where

TRRt 	=	TNUoS Revenue Recovery target for year t.

Rt	=	Forecast Revenue allowed under The Company’s Price Control for year t (this term includes a number of adjustments, including for over/under recovery from the previous year).  For further information, refer to condition F3 of the ESO Licence.

PVCt	=	Forecast Revenue from Pre-Vesting connection charges for year t

    - In normal circumstances, the revenue forecast to be recovered from the initial transport tariffs will not equate to the total revenue target. This is due to a number of factors.  For example, the transport model assumes, for simplicity, smooth incremental transmission investments can be made. In reality, transmission investment can only be made in discrete 'lumps'. The transmission system has been planned and developed over a long period of time.  Forecasts and assessments used for planning purposes will not have been borne out precisely by events and therefore some distinction between an optimal system for one year and the actual system can be expected.

    - As a result of the factors above, in order to ensure adequate recovery of total Transmission Owner revenue, a set of non-locational Transmission Demand Residual Tariffs are calculated, which include infrastructure substation asset costs.  These tariffs are billed alongside the initial transport tariffs for demand only so that the total revenue recovery is achieved. The total amount of revenue to be recovered through Transmission Demand Residual Tariffs is defined as the Transmission Demand Residual.

TDR= TRR- ITRRDPS-ITRRDYR- ITRREE- ITRRGPS- ITRRGYRNS-ITRRGYRS-LCRRGG-AdjRevenue

Where

TDR = Transmission Demand Residual

AdjRevenue	= Adjustment Revenue as per paragraph 14.14.5

## Creation of Charging Bands for use in Transmission Demand Residual Tariff Setting

    - To produce the Transmission Demand Residual Tariffs a set of Charging Bands are to be created for each of the Residual Charging Groups using the following methodology.

- For domestic Final Demand Sites whether connected to the Distribution system or Transmission system there will be one Charging Band and;

- For non-domestic Final Demand Sites connected to the Distribution system there will be four Charging Bands for each of the Residual Charging Groups according to the methodology introduced to Schedule 32  of the DCUSA via DCUSA modification DCP358 and entitled ‘RESIDUAL CHARGING BANDS’ with boundaries set at the 40th, 70th and 85th percentiles and;

- For Final Demand Sites directly connected to the Transmission system there will be four Charging Bands using the latest 24 months of gross Consumption data (from the Final Reconciliation Settlement Run or Reconciliation Settlement Run)  with boundaries set at the 40th, 70th and 93rd percentiles and;

- For Unmetered Supplies there will be one Charging Band.

| Domestic Final Demand Sites |  |
| --- | --- |
| LV No Mic | Band 1 (≤40th percentile) |
|  | Band 2 (>40th percentile – 70th percentile) |
|  | Band 3 (>70th percentile – 85th percentile) |
|  | Band 4 (>85th percentile) |
| LV MIC | Band 1 (≤40th percentile) |
|  | Band 2 (>40th percentile – 70th percentile) |
|  | Band 3 (>70th percentile – 85th percentile) |
|  | Band 4 (>85th percentile) |
| HV | Band 1 (≤40th percentile) |
|  | Band 2 (>40th percentile – 70th percentile) |
|  | Band 3 (>70th percentile – 85th percentile) |
|  | Band 4 (>85th percentile) |
| EHV | Band 1 (≤40th percentile) |
|  | Band 2 (>40th percentile – 70th percentile) |
|  | Band 3 (>70th percentile – 85th percentile) |
|  | Band 4 (>85th percentile) |
| Directly Connected Users Final Demand Sites | Band 1 (≤40th percentile) |
|  | Band 2 (>40th percentile – 70th percentile) |
|  | Band 3 (>70th percentile – 93rd percentile) |
|  | Band 4 (>93rd percentile) |
| Unmetered Supplies |  |

14.15.139 These Charging Bands will be reviewed periodically and be implemented effective from the beginning of each Onshore Transmission Owner price control period.

Transmission Demand Residual Tariff Setting

14.15.140

The Transmission Demand Residual Tariffs are derived from the Transmission Demand Residual value calculated in 14.15.137 and the total aggregate annual consumption of all Final Demand Sites and Unmetered Supplies.

14.15.141

To determine the proportion of the Transmission Demand Residual to be recovered from each Charging Band:

- Where there are Final Demand Sites in a Charging Band the total annual consumption from Final Demand Sites in the Charging Band in question is divided by the total annual consumption from all Final Demand Sites and Unmetered Supplies creating a percentage value.

- Where there are Unmetered Supplies in a Charging Band the total annual consumption from Unmetered Supplies in the Charging Band in question is divided by the total annual consumption from all Final Demand Sites and Unmetered Supplies creating a percentage value.

This percentage is multiplied by the Transmission Demand Residual to give the total value to be recovered from the Charging Band.

14.15.142

## To set the Transmission Demand Residual Tariff for each Charging Band:

- For each Charging Band containing Final Demand Sites the total value to be recovered from the Charging Band as per 14.15.141 is divided by the number of Final Demand Sites in the Charging Band to create a £/site annual charge. This charge is further divided by the number of days in the charging year for which this tariff applies to produce the Transmission Demand Residual Tariff for the Charging Band (£/site/day).

- For each Charging Band containing Unmetered Supplies the total value to be recovered from the Charging Band as per 14.15.141 is divided by the total annual consumption from Unmetered Supplies in the Charging Band in question to create a p/kWh charge. This tariff is also defined as the UMS Tariff.

## Final £/kW Tariff

- The effective Transmission Network Use of System tariff (TNUoS) for generation can now be calculated as the sum of the initial transport wider tariffs for Peak Security and Year Round backgrounds and Adjustment Tariff and local tariff (for generation):

ETGi= ITTGiPS+ ITTGiYRNS+ IFFGiYRS+ AdjTariffi1000+ LTGi

and

The effective Transmission Network Use of System tariff (TNUoS) for the HH Demand Locational can now be calculated as the sum of the initial transport wider tariffs for Peak Security and Year Round backgrounds for half-hourly metered demand:

ETDi= ITTDiPS+ITTDiYR1000

Where

ET	Gi=		Effective Generation TNUoS Tariff expressed in £/kW (ETGi would only be applicable to a Power Station with a PS flag of 1 and ALF of 1; in all other circumstances ITTGiPS, ITTGiYRNS and ITTGiYRS will be applied using Power Station specific data)

AdjTariffi = 		AdjTariff (from 14.14.5) applicable in time period ‘i’.

ET	Di=		Effective HH Demand Locational TNUoS Tariff expressed in £/kW

The effective Transmission Network Use of System tariff (TNUoS) for embedded exports can now be calculated by expressing the embedded export tariff in £/kW values:

ETEEi=EETDi1000

Where

ET	EEi=		Effective Embedded Export TNUoS Tariff expressed in £/kW

For the purposes of the annual Statement of Use of System Charges ETGi will be published as ITTGiPS; ITTGiYRNS, ITTGiYRS, LTGi  and AdjTariffi (if required)

- Where tariffs do not change mid way through a Financial Year, final demand and generation tariffs will be the same as the effective tariffs.

FTEEAi=ETEEi

- Where tariffs are changed part way through the year, the final tariffs will be calculated by scaling the effective tariffs to reflect that the tariffs are only applicable for part of the year and parties may have already incurred TNUoS liability.

and 	FTEEi=12×(ETEEi×Di=114EETDi-FLDi)b×Di=114EETDi

Where:

b = number of months the revised tariff is applicable for

FL = Forecast liability incurred over the period that the original tariff is applicable for

Note: The ETGi element used in the formula above will be based on an individual Power Stations PS flag and ALF for Power Station GGi, aggregated to ensure overall correct revenue recovery.

- If the final HH Demand Locational TNUoS Tariff results in a negative number then this is collared to £0/kW with the resultant revenue to be refunded to tFinal Demand smeared over the remaining demand zones via the Transmission Demand Residual:

If 	, 	then i = 1 to z

Therefore,

Therefore the revised Final Tariff for the gross demand zones with positive Final tariffs is given by:

For i= 1 to z:

For i=z+1 to 14:

Where

NRRTD	=	Non Recovered Revenue Tariff (£/kW)

RFTDi	=	Revised Final Tariff (£/kW)

- The tariffs applicable for any particular year are detailed in the Statement of Use of System Charges. Archived tariff information may also be obtained from the Website.

- The zonal maps referenced in the Statement of Use of System Charges contain detailed information for the Financial Year in question of which Grid Supply Points fall into which TNUoS zones.

- New Grid Supply Points will be classified into zones on the following basis:

- For demand zones, according to the GSP Group to which the Grid Supply Point is allocated for energy market settlement purposes.

- For a User directly connected to the National Electricity Transmission System, the following rules will apply to determine its demand zone:

- If the Connection Site falls within a GSP Group’s geographic area, the new GSP that connects directly at the site will be assigned to the demand zone corresponding to the GSP Group, unless;

- A User is directly connected to the National Electricity Transmission System at a Grid Supply Point that connects to more than one demand zone, in which case, for the purpose of DCLF modelling, the User’s demand will be spread evenly across the relevant GSP Groups to calculate the relevant zonal demand tariffs.

- For generation zones, with reference to the geographic proximity to existing zones and, where close to a boundary between existing zones, with reference to the marginal costs arising from transport model studies.  The GSP will then be allocated to the zone, which contains the most similar marginal costs.

- The Company has available, upon request, the DCLF ICRP transport model, tariff model template and data necessary to run the model, consisting of nodal values of generation and demand connection points to the NETS. The model and data will enable the basic nodal charges to be determined and will also allow sensitivity analysis concerning alternative developments of generation and demand to be undertaken.  The model is available from the Charging Team and whilst it is free of charge, it is provided under licence to restrict its distribution and commercial use.

- The Company will be pleased to run specific sensitivity studies for Users under a separate study contract in line with the fees set out in the Statement of Use of System Charges. Please contact the Charging Team.

- The factors which will affect the level of TNUoS charges from year to year include but are not limited to;

- the forecast level of peak demand on the system

- the Price Control formula (including the effect of any under/over recovery from the previous year),

- the expansion constant,

- the locational security factor,

- the PS flag

- the Year Round Not Shared (YRNS) Flag

- the ALF of a generator

- changes in the transmission network

- HVDC circuit impedance calculation

- changes in the pattern of generation capacity and demand.

- changes in the pattern of embedded exports

- the £/ € exchange rate and expected Generator Output

- Number of Final Demand Sites per Charging Band

- Volume (in kWh) apportioned to each Charging Band

- In accordance with Standard Licence Condition C13, generation directly connected to the NETS 132kV transmission network which would normally be subject to generation TNUoS charges but would not, on the basis of generating capacity, be liable for charges if it were connected to a licensed distribution network qualifies for a reduction in transmission charges by a designated sum, determined by the Authority. Any shortfall in recovery will result in a unit amount increase in gross demand charges to compensate for the deficit. Further information is provided in the Statement of the Use of System Charges.

## Stability & Predictability of TNUoS tariffs

- A number of provisions are included within the methodology to promote the stability and predictability of TNUoS tariffs.  These are described in 14.29.

- Demand tariffs for Demand Users directly connected to the NETS at GSPs that connect to more than one demand zone, will be derived from averages of the Peak Security and Year Round tariffs respectively attributable to each demand zone, as set out below:

Transmission-connected demand user demand C, 200MW

DNO B

400kV node NODE40

DNO A

Demand A, 30MW

Demand B, 70MW

As demand C connects at a Connection Site consisting of connections to more than one demand zone, for the purpose of DCLF Transport modelling and calculation of demand zonal tariffs, demand C is spread evenly across the multiple GSP Groups (A and B). Therefore demand C at NODE40 is split into two parts: 100MW at GSP Group A, and 100MW at GSP Group B. The zonal demand at A and B is calculated as below (copying 14.15.41):

The zonal Peak Security marginal km for demand zone A and B is calculated as follows.

Where:

Di 		=	Demand zone

Dem		=	Positive Nodal Net Demand from transport model, including 100MW at NODE40 by demand C for demand zone A, and 100MW at NODE40 by demand C for demand zone B.

If Nodal Demand from a node is less than 0 (Exporting) the nodal demand will be set to zero and therefore not contribute to the Zonal marginal km

Similarly, the zonal Year Round marginal km for demand zones is calculated as follows:

Once the zonal Peak Security and Year Round tariffs are calculated for demand zones A and B (according to 14.15.97), a set of new demand zonal tariffs will be created for demand C as below –

Zonal Peak Security tariff for demand C = average (Zone A Peak Security tariff, Zone B Peak Security tariff)

Zonal Year Round tariff for demand C = average (Zone A Year Round tariff, Zone B Year Round tariff)

For demand C, ZMkmDCPS=ZMkmDAPS+ZMkmDBPS , and ZMkmDCYR=ZMkmDAYR+ZMkmDBYR

## Allocating Final Demand Sites to Charging Bands

- Users who own or operate a Distribution System shall maintain a methodology in DCUSA entitled “ALLOCATION OF CUSTOMERS TO CHARGING BANDS BY DNO/IDNO PARTIES” (introduced by DCUSA modification DCP360) to allocate Final Demand Sites and Unmetered Supplies to Charging Bands for their respective Distribution System.

- In Charging Bands that are determined in accordance with 14.15.138, Final Demand Sites and Unmetered Supplies will be allocated to Charging Bands as follows;

- For Embedded Final Demand Sites and Unmetered Supplies, Users who own or operate a Distribution System shall allocate Embedded Final Demand Sites and Unmetered Supplies to Charging Bands for their respective network as per the methodology described in 14.15.161

- For Final Demand Sites connected to the NETS, the following hierarchy will apply, starting at (i) and progressing to (iv) to determine the correct Charging Band as created in 14.15.139;

  - Where available, the mean average of the latest 24 months Consumption data for the specific Final Demand Site shall be used.

Where this is not available in terms of (ii), (iii) and (iv);

  - The mean average of as much data Consumption data as is available for the specific Final Demand Site, or;

  - The Company making use of any valid information as is available or made available to best estimate the expected Consumption of the Final Demand Site, or;

  - Should no data or information be available for the specific Final Demand Site, a 12 month mean average of all Consumption from all NETS connected Final Demand Sites shall be used.

- For all Final Demand Sites allocated under 14.15.157 2. (iii) or (iv), in the September following the completion of a full Financial Year for which the Final Demand Site has recorded actual metered data a review of the Charging Band allocation of the specific Final Demand Site will take place by The Company.

- This review, (the “September New Site review”) will use the material change in circumstances as per paragraph 14.15.163 to determine whether the Final Demand Site will be reallocated to a different Charging Band.

- The Company shall notify any Final Demand Site and it’s Supplier(s) should they be included in a September New Site review. This notification shall be no issued later than the 15th calendar day in August prior to the September New Site review.

- Following the conclusion of the September New Site review, The Company shall notify both the Final Demand Site and it’s Supplier(s) of the outcome within 5 Business Days of the completion of the September New Site review. The Transmission Demand Residual Tariff as per the new Charging Band will apply from 1st October immediately following the September New Site review should any Final Demand Sites be reallocated to a different Charging Band.

- The Company shall complete all September New Site reviews by the 15th calendar day of September.

- Final Demand Sites will be subject to the September New Site review no more than once during the duration of their Bilateral Connection Agreement.

- For the purpose of determining the Transmission Demand Residual Tariff liability for a given Final Demand Site, the Final Demand Site will be allocated to a Charging Band.

- Allocation to a Charging Band for NETS connected Final Demand Sites will be in accordance with 14.15.157 2. The Final Demand Site will be allocated into a Charging Band where the Final Demand Site’s gross Consumption value, is less than or equal to the maximum threshold for that Charging Band.

- Allocation to a Charging Band for Embedded Final Demand Sites will be in accordance with the methodology described in 14.15.156 and 14.15.157 1.

- For Mixed Demand Sites connected to the NETS, the provisions of 14.15.158 shall be applied to the Mixed Demand Site’s gross Consumption except where a valid Declaration is provided by the User. This Declaration shall clearly identify the Metering Systems (or Operational Metering Equipment where a Metering System is not viable) used to isolate and identify gross Final Demand Consumption from any other Consumption at the Mixed Demand Site. A Mixed Demand Site’s gross Final Demand Consumption shall either be measured using meters installed at each Final Demand asset, or alternatively, be calculated as the difference between the Mixed Demand Site’s gross Consumption as metered at the site boundary, and consumption metered at each asset which does not consume Final Demand. Should a valid Declaration be provided, any Consumption not associated with Final Demand will be removed before the provisions of 14.15.158 are applied. Where it is unclear whether the Consumption is Final Demand or not, it will be treated as Final Demand.

- For Embedded Mixed Demand Sites, the methodology described in 14.15.156 shall be used.

- For the purpose of determining the Transmission Demand Residual Tariff liability for a given Unmetered Supply, the Unmetered Supply will be allocated to the appropriate Residual Charging Group in accordance with 14.15.138.I

- Final Demand Sites and Unmetered Supplies will be assigned to a Charging Band for the duration of the Onshore Transmission Owner price control, unless;

- The Final Demand Site or and Unmetered Supply is reassigned by an intervention as described in 14.15.161 - 14.15.165, or

- the Authority directs that the Final Demand Site or Unmetered Supply should be reassigned to a different Charging Band.

Once allocated and subject to the intervention process, Final Demand Sites and Unmetered Supplies will be re-allocated to Charging Bands prior to the start of each Onshore Transmission Owner price control in accordance with 14.15.138.

Charging Banding Interventions

- Users who own or operate a Distribution System shall maintain a methodology in DCUSA entitled “EXCEPTIONAL CIRCUMSTANCES RESULTING IN RE-ALLOCATION TO A DIFFERENT BAND WITHIN A PRICE CONTROL PERIOD” (introduced by DCUSA modification DCP360) to manage interventions and disputes from Embedded Final Demand Sites and Unmetered Supplies connected to their respective Distribution System.

- Where a Final Demand Site or Unmetered Supply (or their nominated Supplier(s) or agent) or a User or The Company wishes to challenge the allocation of such a Final Demand Site to a Charging Band, it shall;

- For Embedded Final Demand Sites and Unmetered Supplies, Users who own or operate a Distribution System shall follow the methodology described in 14.15.161 for their respective network

- For Final Demand Sites directly connected to the NETS, follow the process as defined in CUSC Section 7.3 to demonstrate a material change in circumstances.

- For the purposes of 14.15.162 2. a material change in circumstances for NETS connected Final Demand Sites must be demonstrated by;

- the voltage of connection of the Final Demand Site changing with an accompanying signed Construction Agreement; or

- should Consumption data be used as the basis of Directly Connected Final Demand Site Charging Bands, providing 12 months of actual metered gross Consumption data which shows gross Consumption is lower than 50% or greater than 50% of the Consumption data applied to the relevant subheading of 14.15.157. For reductions in Consumption, this shall be accompanied with a signed letter from The Company Director (or equivalent) confirming the rationale for the exceptional and significant changes to consumption (including historical Consumption); or

- A notice to Disconnect is provided in accordance with CUSC Section 5.7.

- Any changes to Transmission Network Use of System Demand Charges as a result of a intervention (as described in 14.15.161) shall be collected or refunded (as appropriate) through the reconciliation process described in 14.25.

- The requirements of 14.15.161 - 14.15.163 shall not preclude the Final Demand Site’s or Unmetered Supply’s right to refer the matter directly to the Authority for determination.

Declarations

- A Declaration is a statement to be submitted by the Registrant of the relevant BM Unit(s) or Single Site, which:

- is signed by one of the Storage Facility Operator’s registered Directors that confirms that a Electricity Storage Facility fulfils the criteria set out in the definitions of SVA Storage Facility and CVA Storage Facility as applicable; and either

  - for SVA Storage Facility only, is submitted in accordance with the BSC and contains other details that are required in accordance with BSC Section S; or

  - for CVA Storage Facility only, identifies the specific BM Units which only perform activities necessary for Electricity Storage and is submitted to The Company.

- is signed by one of the Electricity Generation Facility’s registered Directors that confirms that the Electricity Generation Facility only perform activities necessary for Electricity Generation and is submitted to The Company.

- is signed by one of the Eligible Services Facility’s registered Directors that confirms the Eligible Services Facility can only perform activities necessary for Eligible Services and does not consume any Active Power other than for the provision of Eligible Services and is submitted to The Company.

- is signed by one of the Mixed Demand Site’s registered Directors that confirms the Mixed Demand Site contains both Final Demand and a Generator or Electricity Storage Plant and is submitted to The Company.

The validity of a Declaration for an SVA Storage Facility is determined in accordance with BSC Section S, and of a Declaration for a CVA Storage Facility, Non-Final Demand Site and Eligible Services Facility is determined by The Company.

- A Declaration received by The Company will either be accepted or rejected within three Business Days and shall take effect on the effective date and time as notified to the Registrant. Any disagreement between The Company and the Registrant on the validity of a Declaration will be treated as a Charging Dispute.

- Any Declarations that are accepted by The Company shall be valid in perpetuity. Should circumstance at the relevant BM Unit(s) or Single Site change in such a way that the Declaration is no longer accurate, it is the responsibility of the User to resubmit a revised Declaration as soon as possible to reflect the change in circumstance. The Company will validate any revised Declarations as per 14.15.167.

- The Company shall, at its sole discretion and acting reasonably, have the right to audit and revalidate a previously approved Declaration. The User agrees to support and cooperate with any audit and/or revalidation as required (including access to site if required). An inaccurate Declaration shall immediately be deemed to be invalid and the provisions of this Section 14 applied to the relevant BM Unit(s) or Single Site to:

- reflect no Declaration being in effect for the next calendar day; and

  - retroactively apply Transmission Network Use of System Charges from the date of invalidity to the later of:

  - a previously valid Declaration; or

  - the Final Reconciliation Statement as per paragraph 3.13.7.

- Anticipatory Investment

- Anticipatory Investment (AI) is investment in transmission assets which goes beyond the needs of the immediate offshore development or developments. Where Anticipatory Investment has been identified by the Authority, the generators’ Offshore local tariff will be calculated as per 14.15.121, and 14.15.129 – 14.15.134, but will utilise their individual generation connected and their proportion (AI or non-AI) of the OFTO revenue, Network Export Capacity, and asset ratings rather than those of the total project.

- Any generator(s) connected to the Offshore Transmission System at the point of asset transfer to the OFTO, will become liable for Offshore local tariffs for the non-AI portion of the OFTO revenue. Any subsequent generator(s) will become liable for Offshore local tariffs for the AI portion of the OFTO revenue at the point of connection.

- During the period between the Offshore Transmission System being transferred to the OFTO and the subsequent generator(s) connecting, the AI portion of the OFTO revenue cannot be recovered from the subsequent generator(s). The difference between what is payable to the OFTO but cannot be recovered from the subsequent generator(s) is referred to as the ‘AI Cost Gap.’

- Prior to the subsequent generator(s) connecting, the AI Cost Gap will be recovered from demand customers via the Transmission Demand Residual. The AI Cost Gap will be repaid to demand customers by the subsequent generator(s), once connected to the transmission system, either:

- Through the AI Cost Gap Tariff. This tariff will be applied over a period of time equal to the number of days for which the subsequent generator(s) share of the AI Cost Gap was accrued, rounded up to a whole number of years, in addition to the number of days remaining in the charging year in which the subsequent generator(s) connects (if it connects after the first day of a charging year); or

- alternatively, if decided by the generator(s), be paid via one payment in the charging year in which the subsequent generator(s) connects.

- Calculating the AI Cost Gap: The AI proportion of OFTO revenue associated to the subsequent generator(s), will be identified for each full or partial charging year prior to the subsequent generator(s) connecting. Each year’s value will be inflated in line with the average increase in May – October CPIH as described in 14.15.170.6, to ensure it is in the appropriate price base for the year the AI Cost Gap Tariff becomes applicable. The total of these values will be the AI Cost Gap.

- The inflation to be applied to the AI Cost Gap and the AI Cost Gap Tariff will be calculated as follows:

Inflation in year t=May to October average CPIHt-1May to October average CPIHt-2

- Calculating the AI Cost Gap Tariff: The AI Cost Gap Tariff for the subsequent generator (generator i) which is expressed in £/kW, shall be the ratio of the AI Cost Gap that generator i is liable to pay for the relevant year (£) and the Transmission Entry Capacity (kW) of generator i, i.e:

- AI Cost Gap Tariff for Generator i= n×AI Cost GapN×TECi

- Where:

  - TECi= Transmission Entry Capacity of generator i in kW

  - n = number of days remaining in the year over which the tariff is to be paid

  - N = total number of days over which the tariff is applicable (calculated as per 14.15.170.4)

- The calculation described in 14.15.170.6 shall be used for the initial partial year in which generator i connects (if applicable) and the first full charging year. For each subsequent year that the AI Cost Gap Tariff is applicable after the year of calculation, the full year AI Cost Gap Tariff for generator i shall be inflated in line with the average increase in May to October CPIH as per 14.15.170.6.

# 14.16 Derivation of the Transmission Network Use of System Energy Consumption Tariff and Short Term Capacity Tariffs

- For the purposes of this section, Lead Parties of Balancing Mechanism (BM) Units that are liable for Transmission Network Use of System Demand Charges are termed Suppliers.

- Following calculation of the Transmission Network Use of System £/kW HH Demand Locational Tariff (as outlined in Chapter 2: Derivation of the TNUoS Tariff) for each GSP Group a NHH Demand Locational Tariff is calculated as follows:

p/kWh Tariff = (NHHDF * £/kW Tariff - FLG) *100

NHHCG

Where:

£/kW Tariff = The £/kW Effective HH Demand Locational Tariff (£/kW), as calculated previously, for the GSP Group concerned.

NHHDF  = The Company’s forecast of Suppliers’ non-half-hourly metered Triad Demand (kW) for the GSP Group concerned. The forecast is based on historical data.

FLG  = Forecast Liability incurred for the GSP Group concerned.

NHHCG  = The Company’s forecast of GSP Group non-half-hourly metered total energy consumption (kWh) for the period 16:00 hrs to 19:00hrs inclusive (i.e. Settlement Periods 33 to 38) inclusive over the period the tariff is applicable for the GSP Group concerned.

## Short Term Transmission Entry Capacity (STTEC) Tariff

14.16.3 The Short Term Transmission Entry Capacity (STTEC) tariff for positive zones is derived from the Effective Tariff (ETGi)  annual TNUoS £/kW tariffs (14.15.143).  If multiple set of tariffs are applicable within a single Financial Year, the Final Tariff used in the STTEC calculation will be prorated in an identical manner to that used when calculating a Generators annual liability.  The periods over which the tariff would be prorated would be identical to the periods used when calculating the wider tariff (i.e. over the whole Financial Year, not just the period that the STTEC is applicable for).  STTECs will not be reconciled following a mid year charge change.  The premium associated with the flexible product is associated with the analysis that 90% of the annual charge is linked to the system peak. The system peak is likely to occur in the period of November to February inclusive (120 days, irrespective of leap years). The calculation for positive generation zones is as follows:

STTEC tariff   (£/kW/period)

Where:

FT				=	Final annual TNUoS Tariff expressed in £/kW

Gi				=	Generation zone

STTEC Period	=	A period applied for in days as defined in the CUSC

14.16.4 For the avoidance of doubt, the charge calculated under 14.16.3 above will represent each single period application for STTEC. Requests for multiple /STTEC periods will result in each STTEC period being calculated and invoiced separately.

14.16.5 The STTEC tariff for Generators with negative final tariffs is set to zero to prevent Users receiving greater than 100% of the annual TNUoS payment that would have been received for that capacity under a firm TEC.

## Limited Duration Transmission Entry Capacity (LDTEC) Tariffs

14.16.6 The Limited Duration Transmission Entry Capacity (LDTEC) tariff for positive zones is derived from the equivalent zonal STTEC tariff for up to the initial 17 weeks of LDTEC in a given Financial Year (whether consecutive or not).  For the remaining weeks of the year, the LDTEC tariff is set to collect the balance of the annual TNUoS liability over the maximum duration of LDTEC that can be granted in a single application.  If multiple set of tariffs are applicable within a single Financial Year, the Final Tariff used in the LDTEC calculation will be prorated in an identical manner to that used when calculating a Generators annual liability.  The periods over which the tariff would be prorated would be identical to the periods used when calculating the wider tariff (ie over the whole Financial Year, not just the period that the STTEC is applicable for).  LDTECs will not be reconciled following a mid year charge change:

Initial 17 weeks (high rate):

Remaining weeks (low rate):

where	FT is the final annual TNUoS tariff expressed in £/kW;

Gi is the generation TNUoS zone; and

P is the premium in % above the annual equivalent TNUoS charge as determined by The Company, which shall have the value 0.

14.16.7 The LDTEC tariff for Generators with negative final tariffs is set to zero to prevent Users receiving greater than 100% of the annual TNUoS payment that would have been received for that capacity under a firm TEC.

14.16.8 The tariffs applicable for any particular year are detailed in the Statement of Use of System Charges.  Historical tariffs are also available on the Website.

# 14.17 Demand Charges

## Parties Liable for Demand Charges

- Demand charges are subdivided into charges for demand locational, Transmission Demand Residual, energy and embedded export. The following parties shall be liable for some or all of the categories of demand charges:

- The Lead Party of a Supplier BM Unit;

- Power Stations with a Bilateral Connection Agreement;

- Parties with a Bilateral Embedded Generation Agreement

- Classification of parties for charging purposes, section 14.26, provides an illustration of how a party is classified in the context of Use of System charging and refers to the paragraphs most pertinent to each party.

## Basis of Demand Locational Charges

- Demand Locational charges are based on a de minimis £0/kW charge for Half Hourly and £0/kWh for Non Half Hourly metered demand.

- Chargeable Demand Locational Capacity is the value of Half Hourly metered Triad demand (kW).  Chargeable Energy Capacity is the energy consumption (kWh).  The definition of both these terms is set out below.

- If there is a single set of demand locational tariffs within a Financial Year, the Chargeable Demand Locational Capacity is multiplied by the relevant demand locational tariff, for the calculation of demand locational charges.

- If there is a single set of energy tariffs within a Financial Year, the Chargeable Energy Capacity is multiplied by the relevant energy consumption tariff for the calculation of energy charges.

- If multiple sets of demand locational tariffs are applicable within a single Financial Year, demand locational charges will be calculated by multiplying the Chargeable Demand Locational Capacity by the relevant tariffs pro-rated across the months that they are applicable for, as below,

Annual LiabilityDemand=Chargeable Demand Locational Capacity×a×Tariff 1+b×Tariff 212

Where:

Tariff 1	=	Original tariff,

Tariff 2	=	Revised tariff,

A	= 	Number of months over which the original tariff is applicable,

b	= 	Number of months over which the revised tariff is applicable.

- If multiple sets of energy tariffs are applicable within a single Financial Year, energy  charges will be calculated by multiplying relevant Tariffs by the Chargeable Energy Capacity over the period that that the tariffs are applicable for and summing over the year.

Annual LiabilityEnergy=Tariff 1×T1ST1EChargeable Energy Capacity+Tariff 2×T2ST2EChargeable Energy Capacity

Where:

T1 S	=	Start date for the period for which the original tariff is applicable,

T1 E 	=	End date for the period for which the original tariff is applicable,

T2 S	=	Start date for the period for which the revised tariff is applicable,

T2E	=	End date for the period for which the revised tariff is applicable.

### Basis of Embedded Export Charges

- Embedded export charges are based on a £/kW charge for Half Hourly metered embedded export.

- Chargeable Embedded Export Capacity is the value of Embedded Export at Triad (kW). The definition of this term is set out below.

- If there is a single set of embedded export tariffs within a Financial Year, the Chargeable Embedded Export Capacity is multiplied by the relevant embedded export tariff, for the calculation of embedded export charges.

- If multiple sets of embedded export tariffs are applicable within a single Financial Year, embedded export charges will be calculated by multiplying the Chargeable Embedded Export Capacity by the relevant tariffs pro rated across the months that they are applicable for, as below,

Annual LiabilityDemand=Chargeable Embedded Export Capacity×a×Tariff 1+b×Tariff 212

where:

Tariff 1 =	Original tariff,

Tariff 2 =	Revised tariff,

a	= 	Number of months over which the original tariff is applicable,

b	= 	Number of months over which the revised tariff is applicable.

### Supplier BM Unit

- A Supplier BM Unit charges will be the sum of its energy, demand locational, Transmission Demand Residual and embedded export liabilities where:

- The Chargeable Demand Locational Capacity will be the average of the Supplier BM Unit's half-hourly metered gross demand during the Triad (and the £/kW tariff), and

- The Chargeable Embedded Export Capacity will be the average of the Supplier BM Unit's half-hourly metered embedded export during the Triad (and the £/kW tariff), and

- The Chargeable Energy Capacity will be the Supplier BM Unit's non half-hourly metered energy consumption over the period 16:00 hrs to 19:00 hrs inclusive every day over the Financial Year (and the p/kWh tariff), and

- The Transmission Demand Residual charge for Final Demand Sites will be the sum of the number of sites per Charging Band as served by that Supplier BM Unit multiplied by the number of days the sites were served by that Supplier BM Unit and multiplied by the applicable Transmission Demand Residual Tariff £/site/day as determined in 14.15.142. Where a Connection Site is served by more than one Supplier BM Unit, the charges will be divided by The Company, annually, between the relevant Supplier BM Units in proportion to their annual Consumption from the previous year, and

- The Transmission Demand Residual charge for Unmetered Supplies will be the sum of the forecast monthly volume of Unmetered Supplies per Charging Band as served by that Supplier BM Unit multiplied by the applicable UMS Tariff (p/kWh) as determined in 14.15.142.

### Power Stations with a Bilateral Connection Agreement and Licensable Generation with a Bilateral Embedded Generation Agreement

- The Chargeable Demand Locational Capacity for a Power Station with a Bilateral Connection Agreement or Licensable Generation with a Bilateral Embedded Generation Agreement will be based on the average of the net import over each Triad leg of the BM Units associated with the Power Station (in Appendix C of its Bilateral Connection Agreement or Bilateral Embedded Generation Agreement, including metered additional load) during the Triad.

### Exemptible Generation and Derogated Distribution Interconnectors with a Bilateral Embedded Generation Agreement

- The demand charges for Exemptible Generation and Derogated Distribution Interconnector with a Bilateral Embedded Generation Agreement will be the sum of its gross demand and embedded export liabilities where:

The Chargeable Demand Locational Capacity for Exemptible Generation and Derogated Distribution Interconnectors with a Bilateral Embedded Generation Agreement will be based on the average of the metered gross demand of each BM Unit specified in Appendix C of the Bilateral Embedded Generation Agreement during the Triad.

- The Chargeable Embedded Export Capacity for Exemptible Generation and Derogated Distribution Interconnectors with a Bilateral Embedded Generation Agreement will be based on the average of the metered embedded export of each BM Unit specified in Appendix C of the Bilateral Embedded Generation Agreement during the Triad.

## Small Generators Tariffs

- In accordance with Standard Licence Condition C13, any under recovery from the MAR arising from the small generators discount will result in a unit amount of increase to the Transmission Demand Residual.

## The Triad

- The Triad is used as a short hand way to describe the three Settlement Periods of highest transmission system demand within a Financial Year, namely the half hour Settlement Period of system peak net demand and the two half hour Settlement Periods of next highest net demand, which are separated from the system peak net demand and from each other by at least 10 Clear Days, between November and February of the Financial Year inclusive.  Exports on directly connected Interconnectors and Interconnectors capable of exporting more than 100MW to the Total System shall be excluded when determining the system peak net demand. An illustration is shown below.

Half-hourly metered Demand Locational charges

- For Supplier BMUs and BM Units associated with Exemptible Generation and Derogated Distribution Interconnectors with a Bilateral Embedded Generation Agreement, if the average half-hourly metered gross demand volume over the Triad results in an import, the Chargeable Gross Demand Capacity will be positive resulting in the BMU being charged.

If the average half-hourly metered embedded export volume over the Triad results in an export, the Chargeable Embedded Export Capacity will be negative resulting in the BMU being paid the relevant tariff; where the tariff is positive. For the avoidance of doubt, parties with Bilateral Embedded Generation Agreements that are liable for Generation charges will not be eligible for payment of the embedded export tariff.

Monthly Charges

- Throughout the year Users will submit a Demand Forecast. A Demand Forecast will include:

- half-hourly metered gross demand to be supplied during the Triad for each BM Unit

	half-hourly metered embedded export to be exported during the Triad for each BM Unit

	non-half hourly metered energy to be supplied over the period 16:00 hrs to 19:00 hrs inclusive every day over the Financial Year for each BM Unit

- Throughout the year, Users’ monthly demand charges will be based on;

  - For HH Charges the User’s Demand Forecast  half-hourly  metered embedded export to be supplied during the Triad for each BM Unit, multiplied by the relevant zonal £/kW tariff; and where this results in a positive value the User’s Demand Forecast half hourly metered embedded export to be supplied during the Triad for each BM Unit, multiplied by the relevant zonal £/kW tariff.

  - For NHH Charges the User’s Demand Forecast non-half hourly metered energy to be supplied over the period 16:00 hrs to 19:00 hrs inclusive every day over the Financial Year for each BM Unit, multiplied by the relevant zonal p/kWh tariff.

- The Final Demand Site Count Forecast for the latest day (that The Company has data available for) multiplied by the relevant £/Site/Day Transmission Demand Residual Tariff for the relevant Charging Band.

- the Unmetered Supply Volume Forecast for the latest day (that The Company has data available for) multiplied by the UMS Tariff.

Users’ annual TNUoS demand charges are based on these forecasts with the Demand Forecast split evenly over the 12 months of the year.  Users have the opportunity to vary their Demand Forecasts on a quarterly basis over the course of the year, with the Demand Forecast requested in February relating to the next Financial Year.  Users will be notified of the timescales and process for each of the quarterly updates.  The Company will revise the monthly Transmission Network Use of System Demand Charges by calculating the annual charge based on the above forecasts, subtracting the amount paid to date, and splitting the remainder evenly over the remaining months.  For the avoidance of doubt, only positive Demand Forecasts (i.e. representing a net import from the system), positive FDSC Forecast and positive Unmetered Supply Volume Forecast will be used in the calculation of charges.

Demand Forecasts for a User will be considered positive where:

- The sum of the gross demand forecast and embedded export forecast is positive; and

- The non-half hourly metered energy forecast is positive.

- Users should submit reasonable demand forecasts of gross demand, embedded export and energy in accordance with the CUSC.  The Company shall use the following methodology to derive a forecast to be used in determining whether a User's forecast is reasonable, in accordance with the CUSC, and this will be used as a replacement forecast if the User's total forecast is deemed unreasonable. The Company will, at all times, use the latest available Settlement data.

For existing Users:

- The User’s Triad gross demand and embedded export for the preceding Financial Year will be used where User settlement data is available and where The Company calculates its forecast before the Financial Year. Otherwise, the User's average weekday Settlement Period 35 half-hourly metered (HH) gross demand and embedded export in the Financial Year to date is compared to the equivalent average gross demand and embedded export for the corresponding days in the preceding year.  The percentage difference is then applied to the User's HH gross demand and embedded export at Triad in the preceding Financial Year to derive a forecast of the User's HH gross demand and embedded export at Triad for this Financial Year.

- The User's non half-hourly metered (NHH) energy consumption over the period 16:00 hrs to 19:00 hrs every day in the Financial Year to date is compared to the equivalent energy consumption over the corresponding days in the preceding year.  The percentage difference is then applied to the User's total NHH energy consumption in the preceding Financial Year to derive a forecast of the User's NHH energy consumption for this Financial Year.

For new Users who have completed a Use of System Supply Confirmation Notice in the current Financial Year:

- The User's average weekday Settlement Period 35 half-hourly metered (HH) gross demand and embedded export over the last complete month for which The Company has settlement data is calculated.  Total system average HH gross demand and embedded export for weekday Settlement Period 35 for the corresponding month in the previous year is compared to total system HH gross demand and embedded export at Triad in that year and a percentage difference is calculated.  This percentage is then applied to the User's average HH gross demand and embedded export for weekday Settlement Period 35 over the last month to derive a forecast of the User's HH gross demand and embedded export at Triad for this Financial Year.

- The User's non half-hourly metered (NHH) energy consumption over the period 16:00 hrs to 19:00 hrs every day over the last complete month for which The Company has settlement data is noted.  Total system NHH energy consumption over the corresponding month in the previous year is compared to total system NHH energy consumption over the remaining months of that Financial Year and a percentage difference is calculated.  This percentage is then applied to the User's NHH energy consumption over the month described above, and all NHH energy consumption in previous months is added, in order to derive a forecast of the User's NHH metered energy consumption for this Financial Year.

- Determination of The Company’s Forecast for Demand Charge Purposes illustrates how the demand forecast will be calculated by The Company.

### Reconciliation of Demand Charges and TNUoS Charges in the event of exceeding the limits to Generator charges in the Limiting Regulation

- The reconciliation process is set out in the CUSC.  The demand reconciliation process compares the monthly charges paid by Users against actual outturn charges.  Due to the Settlements process, reconciliation of demand charges is carried out in two stages; initial reconciliation and final reconciliation.

- In the event of annual average transmission charges incurred by Generator Users in the Financial Year not being in compliance with the upper or lower limits in the Limiting Regulation an Ex-Post Reconciliation adjustment will be applied to Generator and Demand Users to bring charges back into compliance.

### Initial Reconciliation of demand charges

- The initial reconciliation process compares Users' demand forecasts, The Company’s FDSC Forecast and Unmetered Supply Volume Forecast and corresponding monthly charges paid over the year against actual outturn data (using latest Settlement data available at the time) and corresponding charges.  Initial reconciliation is carried out in three parts; Initial Reconciliation Part 1 deals with the reconciliation of half-hourly metered demand charges, Initial Reconciliation Part 2 deals with the reconciliation of non-half-hourly metered demand charges and Initial Reconciliation Part 3 deals with the reconciliation of Transmission Demand Residual charges.

### Initial Reconciliation Part 1– Half-hourly metered demand

- The Company will identify the periods forming the Triad once it has received Central Volume Allocation data from the Settlement Administration Agent for all days up to and including the last day of February. Once The Company has notified Users of the periods forming the Triad they will not be changed even if disputes are subsequently resolved which would change the periods forming the Triad.

- Initial outturn charges for half-hourly metered gross demand will be determined using the latest available data of actual average Triad gross demand (kW) multiplied by the zonal gross demand tariff(s) (£/kW) applicable to the months concerned for each zone for that Financial Year.  These actual values are then reconciled against the monthly charges paid in respect of half-hourly gross demand.

- Initial outturn charges for half-hourly metered embedded export will be determined using the latest available data of actual average Triad embedded export (kW) multiplied by the zonal embedded export tariff(s) (£/kW) applicable to the months concerned for each zone for that Financial Year.  These actual values are then reconciled against the monthly charges paid in respect of half-hourly embedded exports.

### Initial Reconciliation Part 2 – Non-half-hourly metered demand

- Actual payments for non-half-hourly metered demand will be determined using the latest available actual energy consumption data (kWh) for the period 16:00 hrs to 19:00 hrs inclusive (i.e. Settlement Periods 33 to 38) over the year multiplied by the energy consumption tariff(s) (p/kWh) applicable to the months concerned for each zone.  These actual values are then reconciled against the monthly charges paid in respect of non-half-hourly energy consumption.

### Initial Reconciliation Part 3 – FDSC and Unmetered Supply Volume

14.17.29A Actual payments for Transmission Demand Residual charges will be determined using the latest FDSC and Unmetered Supply Volume (provided by the Settlement Administration Agent) multiplied by the relevant Transmission Demand Residual Tariff. These actual values are then reconciled against the charges paid by the User.

### Final Reconciliation of demand charges

- The final reconciliation process compares Users' charges (as calculated during the initial reconciliation process using the latest available data) against final outturn demand charges (based on finalised FDSC and Unmetered Supply Volume, settlement data of half-hourly gross demand, embedded exports and non-half-hourly energy consumption).

- Final actual charges will be determined using the Final Demand reconciliation data taken from the Final Reconciliation Settlement Run or the Final Reconciliation Volume Allocation Run.

### Reconciliation of manifest errors

- In the event that a manifest error, or multiple errors in the calculation of TNUoS tariffs results in a material discrepancy in a Users TNUoS tariff, the reconciliation process for all Users qualifying under Section 14.17.34 will be in accordance with Sections 14.17.25 to 14.17.31. The reconciliation process shall be carried out using recalculated TNUoS tariffs.  Where such reconciliation is not practicable, a post-year reconciliation will be undertaken in the form of a one-off payment.

- A manifest error shall be defined as any of the following:

a) an error in the transfer of relevant data between the Transmission Licensees or Distribution Network Operators;

b) an error in the population of the Transport Model with relevant data;

c) an error in the function of the Transport Model; or

d) an error in the inputs or function of the Tariff Model.

- A manifest error shall be considered material in the event that such an error or, the net effect of multiple errors, has an impact of the lesser of either:

        - an error in a User’s TNUoS tariff of at least +/-£0.76/kW; or

        - an error in a User’s TNUoS tariff which results in an error in the annual TNUoS charge of a User in excess of +/-£377,735

Thresholds are stated in 2020/21 money and will be indexed annually by the Transmission Owner Price Index (TOPI) thereafter, starting from 1st April 2021.

- A manifest error shall only be reconciled if it has been identified within the Financial Year for which the error has an effect.  Errors identified outside of this period will not be eligible for reconciliation retrospectively.

Ex-post Reconciliation of Generator and Demand Charges in the event of exceeding the limits to Generator charges in the Limiting Regulation

14.17.36 The Company shall, following the completion of each Financial Year, produce a statement setting out the annual average transmission charges paid in aggregate by Generators in €/MWh as per paragraph 14.14.5.

14.17.37 In the event that the annual average transmission charges attributable to Generators exceeds the upper limit established in the Limiting Regulation then an Ex-post Reconciliation will be calculated for Generation and Demand Users as per the below and will be invoiced at the time of generation reconciliation and initial demand reconciliation.

- The Ex-post Reconciliation amount for Demand Users will be calculated as :

Dadj = GCharge (Actual)- GOA*CapEC*ERA

Where:

Dadj = Revenue to be recovered from Demand

GOA = Actual generator output in the previous Financial Year

CapEC =  The upper limit of the Limiting Regulation

ERA = Actual exchange rate in the previous Financial Year

GCharge (Actual) = Actual charges to Generators in the previous Financial Year

- The rate applied to HH gross Demand, in order to adjust for any TNUoS recovery from generation outside of the range of the Limiting Regulation

DRadj=DadjGTD

Where

DRadj = Rate applied to AHHD in £/kW

GTD = Total actual system metered Gross Triad Demand (kW)

- The rate applied to NHH energy consumption, in order to adjust for compliance with the Limiting Regulation in the Financial Year is calculated by:

ERadj=Dadj-(AHHD × DRadj) ANHHCx 100

Where

AHHD = The actual gross half-hourly metered Triad Demand (kW) for HH Demand

ERadj = Rate applied to energy consumption for the Demand Recovery in p/kWh

ANHHC = Total actual annual non-half-hourly metered energy consumption (kWh) for the period 16:00 hrs to 19:00 hrs inclusive (i.e. Settlement Periods 33 to 38) for each day of the preceding Financial Year ,

- The Ex-Post Reconciliation amount for Generation Users will be calculated as:

Gadj = Dadj*-1

Where:

Dadj = Revenue to be recovered from demand Users

Gadj = Revenue to be paid to generation Users

- The rate applied to Generator Chargeable Capacity in the preceding Financial Year, in order to adjust for any recovery of TNUoS from generation outside of the range of the Limiting Regulation, is:

GRadj = GadjChargeableCapacity

Where:

GRadj = Adjustment rate to be applied to Generators

Chargeable Capacity = As per paragraph 14.18.6

14.17.38 In the event that the annual average transmission charges attributable to Generators is below the lower limit established in the Limiting Regulation then an Ex-post Reconciliation will be calculated for Generator and Demand Users as per the below and will be invoiced at the time of generation reconciliation and initial demand reconciliation.

- Ex-post reconciliation for Demand Users:

Dadj = GCharge (Actual)-0

Where:

Dadj = Revenue to be credited to Demand

GCharge (Actual) = Actual charges to Generators in the previous Financial  Year

- The rate applied to HH gross Demand, in order to adjust for any recovery of TNUoS from generation outside of the range of the Limiting Regulation

DRadj=DadjGTD

Where

DRadj = Rate applied to AHHD in £/kW

GTD = Total actual system metered gross triad demand (kW)

- The rate applied to NHH energy consumption, in order to adjust for compliance with the Limiting Regulation in the Financial Year is calculated by:

ERadj=Dadj -(AHHD× DRadj) ANHHCx 100

Where

AHHD = The actual gross half-hourly metered Triad Demand (kW) for HH Demand

ERadj = Rate applied to energy consumption for the Demand Recovery in p/kWh

ANHHC = Total actual annual non-half-hourly metered energy consumption (kWh) for the period 16:00 hrs to 19:00 hrs inclusive (i.e. Settlement Periods 33 to 38) for each day of the preceding Financial Year

Ex-post reconciliation for Generation Users:

The recovery from Generator Users will be

Gadj = Dadj*-1

Where:

Dadj =  Revenue to be dispersed to Demand

Gadj = Revenue to be recovered from Generation

- The rate applied to Generator Chargeable Capacity in the preceding Financial Year, in order to adjust for any recovery of TNUoS from generation outside of the range of the Limiting Regulation, is:

GRadj = GadjChargeableCapacity

Where:

GRadj = Adjustment rate to be applied to Generators

Chargeable Capacity = As per paragraph 14.18.6

- Implementation of P272

      - BSC modification P272 requires Suppliers to move Profile Classes 5-8 to Measurement Class E - G (i.e. moving from NHH to HH settlement) by April 2016. The majority of these meters are expected to transfer during the preceding Financial Year up until the implementation date of P272 and some meters will have been transferred before the start of 1ST April 2015. A change from NHH to HH within a Financial Year would normally result in Suppliers being liable for TNUoS for part of the year as NHH and also being subject to HH charging. This section describes how The Company will treat this situation in the transition to P272 implementation for the purposes of TNUoS charging; and the forecasts that Suppliers should provide to The Company.

      - Notwithstanding 14.17.13, for each Financial Year which begins after 31 March 2015 and prior to implementation of BSC Modification P272, all demand associated with meters that are in NHH Profile Classes 5 to 8 at the start of that Financial Year as well as all meters in Measurement Classes E G will be treated as Chargeable Energy Capacity (NHH) for the purposes of TNUoS charging for the full Financial Year.

      - The Company will calculate the Chargeable Energy Capacity associated with meters that have transferred to HH settlement but are still treated as NHH for the purposes of TNUoS charging from Settlement data provided directly from Elexon i.e. Suppliers need not Supply any additional information if they accept this default position

      - The forecasts that Suppliers submit to The Company under CUSC 3.10, 3.11 and 3.12 for the purpose of TNUoS monthly billing referred to in 14.17.20 and 14.17.21 for both Chargeable Demand Capacity and Chargeable Energy Capacity should reflect this position i.e. volumes associated those Metering Systems that have transferred from a Profile Class to a Measurement Class in the BSC (NHH to HH settlement) but are to be treated as NHH for the purposes of TNUoS charging should be included in the forecast of Chargeable Energy Capacity and not Chargeable Demand Capacity.

      - Where a Supplier wishes for Metering Systems that have transferred from Profile Class to Measurement Class in the BSC (NHH to HH settlement) prior to 1st April 2015, to be treated as Chargeable Demand Capacity (HH/ Measurement Class settled) it must inform The Company prior to October 2015. The Company will treat these as Chargeable Demand Capacity (HH / Measurement Class settled) for the purposes of calculating the actual annual liability for the Financial Years up until implementation of P272. For these cases only, the Supplier should notify The Company of the Meter Point Administration Number(s) (MPAN). For these notified meters the Supplier shall provide The Company with verified metered demand data for the hours between 4pm and 7pm of each day of each Financial Year up to implementation of P272 and for each Triad half hour as notified by The Company prior to May of the following Financial Year up until two years after the implementation of P272 to allow reconciliation (e.g. May 2017 and May 2018 for the Financial Year 2016/17). Where the Supplier fails to provide the data or the data is incomplete for a Financial Year TNUoS charges for that MPAN will be reconciled as part of the Supplier’s NHH BMU (Chargeable Energy Capacity). Where a Supplier opts, if eligible, for TNUoS liability to be calculated on Chargeable Demand Capacity it shall submit the forecasts referred to in 14.17.35.4 taking account of this.

      - The Company will maintain a list of all MPANs that Suppliers have elected to be treated as HH. This list will be updated monthly and will be provided to registered Suppliers upon request.

- HH Elective Metering from 1st April 2017. The following section describes how meters migrating to, or already within, Measurement Classes E,F and G will be charged in terms of TNUoS after 31st March 2017.

- A change from NHH to HH within a Financial Year would normally result in Suppliers being liable for TNUoS for part of the year as NHH and also being subject to HH charging. This section describes how The Company will treat this situation for Non-Half Hourly (NHH) meters migrating to Measurement Classes E, F & G for the Financial Year which begins after 31 March 2017.

- Notwithstanding 14.17.13, for each Financial Year which begins after 31 March 2017 demand associated with Measurement Classes F and G will be treated as Chargeable Energy Capacity (NHH) for the purposes of TNUoS charging. Demand associated with Measurement Class E will continue to be treated as Chargeable Demand Capacity (HH).

- The Company will calculate the Chargeable Energy Capacity associated with meters that have transferred to HH settlement but are still treated as NHH for the purposes of TNUoS charging from Settlement data provided directly from ELEXON i.e. Suppliers need not Supply any additional information.

- The forecasts that Suppliers submit to The Company under CUSC 3.10, 3.11, 3.12 and 14.17.19 for the purpose of TNUoS monthly billing referred to in 14.17.20 and 14.17.21 for both Chargeable Demand Capacity and Chargeable Energy Capacity should reflect the basis on which demand will be charged for TNUoS.

- The following section describes the arrangements that will apply in relation to introduction of half-hourly settlement on a market-wide basis.

14.17.41.1	Subject to 14.17.40.2, when an MPAN is settled under the MHHS Target Operating Model (as defined in Section C12 of the Balancing and Settlement Code) its associated BM Unit gross demand, embedded export or energy consumption will be charged TNUoS as per 14.17.13, except in the following circumstances:

          - When an MPAN with a non half-hourly Metering System transitions to a half hourly Metering System with a current transformer at Non-Domestic Premises (as defined in a Supply Licence) and is reported as such from BSCCo (as defined in the Balancing and Settlement Code), to The Company, its associated BM Unit gross demand or embedded export will be treated as Chargeable Demand Locational Capacity or Chargeable Embedded Export Capacity respectively.

          - When an MPAN with a non half-hourly Metering System transitions to a half-hourly Metering System with whole current metering at Non- Domestic Premises and is reported as such from BSCCo to The Company, its associated BM Unit energy consumption will be treated as Chargeable Energy Capacity.

          - When an MPAN transitions to a half-hourly Metering System at Domestic Premises (as defined in a Supply Licence) and is reported as such from BSCCo to The Company, its associated BM Unit energy consumption will be treated as Chargeable Energy Capacity

## Further Information

- Paragraph 14.25 (Reconciliation of Demand Related Transmission Network Use of System Charges) of this statement illustrates how the monthly charges are reconciled against the actual values for gross demand, embedded consumption and consumption for half-hourly gross demand, embedded export and non-half-hourly metered demand respectively.

- The Statement of Use of System Charges contains the £/kW zonal demand locational tariffs, the £/kW zonal embedded export tariffs, the p/kWh energy consumption tariffs, and the Transmission Demand Residual tariffs  for the current charging year.

- Transmission Network Use of System Charging Flowcharts of this statement contains flowcharts demonstrating the calculation of these charges for those parties liable.

# 14.18 Generation charges

## Parties Liable for Generation Charges

- The following CUSC parties shall be liable for generation charges:

- Parties of Generators that have a Bilateral Connection Agreement with The Company.

- Parties of Licensable Generation that have a Bilateral Embedded Generation Agreement with The Company.

- 14.26 Classification of parties for charging purposes provides an illustration of how a party is classified in the context of Use of System charging and refers to the relevant paragraphs most pertinent to each party.

## Structure of Generation Charges

- Generation Tariffs are comprised of Wider and Local Tariffs. The Wider Tariff is comprised of (i) a Peak Security element, (ii) a Year Round Not-Shared element, (iii) Year Round Shared element and (iv) the Adjustment tariff (if required). The Peak Security element of the Wider Tariff is not applicable for intermittent generators as the PS flag is set to zero. The Year Round Not Shared element is multiplied by the YRNS Flag, which for Non-Conventional Carbon generators results  in no change to the tariff, whereas for Conventional Carbon generators the tariff is reduced by ALF

- The Local Tariff contains a substation element and may also contain a circuit element.  Specifically, all transmission connected generation will be liable to pay a local substation charge, with some of these also being liable to pay a local circuit charge. For the avoidance of doubt, embedded generation has a zero local tariff.

- The intention of the charging rules is to charge the same physical entity only once.

- The basis of the generation charge for Power Stations is the Chargeable Capacity and the short-term chargeable capacity (as defined below for positive and negative charging zones).

- If there is a single set of Wider and Local generation tariffs within a Financial Year, the Chargeable Capacity is multiplied by the relevant generation tariff to calculate the annual liability of a generator.

Local Annual Liability=Chargeable Capacity×Local Tariff

The Wider Tariff is broken down into components as described in 14.18.3. The breakdown of the Wider Charge for Conventional and Intermittent Power Stations are given below:

Conventional Low Carbon-

Wider Annual Liability=Chargeable Capacity ×PS Tariff+YRNS Tariff+YRS Tariff×ALF+AdjTariff

Conventional Carbon

Wider Annual Liability= Chargeable Capacity x (PS Tariff+(YRNS Tariff × ALF)+(YRS Tariff × ALF)+Adj Tariff)

Wider Annual Liabiity= Chargeable Capacity × (PS Tariff+(YRNS Tariff × ALF)+(YRS Tariff × ALF)+AdjTariff)Wider Annual Liabiity Chargeable Capacity x (PS Tariff+(YRNS Tariff × ALF)+(YRS Tariff × ALF)+Residual Tariff)Wider Annual Liabiity Chargeable Capacity x (PS Tariff+(YRNS Tariff × ALF)+(YRS Tariff × ALF)+Residual Tariff

Intermittent -

Wider Annual Liability=Chargeable Capacity×YRNS Tariff+YRS Tariff×ALF+AdjTariff

Where:

PS Tariff	=	Wider Peak Security Tariff

YRNS Tariff 	=	Wider Year Round Not-Shared Tariff

YRS Tariff	=	Wider Year Round Shared Tariff

Adj Tariff	=	Adjustment Tariff

- If multiple sets of Wider and Local generation tariffs are applicable within a single Financial Year, the Chargeable Capacity is multiplied by the relevant tariffs pro rated over the entire Financial Year, across the months that they are applicable for.

where:

Liability 1	=	Original annual liability,

Liability 2	=	Revised annual liability,

a	=	Number of months over which the original liability is applicable,

b	=	Number of months over which the revised liability is applicable.

- For the avoidance of doubt if there are multiple sets of Wider and Local generation tariffs applicable within a single Financial Year and a tariff changes from being positive to negative or vice versa, the Chargeable Capacity for the entire Financial Year will be determined based on the net position of the pro rated tariffs for each affected generator.

## Basis of Wider Generation Charges

### Generation with positive wider tariffs

- The Chargeable Capacity for Power Stations with positive wider generation tariffs is the highest Transmission Entry Capacity (TEC) contracted for that Power Station. In the event that the TEC available to a Power Station increases (a TEC Increase) or decreases (a TEC Decrease) during a Financial Year, the Chargeable Capacity will increase or decrease (as applicable) on the date that such TEC Increase or TEC Decrease takes effect. For the avoidance of doubt  prorated charges are intended for generating assets permanently increasing or decreasing TEC in their Bilateral Agreement as  per 14.18.19.5..  A Power Station should not exceed its TEC in any Charging Year as to do so would be in breach of the CUSC, except where it is entitled to do so under the specific circumstances laid out in the CUSC (e.g. where a User has been granted Short Term Transmission Entry Capacity, STTEC).  For the avoidance of doubt, TNUoS Charges will be determined on the chargeable capacity regardless of whether or not it enters into a temporary TEC Exchange (as defined in the CUSC).

- The short-term chargeable capacity for Power Stations situated with positive generation tariffs is any approved STTEC or LDTEC applicable to that Power Station during a valid STTEC Period or LDTEC Period, as appropriate.

- For Power Stations, the short-term chargeable capacity for LDTEC with positive generation tariffs referred to in Paragraph 14.18.11 will be the capacity purchased either on a profiled firm or indicative basis and shall be assessed according to the capacity purchased on a weekly basis.  The short-term chargeable capacity for LDTEC in any week may comprise of a number of increments, which shall be determined by considering LDTEC purchased previously in the Financial Year (whether or not in the same LDTEC Period).  For example, if in a given week the LDTEC is 200MW but in a previous week the LDTEC had been 150MW, the short-term chargeable capacity in the latter week would comprise of two increments: one of 150MW and a second of 50MW.  Further examples are provided in 14.16.6.

### Generation with negative wider tariffs

- The Chargeable Capacity for Power Stations with negative wider generation tariffs is the average of the capped metered volumes during the three Settlement Periods described in 14.18.14 below, for the Power Station (i.e. the sum of the metered volume of each BM Unit associated with Power Station in Appendix C of its Bilateral Agreement).  A Power Station should not exceed its TEC as to do so would be in breach of the CUSC, except where it is entitled to do so under the specific circumstances laid out in the CUSC (e.g. where a User has been granted Short Term Transmission Entry Capacity).  If TEC is exceeded, the metered volumes would each be capped by the TEC as per Bilateral Agreement available to   For the avoidance of doubt, Transmission Network Use of System Charges will be determined on the Chargeable Capacity (as per 14.18.10)  regardless of whether or not it enters into a temporary TEC Exchange (as defined in the CUSC).

- The three Settlement Periods are those of the highest metered volumes for the Power Station and the two half hour Settlement Periods of the next highest metered volumes which are separated from the highest metered volumes and each other by at least 10 Clear Days, between November and February of the relevant Financial Year inclusive.  These Settlement Periods do not have to coincide with the Triad.

Example

If the highest TEC for a Power Station were 250MW and the highest metered volumes and resulting capped metered volumes were as follows:

| Date | 19/11/08 | 13/12/08 | 06/02/09 |
| --- | --- | --- | --- |
| Highest Metered Volume in month (MW) | 245.5 | 250.3 | 251.4 |
| Capped Metered Volume (MW) | 245.5 | 250.0 | 250.0 |

Then, the chargeable Capacity for the Power Station would be:

= 248.5 MW

Note that in the example above, the Generator has exceeded its TEC on 13 December 2007 and 6 February 2008 and would therefore be in breach of the CUSC unless the generator had an approved STTEC or LDTEC value. (The STTEC and LDTEC charge for negative zones is currently set at zero).

- The short-term chargeable capacity for Power Stations with negative generation tariffs is any approved STTEC or LDTEC applicable to that Power Station during a valid STTEC Period or LDTEC Period, as applicable.

- For Power Stations with negative generation tariffs, the short-term chargeable capacity for LDTEC referred to in Paragraph 14.18.15 will be the capacity purchased either on a profiled firm or indicative basis and shall be assessed according to the capacity purchased on a weekly basis.  The short-term chargeable capacity for LDTEC in any week may comprise of a number of increments, which shall be determined by considering LDTEC purchased previously in the Financial Year (whether or not in the same LDTEC Period).  For example, if in a given week the LDTEC is 200MW but in a previous week the LDTEC had been 150MW, the short-term chargeable capacity in the latter week would comprise of two increments: one of 150MW and a second at 50MW.

- As noted above, a negative LDTEC tariff in negative generation charging zones is set to zero.  Accordingly no payments will be made for use of LDTEC (in any of its forms) in these zones.

## Basis of Local Generation Charges

- The Chargeable Capacity for Power Stations will be the same as that used for wider generation charges, except that each component of the local tariff shall be considered separately as to whether it is a positive or negative tariff component.  This means that where a local circuit tariff is negative, the final charging liability for this element will be based on actual metered output as described in Paragraph 14.18.13.

## Monthly Charges

14.18.19 	The Transmission Network Use of System Generation Charges for each Financial Year will be based on the Power Station Transmission Entry Capacity (TEC) for each User as set out in their Bilateral Agreement.

This annual Transmission Network Use of System generation charge is paid as follows:

      - The User shall be liable to pay The Company the annual Transmission Network Use of System Generation Charges, pro-rated by a factor determined by the number of days from the relevant Charging Date to the last day of the Financial Year in which it occurs, divided by the total number of days in that Financial Year, and

      - For each subsequent Financial Year, the User shall be liable for the annual Transmission Network Use of System Generation Charges split evenly over the months in the Financial Year, and

      - The Transmission Network Use of System Generation Charges in the Financial Year in which the User’s Equipment (or, where relevant, the applicable stage/phase of the User’s Equipment) is Disconnected from the National Electricity Transmission System at the Connection Site shall be apportioned as follows: the User shall be liable to pay The Company the annual Transmission Network Use of System Generation Charges, pro-rated by a factor determined by: the number of days from the first day of the Financial Year in which the relevant Disconnection occurs, to the date of that Disconnection divided by the total number of days in that Financial Year.

      - For generation that is permanently increasing or reducing its Transmission Entry Capacity, the Transmission Network Use of System Generation Charges in the Financial Year in which the increase or decrease occurs shall be apportioned. For the avoidance of doubt, nothing in this section 14.18.19 removes the need to provide notice under section 6.30.1 of the CUSC, where applicable in the circumstances.

      - For positive final generation tariffs, if a User’s Transmission Entry Capacity  increases or decreases during a Financial Year, the party will be liable for the additional or reduced charge incurred (as applicable) on a pro-rated basis (as defined in 14.18.19.2) which will be recovered uniformly across the remaining chargeable months in the relevant Financial Year (subject to Paragraph 14.18.19.6 below).

      - If a Power Station Changes its Transmission Entry Capacity in its Bilateral Agreement and later reverses that change, such undergoes a declared change in Transmission Entry Capacity (and specified in its Bilateral Agreement) followed by a later directional reversal of that change, such changes shall not be treated as permanent (and therefore will not be subject to pro-rated charging) unless a minimum dormant period of 24 consecutive calendar months has elapsed between the opposing changes.

Any declared permanent increase or decrease in TEC which benefited from pro-rated monthly charges (in accordance with Clause 14.18.19.4) which are subsequently reversed will result in charges being adjusted with effect from the date of the initial increase/decrease along with interest being charged on the sums due.

- The provisions described above for increases in TEC during the Financial Year shall not apply where the LDTEC (in any of its forms) has been approved for use before the TEC is available, which will typically mean the LDTEC has been approved after the TEC increase has been approved.  In such instances, the party shall commence payments for TEC during the LDTEC Period for LDTEC purchased up to the future level of TEC and LDTEC Charges will only apply to LDTEC that is incremental to the TEC increase.  For the avoidance of doubt, where TEC has been approved after LDTEC in a given year, these provisions shall not apply and the LDTEC shall be considered additional to the TEC and charged accordingly.

## Ad hoc Charges

- For each STTEC period successfully applied for, a charge will be calculated by multiplying the STTEC by the tariff calculated in accordance with Paragraph 14.16.3. The Company will invoice Users for the STTEC charge once the application for STTEC is approved.

- For Power Stations utilising LDTEC (in any of its forms) the LDTEC Charge for each LDTEC Period is the sum of the charging liabilities associated with each incremental level of short term chargeable capacity provided by LDTEC within the LDTEC Period (assessed on a weekly basis).  The charging liability for a given incremental level of short term chargeable capacity is the sum of:

i)	the product of the higher tariff rate (calculated in accordance with Paragraph 14.16.6) and capacity purchased at this increment for the first 17 weeks in a Financial Year (whether consecutive or not); and

ii)	the product of the lower tariff rate (calculated in accordance with Paragraph 14.16.6) and capacity purchased at this increment in any additional weeks within the same Financial Year (whether consecutive or not).

- For each LDTEC Period successfully applied for, the LDTEC Charge will be split evenly over the relevant LDTEC Period and charged on a monthly basis.  LDTEC charges will apply to both LDTEC (in any of its forms) and Temporary Received TEC held by a User.  For the avoidance of doubt, the charging methodology will not differentiate between access rights provided to a generator by LDTEC or through Temporary Received TEC obtained through a Temporary TEC Exchange (as defined in the CUSC).

Example

The diagrams below show two cases where LDTEC has been purchased: in Case A, two LDTEC Periods have been purchased; and in Case B one LDTEC Period has been purchased.  The total capacity purchased in both cases is the same.  The top diagrams illustrate the capacity purchased, while lower diagrams illustrate the incremental levels of short term chargeable capacities of LDTEC and the tariff rate that would apply to that capacity.

In both cases, the total amount charged for the LDTEC would be the same:

| - Capacity charges at the higher tariff rate: - 17 weeks at the 100MW increment - 12 weeks at the 30MW increment | - Capacity charges at the lower tariff rate: - 19 weeks at the 100MW increment |
| --- | --- |

## Embedded Transmission Use of System Charges “ETUoS”

- The ETUoS charges are a component of Use of System charges levied on offshore generators whose offshore transmission connection is embedded in an onshore distribution network.  The charge relates to the provision and use of the onshore distribution network.

- The main purpose of ETUoS charges is to pass through the charges that are levied by the DNO on the NETSO to the offshore generator(s).  This charge, known as the ETUoSDNO charge, reflects the charges levied by the DNO for the costs of any works on and use of the DNO network in accordance with the DNO’s charging statements and will include, but is not limited to, upfront charges and capital contributions in respect of any works as well as the ongoing and annual Use of System charges for generation connected to the distribution network.

- In the case of some relevant transitional offshore generation projects, ETUoS will also be used to pass through historic DNO capital contributions forming part of the Offshore Transmission Owner tender revenue stream, this is known as the ETUoSOFTO tariff.

- The ETUoSOFTO tariff shall be the ratio of the Offshore Transmission Owner revenue (£) associated with DNO capital contributions and the Transmission Entry Capacity (TEC) of the generation connected to the offshore substation.

- In the year of asset transfer to the OFTO, the ETUoSOFTO tariff would be calculated as follows:

DNRevOFTO1TEC

Where:

DNRevOFTO1 		=	The offshore revenue associated to DN

capital contributions, in £, for Year 1

TEC 			=	the Transmission Entry Capacity of the

generation connected to the offshore substation

- In all subsequent years, the ETUoSOFTO tariff would be calculated as follows:

AvDNRevOFTOTEC

Where:

AvDNRevOFTO	=	The annual offshore revenue associated to DN capital contributions averaged over the remaining years of the National Electricity Transmission System Operator (NETSO) price control, in £

TEC	=	the Transmission Entry Capacity of the generation connected to the offshore substation

- ETUoSOFTO tariffs shall be reviewed at the start of every onshore price control period. For each subsequent year within the price control period, these shall be inflated in the same manner as the associated Offshore Transmission Owner Revenue.

- The specific nature of the ETUoSDNO charge and the payment profile for these will depend upon the charging arrangements of the relevant DNO and reference should be made to the relevant DNO’s charging statement.  In terms of applicable transitional offshore generation projects the ETUoSOFTO payment profile will be consistent with the recovery of the Offshore Transmission Owner revenue stream, and paragraph 14.18.27.

- Where a DNO’s charge relates to more than one offshore generator, the related ETUoSDNO charge will represent a straight pass through of the distribution charge specific to each relevant offshore generator.  Where specific information is not available, charges will be pro-rated based on the TEC of the relevant offshore generators connected to that offshore network.

- Invoices for ETUoSDNO charges shall be levied by The Company on the offshore generator as soon as reasonably practicable after invoices have been received by The Company for payment such that The Company can meet its payment obligations to the DNO.  The initial payments and payment dates will be outlined in a User’s Construction Agreement and/or Bilateral Agreement.

- As the ETUoSDNO charges reflect the DNO charges to The Company, such charges will be subject to variation when varied by the DNO.  Where the User disputes regarding the ETUoSDNO charge please note that this will result in a dispute between The Company and DNO under the DCUSA.

## Reconciliation of Generation Charges

- The reconciliation process is set out in the CUSC and in line with the principles set out above.

- In the event of a manifest error in the calculation of TNUoS charges which results in a material discrepancy in a User’s TNUoS charge as defined in Sections 14.17.33 to 14.17.35, the generation charges of Users qualifying under Section 14.17.34 will be reconciled in line with 14.18.19 and 14.18.20 using the recalculated tariffs.

## Further Information

- The Statement of Use of System Charges contains the £/kW generation zonal tariffs for the current Financial Year.

# 14.19 Data Requirements

## Data Required for Charge Setting

- Users who are Generators or Interconnector Asset Owners provide to The Company a forecast for the following Financial Year of the highest Transmission Entry Capacity (TEC) applicable to each Power Station or Interconnector for that Financial Year. For Financial Year 2008/9 Scottish Generators or Interconnector Asset Owners provide to The Company a forecast of the equivalent highest ‘export’ capacity figure.  This data is required by The Company as the basis for setting TNUoS tariffs.  The Company may request these forecasts in the November prior to the Financial Year to which they relate, in accordance with the CUSC.  Additionally users who are Generators provide to The Company details of their generation plant type.

- Users who are owners or operators of a User System (e.g. Distribution companies) provide a forecast for the following Financial Year of the Natural Demand attributable to each Grid Supply Point equal to the forecasts of Natural Demand under both Annual Average Cold Spell (ACS) Conditions and a forecast of the average metered Demand attributable to such Grid Supply Point for the Triad.  This data Electricity Ten Year Statement (ETYS) is compiled from week 24 data submitted in accordance with the Grid Code.

- The BSCCo will provide data to The Company with respect to Final Demand Site counts and Unmetered Supply volumes to enable the development of the Transmission Demand Residual Tariffs.

- For the following Financial Year, The Company shall use these forecasts as the basis of Transmission Network Use of System charges for such Financial Year.  A description of how this data is incorporated is included in 14.15 Derivation of the Transmission Network Use of System Tariff.

- If no data is received from the User, then The Company will use the best information available for the purposes of calculation of the TNUoS tariffs.  This will normally be the forecasts provided for the previous Financial Year.

## Data Required for Calculating Users’ Charges

- In order for The Company to calculate Users' TNUoS charges, Users who are Suppliers shall provide to The Company forecasts of half-hourly and non-half-hourly demand in accordance with paragraph 14.17.19 and 14.17.20 and in accordance with the CUSC.

# 14.20 Applications

- Application fees are payable in respect of applications for new Use of System agreements; modifications to existing agreements; and applications for short-term access products or services. These are based on the reasonable costs that transmission licensees incur in processing these applications.

Applications for short-term access

- Application fees for short-term access products or services are fixed and detailed in the Statement of Use of System Charges. These are non-refundable except for the following limited instances:

- Where a User (or Users) withdraw their application in accordance with any interactivity provisions that may be contained within the CUSC; or

- Where the application fee covers ongoing assessment work that is contingent on the acceptance of the offer.

- In either case, the refunded amount will be proportional to the remaining assessment time available.

- To ensure that application fees for short-term access are cost reflective, fees may be comprised of a number of components.  For instance, the LDTEC Request Fee is comprised of a number of components and the total fee payable is the sum of those components that apply to the type(s) of LDTEC Offer(s) requested.  For example:

- The LDTEC Request Fee for an LDTEC Block Offer is the basic request fee.

- The LDTEC Request Fee for an LDTEC Indicative Block Offer is the sum of the basic request fee and the additional rolling assessment fee.

- The LDTEC Request Fee payable for a combined LDTEC Block Offer and LDTEC Indicative Block Offer is the sum of the basic request fee, the additional rolling assessment fee, and the additional combined application fee.

Applications for new or modified existing Use of System Agreements

- Users can opt to pay a fixed price application fee in respect of their application or pay the actual costs incurred. The fixed price fees for applications are detailed in the Statement of Use of System Charges.

- If a User chooses not to pay the fixed fee, the application fee will be based on an advance of transmission licensees’ Engineering and out-of pocket expenses and will vary according to the size of the scheme and the amount of work involved. Once the associated offer has been signed or lapsed, a reconciliation will be undertaken. Where actual expenses exceed the advance, The Company will issue an invoice for the excess. Conversely, where The Company does not use the whole of the advance, the balance will be returned to the User.

- The Company will refund the first application fee paid (the fixed fee or the amount post-reconciliation) and consent payments made under the Construction Agreement for new or modified existing agreements.  The refund shall be made either on commissioning or against the charges payable in the first three years of the new or modified agreement. The refund will be net of external costs.

- The Company will not refund application fees for applications to modify a new agreement or modified existing agreement at the User’s request before any charges become payable. For example, The Company will not refund an application fee to delay the provision of a new connection if this is made prior to charges becoming payable.

# 14.21 Transport Model Example

For the purposes of the DCLF Transport algorithm, it has been assumed that the value of circuit impedance is equal to the value of circuit reactance.

Consider the following 3-node network, where generation at node A is intermittent and generation at node B is conventional:

Denotes cable

For both Peak Security and Year Round  generation  backgrounds, the nodal generation is scaled according to the relevant Scaling Factors, as per 14.15.7, such that total system generation equals total system demand minus total imports from external systems.

Scaling Factor Adjustment to 10% minimum

If the SQSS process gives a variable scaling factor less than 10%, then it will be adjusted along with the fixed factors as per the following example.

Total Unscaled TEC = 107600
ACS Demand = 49870

Increasing the variable scaling factor to 10% results in an increase of 5190MW to total scaled generation, which is then counteracted by a uniform 10.06% reduction to all fixed scaling factors (N.B some rounding included below). “Scaling Factor after 10% floor applied” column in the table below gives the required scaling factors to ensure total scaled TEC is equal to 49870 (ACS).

| Generator Type | TEC | Type | Year Round Background SQSS Scaling Factor | Scaled Generation (SQSS) | Scaling Factor after 10% floor applied | Scaled Generation after floor applied for variable |
| --- | --- | --- | --- | --- | --- | --- |
| Biomass | 2000 | Variable | -5% | -100 | 10.00% | 200 |
| CCGT | 30000 | Variable | -5% | -1500 | 10.00% | 3000 |
| CHP | 2000 | Variable | -5% | -100 | 10.00% | 200 |
| Coal | 0 | Variable | -5% | 0 | 10.00% | 0 |
| Hydro | 600 | Variable | -5% | -30 | 10.00% | 60 |
| Interconnectors | 10000 | Fixed | 100% | 10000 | 89.94% | 8994 |
| Nuclear | 6000 | Fixed | 85% | 5100 | 76.45% | 4587 |
| OCGT | 2000 | Fixed | 0% | 0 | 0% | 0 |
| Pump Storage | 10000 | Fixed | 50% | 5000 | 44.97% | 4497 |
| Tidal | - | Fixed | 70% | 0 | 62.96% | 0 |
| Wave | - | Fixed | 70% | 0 | 62.96% | 0 |
| Wind Offshore | 25000 | Fixed | 70% | 17500 | 62.96% | 15740 |
| Wind Onshore | 20000 | Fixed | 70% | 14000 | 62.96% | 12592 |

Total scaled generation for fixed plant types in SQSS = 51600
Reduction required/SQSS fixed generation = 5190/51600 = 10.06%

Peak Security background:

A fixed scaling factor of 0% is applied to intermittent generation at node A and a variable scaling factor is applied to the conventional generation at node B so that the total generation is equal to the total demand.

Node A Generation =

0 * 643MW = 0MW

Node B Generation = 1150/

1500 * 1500MW = 1150MW

This gives the following balanced system , where the actual generation after the application of scaling factors is shown:

Assuming Node A is the reference node†,, each 400kV circuit has impedance X, the 275kV circuit has impedance 2X, the 400kV cable circuit expansion factor is 10 and the 275kV overhead line circuit expansion factor is 2, the DCLF transport algorithm calculates the base case power flows for Peak Security background as follows:

Node B exports, whilst Nodes A and C import. Hence the DCLF algorithm derives flows to deliver export power from Node B to meet import needs at Nodes A and C.

Step 1:	Net export from Node B to Node A is 100MW; both routes BA and BC-CA have impedance 2X; hence 50MW would flow down both routes.

Step 2:	Net export from Node B to Node C is 1000MW; route BC has impedance X and route BA-AC has impedance 3X; hence 750MW would flow down BC and 250MW along BA-AC

Step 3:	Using super-position to add the flows derived in Steps 1 and 2 derives the following;

Flow AC	= -50MW + 250MW	=	200MW

Flow AB	= -50MW – 250MW	=	-300MW

Flow BC	=  50MW + 750MW	=	800MW

Year Round background:

A fixed scaling factor of 70% is applied to intermittent generation at node A and a variable scaling factor is applied to the conventional generation at node B so that the total generation is equal to the total demand.

Node A Generation = 70% * 643MW = 450MW

Node B Generation = (1150-450)/1500 * 1500MW = 700MW

This gives the following balanced system, where the actual generation after the application of scaling factors is shown:

Assuming the same circuit impedances and expansion factors as used above in the Peak Security background, the DCLF transport algorithm calculates the base case power flows for Year Round background as follows:

Nodes A and B export, whilst Node C imports. Hence the DCLF algorithm derives flows to deliver export power from Nodes A and B to meet import needs at Node C.

Step 1:	Net export from Node A is 350MW; route AC has impedance X and route AB-BC has impedance 3X; hence 262.5MW would flow down AC and 87.5MW along AB-BC

Step 2:	Net export from Node B is 650MW; route BC has impedance X and route BA-AC has impedance 3X; hence 487.5MW would flow down BC and 162.5MW along BA-AC

Step 3:	Using super-position to add the flows derived in Steps 1 and 2 derives the following;

Flow AC	=  262.5MW + 162.5MW	=	425MW

Flow AB	= 87.5MW – 162.5MW	=	-75MW

Flow BC	= 87.5MW + 487.5MW	=	575MW

Then, based on the background giving rise to highest flow, each circuit is tagged as either Peak Security or Year Round.

Therefore, circuits AB and BC are tagged as Peak Security and AC is tagged as Year Round.

Total Peak Security cost = (300 X  6) + (800 X 26) = 22,600MWkm

(base case)

Total Year Round cost = 425 X 10  = 4,250 MWkm

(base case)

We then ‘inject’ 1MW of generation at each node with a corresponding 1MW offtake (demand) at the reference node and recalculate the total Peak Security MWkm cost and Year Round MWkm cost (noting that each circuit is only in one background).  The difference  from the base case for Peak Security and Year Round costs is the marginal km or shadow cost for Peak Security and Year Round networks respectively.  The size and direction of the incremental MW is shown below along with the resultant when superimposed on the relevant base case flow (i.e. higher of the Peak Security and Year Round) depicted in brackets:.

To calculate relevant Peak Security and Year Round the marginal km for node C:

Total Peak Security Cost = (300.25 x 6) + (799.75 x 26) =22,595

Total Year Round Cost = 424.25 X 10 = 4,242.5 MWkm

Marginal Peak Security cost = Incremental total Peak Security cost – Base case total Peak Security cost

= 22595 – 22600 = -5MWkm

Marginal Year Round cost = Incremental total Year Round cost – Base case total Year Round cost

= 4242.5 – 4250 = -7.5MWkm

Thus the overall cost has reduced by .5 for Peak Security (i.e. the marginal km = 5) and by 7.5 for Year Round (i.e the Year Round marginal km = -7.5)

# 14.22 Illustrative Calculation of Boundary Sharing Factors (BSFs) and Shared / Not-Shared incremental km

The following illustrative example shows how the boundary sharing factors and shared /  not-shared incremental km are calculated for the transmission system described in the table below.

| Generation Charging Zone | A | B | C | D |
| --- | --- | --- | --- | --- |
| Zonal MWkm | 450 | 350 | 150 | 100 |

The diagram below shows the expanded connectivity of this transmission system.

up

The above figure illustrates how the Year Round marginal km are split into Shared and Not-Shared.

(a) For Boundary AB (where 50MW of the generation is Low Carbon (LC) and 0MW of the generation is Carbon (C) and Year Round boundary marginal km = 100km)  -

which is greater than 0.5, therefore the following formula will be used to calculate the Boundary Sharing Factor (BSF) –

Year Round Shared marginal km = 0.0 * 100km = 0 km

Year Round Not-Shared marginal km = (100 – 0)km = 100 km

(b) For Boundary BC (where 130MW of generation is Low Carbon (LC) and 50MW of generation is Carbon (C) and Year Round boundary marginal km = 200km) –

which is greater than 0.5, therefore the following formula will be used to the BSF –

Year Round Shared marginal km = 0.556 * 200km = 111 km

Year Round Not-Shared marginal km = (200 – 111)km = 89 km

(c) For Boundary CD (where 250MW of generation is Low Carbon (LC) and 170MW of generation is Carbon (C) and Year Round boundary marginal km = 50km) –

which is greater than 0.5, therefore the following formula will be used to calculate the BSF –

Year Round Shared marginal km = 0.81 * 50km = 40.5 km

Year Round Not-Shared marginal km = (50 – 40.5)km = 9.5 km

(d) For Boundary D-rest of system (where 330MW of generation is Low Carbon (LC) and 330MW of generation is Carbon (C) and Year Round boundary marginal km = 100km) –

therefore it is at the threshold at which maximum sharing occurs between LC and C generation. Therefore 100% of the Year Round zonal marginal km will be shared. (i.e. BSF=1.0);

Year Round Shared marginal km = 1.0 * 100 = 100 km

Year Round Not-Shared marginal km = (100 – 100)km = 0 km

The shared zonal marginal km for each generation charging zone will be the sum of the relevant shared boundary marginal km as shown in the table below (assuming the node below D is the centre of the system i.e. zonal MWkm of 0).  These not-shared zonal incremental km are then use to calculate wider £/kW generation tariffs.

| Boundary/Zone | A | B | C | D |
| --- | --- | --- | --- | --- |
| A-B | 0 |  |  |  |
| B-C | 111 | 111 |  |  |
| C-D | 40.5 | 40.5 | 40.5 |  |
| D-rest of system | 100 | 100 | 100 | 100 |
| Shared Zonal MWkm | 251.5 | 251.5 | 140.5 | 100 |
| Total Zonal MWkm | 450 | 350 | 150 | 100 |

The not-shared zonal marginal km for each generation charging zone will be the sum of the relevant not-shared boundary marginal km as shown in the table below (assuming the node below D is the centre of the system i.e. zonal MWkm of 0).  These not-shared zonal incremental km are then use to calculate wider £/kW generation tariffs.

| Boundary/Zone | A | B | C | D |
| --- | --- | --- | --- | --- |
| A-B | 100 |  |  |  |
| B-C | 89 | 89 |  |  |
| C-D | 9.5 | 9.5 | 9.5 |  |
| D-rest of system | 0 | 0 | 0 | 0 |
| Not-Shared Zonal MWkm | 198.5 | 98.5 | 9.5 | 0 |
| Total Zonal MWkm | 450 | 350 | 150 | 100 |

# 14.23 Example: Calculation of Zonal Generation Tariff

Wider

Let us consider all nodes in a generation zone in this example.

The table below shows a sample output of the transport model comprising the node, the Peak Security wider nodal marginal km and Year Round wider nodal marginal km (observed on non-local assets) of an injection at the node with a consequent withdrawal across distributed reference node, the generation sited at the node, scaled to ensure total national generation equals total national demand, for both Peak Security and Year Round generation backgrounds..

| Gen Zone | Node | Wider Nodal Marginal km (Peak Security) | Scaled Generation (Peak Security) | Wider Nodal Marginal km (Year Round) | Scaled Generation (Year Round) |
| --- | --- | --- | --- | --- | --- |
| 4 | ABNE10 | 5.73 | 0.00 | 459.90 | 0.00 |
| 4 | CLAY1S | 239.67 | 0.00 | 306.47 | 0.00 |
| 4 | CLUN1S | 46.41 | 22.90 | 502.16 | 18.76 |
| 4 | COUA10 | 45.39 | 0.00 | 423.30 | 0.00 |
| 4 | DYCE1Q | 162.70 | 0.00 | 357.81 | 0.00 |
| 4 | ERRO10 | 46.82 | 56.13 | 534.03 | 45.99 |
| 4 | FIDD1B | 91.88 | 0.00 | 220.59 | 0.00 |
| 4 | FINL1Q | 79.69 | 12.35 | 495.63 | 10.12 |
| 4 | GRIF1S | 33.31 | 0.00 | 521.16 | 71.40 |
| 4 | KIIN10 | 79.69 | 0.00 | 495.63 | 0.00 |
| 4 | LOCH10 | 79.69 | 35.18 | 495.63 | 28.82 |
| 4 | MILC10 | #### 117.69 | 0.00 | 328.86 | 0.00 |
| 4 | PERS20 | #### 266.00 | 0.00 | 384.05 | 0.00 |
| 4 | TUMB1Q | #### 46.82 | 0.00 | 536.27 | 0.00 |
|  |  | #### Totals | 126.56 |  | 175.09 |

In order to calculate the generation tariff we would carry out the following steps.

- calculate the generation weighted wider nodal shadow costs.

For this example zone this would be as follows:

| Gen Zone | Node | Wider Nodal Marginal km (Peak Security) | Scaled Generation (Peak Security) (MW) | Gen Weighted Wider Nodal Marginal km (Peak Security) | Wider Nodal Marginal km (Year Round) | Scaled Generation (Year Round) (MW) | Gen Weighted Wider Nodal Marginal km (Year Round) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | CLUN1S | 46.41 | 22.90 | 8.39 | 502.16 | 18.76 | 53.80 |
| 4 | ERRO10 | 46.82 | 56.13 | 20.76 | 534.03 | 45.99 | 140.27 |
| 4 | FINL1Q | 79.69 | 12.35 | 7.77 | 495.63 | 10.12 | 28.65 |
| 4 | GRIF1S | N/A | N/A | N/A | 521.16 | 71.40 | 212.52 |
| 4 | LOCH10 | 79.69 | 35.18 | 22.15 | 495.63 | 28.82 | 81.58 |
|  |  | Totals | 126.56 |  |  | 175.09 |  |

i.e. 79.69 x 35.18

126.56

- sum the generation weighted wider nodal shadow costs to give Peak Security and Year Round zonal figures

For this example zone  this would be:

.Peak Security: (8.39 + 20.76+7.77+22.15) km = 59.07km

Year Round: (53.80 + 140.27 + 28.65 + 212.52 + 81.58) = 516.82 km

- In this example we have assumed that accounting for sharing in the Year Round background gives:

Year Round Shared marginal km = 344.56km

Year Round Not-Shared marginal km = 172.26km

- calculate the initial Peak Security wider transport tariff, Year Round Shared wider transport tariff and Year Round Not-Shared wider transport tariff by multiplying the figure in (iii) above by the expansion constant (& dividing by 1000 to put into units of £/kW).

For zone 4 and assuming an expansion constant of £10.07/MWkm and a locational security factor of 1.8:

(a)	 Initial Peak Security wider tariff –

59.07 km * £10.07/MWkm * 1.8	=	£1.071/kW

1000

- Initial Year Round Shared wider tariff -

344.56 km * £10.07/MWkm * 1.8	=	£6.245/kW

1000

- Initial Year Round Not-Shared wider tariff -

172.26 km * £10.07/MWkm * 1.8	 =	£1.309/kW

1000

Local

- If we assume (for the sake of this example) that the generator connecting at CLUN1S is a thermal plant with a Peak Security flag of 1 and an Annual Load Factor (ALF) of 60%, which connects via 10km of 132kV 100MVA rated single circuit overhead line from the nearest MITS node, with no redundancy, the substation is rated at less than 1320MW, and there is no other generation or demand connecting to this circuit, then:

a) The local substation tariff shall be as published in accordance with paragraph 14.15.122. For this example the local substation tariff will be £0.133/kW; and

- Running the transport model with a local circuit expansion factor of 10.0 applied to the 10km of overhead line connecting CLUN1S to the nearest MITS node and the wider circuit expansion factors applied to all other circuits, gives a local nodal maginal cost of 100MWkm. This is the additional MWkm costs associated with the node’s local assets. Applying the expansion constant of £10.07/MWkm and local security factor of 1.0 and dividing by 1000 gives a local circuit tariff of £1.007/kW.

Adjustment Tariff

- We now need to calculate the Adjustment Tariff.  This is calculated by taking the Adjustment Revenue and dividing  this by the Chargable Generation Capacity (as per to 14.14.5 (vii) (h)) create a £/kW figure

Assuming annual average transmission charges paid by Generators is due to fall below €0, The Company will add Adjustment Revenue to ensure that compliance with the Limiting Regulation is maintained – in this example let us assume it is £260m and the GB-wide generation Chargable Capacity is 60GW (60,000,000kW). This would mean the non-locational Adjustment Tariff would be calculated as;

Adjustment Tariff AdjTariff= Adjustment RevenueChargable Capacity

Adjustment Tariff AdjTariff= £260m60,000,000kW

Adjustment Tariff AdjTariff=£4.33/kW

- Therefore the charges for thermal plant with a TEC of 100MW and an ALF of 60%, connecting at CLUN1S is:

= Wider Peak Security Tariff * PS Flag * TEC	= 1.071 * 1 * 100,000

= Wider Year Round Shared Tariff * ALF * TEC	= 6.245 * 0.6 * 100,000

= Wider Year Round Not-Shared Tariff * TEC	= 1.309 * 100, 000

= Local substation Tariff * TEC			= 0.133 * 100,000

= Local circuit Tariff * TEC				= 1.007 * 100,000

= Adjustment Tariff * TEC				=  4.33 * 100,000

(effectively, £11597/kW * 100,000kW = £1,159,700)

(viii)  Alternatively, if we assume that the generator connecting at CLUN1S is an intermittent wind generation plant (instead of a thermal plant) with a TEC of 100MW, PS Flag of 0 and an ALF of 30%, then the charges payable will be –

= Wider Peak Security Tariff * PS Flag * TEC 	= 1.071 * 0 * 100,000

= Wider Year Round Shared Tariff * ALF * TEC	= 6.245 * 0.3 * 100,000

= Wider Year Round Not-Shared Tariff * TEC	= 1.309 * 100, 000

= Local substation Tariff * TEC			= 0.133 * 100,000

= Local circuit Tariff * TEC				= 1.007 * 100,000

= Adjustment Tariff * TEC				= 4.33 * 100,000

(effectively, £8.653/kW * 100,000kW = £865,300)

# 14.24 Example: Calculation of Zonal Demand Locational Tariff

Let us consider all nodes in the same demand zone in this example

The table below shows an example output of the transport model comprising the node, the Peak Security and Year Round nodal marginal km of an injection at the node with a consequent withdrawal at the distributed reference node, the generation sited at the node, scaled to ensure total national generation = total national net demand and the net demand sited at the node.

Where the Demand (MW) is negative this indicates that the Demand node is Exporting rather than importing.

| Demand Zone | Node | Peak Security Nodal Marginal km | Year Round Nodal Marginal km | Demand (MW) |
| --- | --- | --- | --- | --- |
| 1 | A | 110 | 80 | 100 |
| 1 | B | 140 | 90 | 100 |
| 1 | C | 120 | 80 | 0 |
| 1 | D | 100 | 100 | -50 |
| 1 | E | 100 | 70 | 50 |
|  |  | Totals |  | 200 |

| Demand Zone | Node | Peak Security Nodal Marginal km | Year Round Nodal Marginal km | Net Demand (MW) |
| --- | --- | --- | --- | --- |
| 14 | ABHA4A | -77.25 | -230.25 | 127 |
| 14 | ABHA4B | -77.27 | -230.12 | 127 |
| 14 | ALVE4A | -82.28 | -197.18 | 100 |
| 14 | ALVE4B | -82.28 | -197.15 | 100 |
| 14 | AXMI40_SWEB | -125.58 | -176.19 | 97 |
| 14 | BRWA2A | -46.55 | -182.68 | 96 |
| 14 | BRWA2B | -46.55 | -181.12 | 96 |
| 14 | EXET40 | -87.69 | -164.42 | 340 |
| 14 | HINP20 | -46.55 | -147.14 | 0 |
| 14 | HINP40 | -46.55 | -147.14 | 0 |
| 14 | INDQ40 | -102.02 | -262.50 | 444 |
| 14 | IROA20_SWEB | -109.05 | -141.92 | 462 |
| 14 | LAND40 | -62.54 | -246.16 | 262 |
| 14 | MELK40_SWEB | 18.67 | -140.75 | 83 |
| 14 | SEAB40 | 65.33 | -140.97 | 304 |
| 14 | TAUN4A | -66.65 | -149.11 | 55 |
| 14 | TAUN4B | -66.66 | -149.11 | 55 |
|  |  | #### Totals |  | 2748 |

In order to calculate the gross demand tariff we would carry out the following steps:

- Change Negative Demand values to 0 (zero) , which in this example is Node D

| Demand Zone | Node | Peak Security Nodal Marginal km | Year Round Nodal Marginal km | Demand (MW) |
| --- | --- | --- | --- | --- |
| 1 | A | 110 | 80 | 100 |
| 1 | B | 140 | 90 | 100 |
| 1 | C | 120 | 80 | 0 |
| 1 | D | 100 | 100 | 0 |
| 1 | E | 100 | 70 | 50 |
|  |  | Totals |  | 250 |

- calculate the demand weighted nodal shadow costs

For this example zone this would be as follows:

| Demand Zone |  | Node | Peak Security Nodal Marginal km | Year Round Nodal Marginal km | Demand (MW) | Peak Security Demand Weighted Nodal Marginal km | Year Round Demand Weighted Nodal Marginal km |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A |  | 110 | 80 | 100 | 44 | 32 |
| 1 | B |  | 140 | 90 | 100 | 56 | 36 |
| 1 | C |  | 120 | 80 | 0 | 0 | 0 |
| 1 | D |  | 100 | 100 | 0 | 0 | 0 |
| 1 | E |  | 100 | 70 | 50 | 20 | 14 |
|  |  |  | Totals |  | 250 | 120 | 82 |

- sum the Peak Security and Year Round demand weighted nodal shadow costs to give zonal figures. For this example zone this is shown in the above table and is 120km for Peak Security background and 82km for Year Round background.

- i.) calculate the transport (locational) tariffs by multiplying the figures in (ii) above by -1. This changes the original Nodal Marginal Km for injecting (Generation) into Nodal Marginal Km for withdrawing (Demand). Then multiply by the expansion constant, the locational security factor and then  divide by 1000 to put into units of £/kW:

For this example zone, assuming an expansion constant of £10.07/MWkm and a locational security factor of 1.80:

a)	Peak Security tariff –

- (120km * £10.07/MWkm * 1.8) 	=	-£2.47/kW

1000

b) Year Round tariff -

- (82* £10.07/MWkm * 1.8) 	=	-£1.49/kW

1000

The Locational signal for Demand within this zone is negative for both Peak and Year Round, which indicates withdrawing at this part of the network, reduces total system flows.

ii.) A NHH locational demand element is calculated in accordance with the methodology given in 14.16.2.

- i.) We now need to calculate the Transmission Demand Residual Tariffs.  This is calculated by first taking the total revenue to be recovered from demand less the revenue which would be recovered through the demand locational and energy tariffs and revenue recovery through embedded export tariffs.

Assuming the total revenue to be recovered from gross GSP group demand=  £779m.   Assuming the total recovery from GSP group demand locational and energy tariffs is £140m, total recovery from embedded export tariffs is -£10m  the total revenue to be recovered through the Transmission Demand Residual Tariffs will be as follows:

£779m-£140m- -£10m=£649m

ii.) The total revenue to be recovered from the Transmission Demand Residual Tariffs as calculated in (i) above is then apportioned between the Charging Bands as set in 14.15.138 by the  sum of the annual energy consumption of the Final Demand Sites or Unmetered Supplies as appropriate allocated to a Charging Band as divided by the total annual energy consumption from all GB Final Demand Sites and Unmetered Supplies.

iii.) An example is as follows:

The total annual consumption of all GB Final Demand Sites and Unmetered Supplies is 100TWh.

The sum of the annual energy consumption of all Final Demand Sites in HV Charging Band 1 is 1TWh

Using the example of total revenue to be recovered through the Transmission Demand Residual Tariffs above of £649m, HV Charging Band 1 Final Demand Sites will be liable for 1% of this cost.

The annual cost per site will be £6,490,000N where N is the total number of sites in HV Charging Band 1.

If, in this example, N = 12,000 the annual charge per Final Demand Site in HV Charging Band 1 will be £540.83.

The Transmission Demand Residual Tariff per Final Demand Site in HV Charging Band 1 will be set as a daily charge. The annual charge, in this example £540.83, will be divided by the number of days in the charging year to deliver a Transmission Demand Residual Tariff (£/site/day).

In this example; £540.83365 days  or £1.48/site/day.

(vii)	The Transmission Demand Residual tariff is subject to further adjustment to allow for the minimum £0/kW gross demand charge.

14.25 Reconciliation of Demand Related Transmission Network Use of System Charges

This appendix illustrates the methodology used by The Company in the reconciliation of Transmission Network Use of System charges for demand. The example highlights the different stages of the calculations from the monthly invoiced amounts, right through to Final Reconciliation.

## Monthly Charges - HH and NHH

Suppliers provide half-hourly (HH) gross demand and embedded export forecasts and non-half-hourly (NHH) demand forecasts by BM Unit every quarter; The Company provides FDSC Forecasts and Unmetered Supply Volume Forecasts on behalf of Suppliers. An example of such forecasts and the corresponding monthly invoiced amounts, based on tariffs of £10.00/kW for gross demand, £5.00/kW for embedded export and 1.20p/kWh for energy consumption, is as follows:

HH and NHH Forecasts

|  | Forecast HH Triad Gross Demand HHDF (kW) | HH Gross Demand Monthly Invoiced Amount (£) | Forecast HH Triad Embedded Export HHEEF (kW) | HH Embedded Generation Monthly Invoiced Amount (£) | Forecast NHH Energy Consumption NHHCF(kWh) | NHH Monthly Invoiced Amount (£) | Net Monthly Invoiced Amount (£) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Apr | 12,000 | 10,000 | -600 | (250) | 15,000,000 | 15,000 | 24,750 |
| May | 12,000 | 10,000 | -600 | (250) | 15,000,000 | 15,000 | 24,750 |
| Jun | 12,000 | 10,000 | -600 | (250) | 15,000,000 | 15,000 | 24,750 |
| Jul | 12,000 | 10,000 | -600 | (250) | 18,000,000 | 19,000 | 28,750 |
| Aug | 12,000 | 10,000 | -600 | (250) | 18,000,000 | 19,000 | 28,750 |
| Sep | 12,000 | 10,000 | -600 | (250) | 18,000,000 | 19,000 | 28,750 |
| Oct | 12,000 | 10,000 | -600 | (250) | 18,000,000 | 19,000 | 28,750 |
| Nov | 12,000 | 10,000 | -600 | (250) | 18,000,000 | 19,000 | 28,750 |
| Dec | 12,000 | 10,000 | -600 | (250) | 18,000,000 | 19,000 | 28,750 |
| Jan | 7,200 | (6,000) | -600 | (250) | 18,000,000 | 19,000 | 12,750 |
| Feb | 7,200 | (6,000) | -600 | (250) | 18,000,000 | 19,000 | 12,750 |
| Mar | 7,200 | (6,000) | -600 | (250) | 18,000,000 | 19,000 | 12,750 |
| Total |  | 72,000 |  | (3,000) |  | 216,000 | 285,000 |

As shown, for the first nine months the Supplier provided a 12,000kW HH triad gross demand forecast, and hence paid HH gross demand monthly charges of £10,000 ((12,000kW x £10.00/kW)/12) for that BM Unit.  In January the Supplier provided a revised forecast of 7,200kW, implying a forecast annual charge reduced to £72,000 (7,200kW x £10.00/kW).  The Supplier had already paid £90,000, so the excess of £18,000 was credited back to the supplier in three £6,000 instalments over the last three months of the year.

The Supplier provided an embedded export triad forecast of -600kW and hence was paid an embedded export credit of £250 ((600kW x £5.00/kW)/12) for that BM Unit (For the avoidance of doubt, if the embedded export tariff is negative this will result in a debit).

The Supplier also initially provided a 15,000,000kWh NHH energy consumption forecast, and hence paid NHH monthly charges of £15,000 ((15,000,000kWh x 1.2p/kWh)/12) for that BM Unit.  In July the Supplier provided a revised forecast of 18,000,000kWh, implying a forecast annual charge increased to £216,000 (18,000,000kWh x 1.2p/kWh).  The Supplier had already paid £45,000, so the remaining £171,000 was split into payments of £19,000 for the last nine months of the year.

The right hand column shows the net monthly HH and NHH charges for the BM Unit.

Monthly Charge – TDR

The User shall not be required to submit forecasts of FDSC or Unmetered Supply Volume. The Company shall use the latest daily actual FDSC and Unmetered Supply Volume prior to the forecast as the basis of the forecast.

As an example, at the start of the year the supplier has sites in just three of the charging bands, as shown below. The April invoice is calculated as the annual liability divided by 12.

| Charging Band | TDR Quantity | Tariff | April Invoice |
| --- | --- | --- | --- |
| Band 1 | 240 Sites | £1/Site/Day | = 240 x 1 x 365 / 12 = £7,300 |
| Band 2 | 180 Sites | £2/Site/Day | = 180 x 2 x 365 / 12 = £10,950 |
| UMS | 4800kWh/day | £0.012/kWh | = 4800 x 0.012 x 365 / 12 = £1,752 |
|  |  | Total | £20,002 |

Each month The Company recalculates the forecast liability for the full year, based on any revision to the FDSC and Unmetered Supply Volume for the previous months, and the latest forecast for the remainder of the year.  The amount the supplier has already paid is deducted from this annual liability, and the balance is divided by the number of months left to bill.

## Initial Reconciliation (Part 1a – HH Demand)

The Supplier’s outturn HH triad gross demand, based on latest settlement data (and therefore subject to change in subsequent settlement runs), was 9,000kW.  The HH triad gross demand reconciliation charge is therefore calculated as follows:

HHD Reconciliation Charge 	= (HHDA  - HHDF) x £/kW Tariff

= (9,000kW - 7,200kW) x £10.00/kW

= 1,800kW x £10.00/kW

= £18,000

To calculate monthly interest charges, the outturn HHD charge is split equally over the 12-month period.  The monthly reconciliation amount is the monthly outturn HHD charge less the HH gross demand monthly invoiced amount.  Interest payments are calculated based on these monthly reconciliation amounts using Barclays Base Rate.

## Initial Reconciliation (Part 1b – Embedded Export)

The Supplier’s outturn HH triad embedded export, based on latest settlement data (and therefore subject to change in subsequent settlement runs), was 500kW.  The HH triad embedded export reconciliation charge is therefore calculated as follows:

HHEE Reconciliation Charge	= (HHEEA – HHEEF) x £/kW Tariff

= (-500kW - -600kW) x £5.00/kW

= 100kW x £5.00/kW

= £500

To calculate monthly interest charges, the outturn HHEE charge is split equally over the 12-month period.  The monthly reconciliation amount is the monthly outturn HHEE charge less the HH embedded generation monthly invoiced amount.  Interest payments are calculated based on these monthly reconciliation amounts using Barclays Base Rate.

## Initial Reconciliation (Part 2 – NHH Demand)

The Supplier's outturn NHH energy consumption, based on latest settlement data, was 17,000,000kWh.  The NHH energy consumption reconciliation charge is therefore calculated as follows:

NHHC Reconciliation Charge	= (NHHCA  - NHHCF) x p/kWh Tariff

100

= (17,000,000kWh - 18,000,000kWh) x 1.20p/kWh

100

= -1,000,000kWh x 1.20p/kWh

100

= -£12,000

The monthly reconciliation amount is equal to the outturn energy consumption charge for that month less the NHH monthly invoiced amount.  Interest payments are calculated based on these monthly reconciliation amounts using Barclays Base Rate.

## Initial Reconciliation (Part 3 - TDR)

The Supplier’s outturn Transmission Demand Residual charge is calculated for each month using FDSC and Unmetered Supply Volume based on latest settlement data as follows,

| Charging Band | Latest TDR Quantity | Tariff | April Charge | Original April Invoice | Initial Demand Reconciliation for April |
| --- | --- | --- | --- | --- | --- |
| Band 1 | 230 Sites | £1/Site/Day | = 230 x 1 x 30 days = £6,900 | £7,300 | -£400 |
| Band 2 | 200 Sites | £2/Site/Day | = 200 x 2 x 30 = £12,000 | £10,950 | £1,050 |
| UMS | 5000kWh/day | £0.012/kWh | = 5000 x 0.012 x 30 = £1,800 | £1,752 | £48 |
|  |  | Total | £20,700 | £20,002 | £698 |

If the Supplier’s TDR quantity was the same for every month of the year, the total TDR reconciliation charge would be:

TDR Reconciliation Charge	= [ (230-240) * 1 + (200-180) * 2 + (5000-4800) * 0.012 ] *365

= £11,826

The monthly reconciliation amount is the monthly outturn Transmission Demand Residual Charge less the monthly invoiced amount.  Interest payments are calculated based on these monthly reconciliation amounts using Barclays Base Rate.

On the above examples, the net initial TNUoS demand reconciliation charge (across HH, EE, NHH and TDR, but excluding interest) is therefore £18,000 + £500 - £12,000 + £11,826 = £18,326.

## Final Reconciliation

Finally, let us now suppose that after all final Settlement data has been received (up to 14 months after the relevant dates) as shown in the below table;

| Settlement Data item | Initial Value | Final Value |
| --- | --- | --- |
| HH triad gross demand | 9,000kW | 9,500kW |
| HH triad embedded export | -500kW | -550kW |
| NHH energy consumption | 17,000,000kWh | 16,700,000kWh |
| Band 1 | 230 Sites | 235 Sites |
| Band 2 | 200 Sites | 195 Sites |
| UMS | 5,000kWh/day | 5,100kWh/day |

This would mean the Final Reconciliation calculations would be.

Final HH Gross Demand	= (9,500kW - 9,000kW) x £10.00/kW

Reconciliation Charge	= £5,000

Final HH Embedded Export		= (-550kW - -500kW) x £5.00/kW

Reconciliation Charge		= -£250

Final NHH Reconciliation Charge 	= (16,700,000kWh – 17,000,000kWh) x 1.20p/kWh

100

= -£3,600

FDSC Charging Band 1	= (235 Sites – 230 Sites) x £1/Site/Day x 365 days = £150

FDSC Charging Band 2 	= (195 Sites – 200 Sites) x £2/Site/Day x 365 Days = -£300

UMS Charging Band= (5,100kWh/day – 5,000kWh/day) x 0.012/kWh x 365 Days = -£36

Consequently, the net final TNUoS demand reconciliation charge will be £5,000  -£250  -£3,600 + £150 - £300 +- £36 = £1,036

Monthly reconciliation amounts are calculated in a similar way as for the Initial Reconciliation, being (i) for HH Demand and Embedded Export, the outturn annual charge divided by 12, and for NHH Demand and TDR it is the outturn charge for the month; less (ii) the amount already invoiced for that month (including through the Initial Reconciliation). Interest payments are calculated based on these monthly reconciliation amounts using Barclays Base Rate.

Terminology:

HHDA = The Supplier's outturn half-hourly metered Triad Gross Demand (kW) for the demand zone concerned.

HHDF = The Supplier's forecast half-hourly metered Triad Gross Demand (kW) for the demand zone concerned.

HHEEA = The Supplier's outturn half-hourly metered Triad Embedded Export (kW) for the demand zone concerned.

HHEEF = The Supplier's forecast half-hourly metered Triad Embedded Export (kW) for the demand zone concerned.

NHHCA = The Supplier's outturn non-half-hourly metered daily Energy Consumption (kWh) for the period 16:00 hrs to 19:00 hrs inclusive (i.e. Settlement Periods 33 to 38) from April 1st to March 31st, for the demand zone concerned.

NHHCF = The Supplier's forecast non-half-hourly metered daily Energy Consumption (kWh) for the period 16:00 hrs to 19:00 hrs inclusive (i.e. Settlement Periods 33 to 38) from April 1st to March 31st, for the demand zone concerned.

£/kW Tariff  = The £/kW Gross Demand or Embedded Export Tariff as shown in Schedule 1 of The Statement of Use of System Charges for the demand zone concerned.

p/kWh Tariff  = The Energy Consumption Tariff shown in Schedule 1 of The Statement of Use of System Charges for the demand zone concerned.

# 14.26 Classification of parties for charging purposes

In the event of any conflict between this Appendix and the main text within this Statement, the main text within the Statement shall take precendence.

In the following diagrams, the parties liable for Transmission Network Use of System charges are outlined in red.

| SUPPLIER |  |
| --- | --- |
| Supplier Use of System Agreement #### Supplier BMUn Supplier ##### BMU1 |  |
| Demand Charges See 14.17.13 and 14.17.18. | Generation Charges None. |

| POWER STATION WITH A BILATERAL CONNECTION AGREEMENT |  |
| --- | --- |
| Additional Load BMU Bilateral Connection Agreement Appendix C Station Load BMU #### Gen Unit BMUn Gen Unit ##### BMU1 |  |
| Demand Charges See 14.17.18. | Generation Charges See 14.18.1 i) and 14.18.3 to 14.18.9 and 14.18.18. For generators in positive zones, see 14.18.10 to 14.18.12. For generators in negative zones, see 14.18.13 to 14.18.17. |

| PARTY WITH A BILATERAL EMBEDDED GENERATION AGREEMENT |  |
| --- | --- |
| Bilateral Embedded Generation Agreement Appendix C Station Load BMU #### Gen Unit BMUn Gen Unit ##### BMU1 Additional Load BMU |  |
| Demand Charges See 14.17.14, 14.17.15 and 14.17.18. | Generation Charges See 14.18.1 ii). For generators in positive zones, see 14.18.3 to 14.18.12 and 14.18.18. For generators in negative zones, see 14.18.3 to 14.18.9 and 14.18.13 to 14.18.18. |

# 14.27 Transmission Network Use of System Charging Flowcharts

The following flowcharts illustrate the parties liable for Demand and Generation TNUoS charges and the calculation of those charges.

In the event of any conflict between this Appendix and the main text within this Statement, the main text within the Statement shall take precedence.

## Demand Charges

Generation

Charges

# 14.28 Example: Determination of The Company’s Forecast for Demand Charge Purposes

The Company will use the latest available settlement data for calculation of HH demand and NHH energy consumption forecasts for the Financial Year.

The Financial Year runs from 1st April to 31st March inclusive and for the purpose of these examples the year April 2005 to March 2006 is used.

Where the preceding year’s settlement data is not available at the time that The Company needs to calculate its forecast, The Company will use settlement data from the corresponding period in Financial Year minus two unless indicated otherwise.

All values used with the examples are purely for illustrative purposes only.

## i)	Half-Hourly (HH) Metered Demand Forecast – Existing User

At the time of calculation of a HH demand forecast before the relevant Financial Year (approximately 10th March), The Company will be aware at a system level which dates will be used for the determination of Triad.  However, The Company may not have settlement data at a User level if the Triad dates were to span a period that includes the latter half of February.

When undertaking forecasting before the relevant Financial Year, The Company will use the User’s Triad demand for the previous year for its forecast providing it holds User settlement data for this period, thus:

F =	T

where:

F =	Forecast of User’s HH demand at Triad for the Financial Year

T =	User’s HH gross demand and embedded export at Triad in Financial Year minus one

Where The Company determines its forecast within a Financial Year:

F = T * D/P

where:

F =	Forecast of User’s HH demand at Triad for the Financial Year

T =	User’s HH gross demand and embedded export at Triad in the preceding Financial Year

D =	User’s average half hourly metered gross demand and embedded export in Settlement Period 35 in the Financial Year to date

P =	User’s average half hourly metered gross demand and embedded export in Settlement Period 35 for the period corresponding to D in the preceding Financial Year

Where The Company determines its forecast before the relevant Financial Year and User settlement data for the Triad period is not available, The Company shall apply the formula immediately above (within year forecast) but substitute the following definitions for the values T, D, and P:

T =	User’s HH gross demand and embedded export at Triad in the Financial Year minus two

D =	User’s average half hourly metered gross demand and embedded export in Settlement Period 35 in the Financial Year minus one, to date

P =	User’s average half hourly metered gross demand and embedded export in Settlement Period 35 for the period corresponding to D in the Financial Year minus two

Example (where User settlement data is not yet available for the Triad period):

The Company calculates a HH demand forecast on the above methodology at 10th March 2005 for the period 1st April 2005 to 31st March 2006.

Gross demand:

F = 10,000 * 13,200 / 12,000

F = 11,000 kW

where:

T =	10,000 kW (period November 2003 to February 2004)

D =	13,200 kW (period 1st April 2004 to 15th February 2005#)

P =	12,000 kW (period 1st April 2003 to 15th February 2004)

# Latest date for which settlement data is available.

Embedded export:

F = -280 * -300 / -350

F = -240 kW

where:

T =	-280 kW (period November 2003 to February 2004)

D =	-300 kW (period 1st April 2004 to 15th February 2005#)

P =	-350 kW (period 1st April 2003 to 15th February 2004)

# Latest date for which settlement data is available.

## ii)	Non Half-Hourly (NHH) Metered Energy Consumption Forecast – Existing User

F = E * D/P

where:

F =	Forecast of User’s NHH metered energy consumption for the Financial Year

E =	User’s summed NHH energy consumption over the hours 16:00 to 19:00 for each day in the preceding Financial Year

D =	User’s summed NHH energy consumption for the hours 16:00 to 19:00 for each day for the Financial Year to date

P =	User’s summed NHH energy consumption for the hours 16:00 to 19:00 for each day for the period corresponding to D in the preceding Financial Year

Example:

The Company calculates a NHH energy consumption forecast on the above methodology at 10th June 2005 for the period 1st April 2005 to 31st March 2006.

F =	50,000,000 * 4,400,000 / 4,000,000

F =	55,000,000 kWh

where:

E =	50,000,000 kWh (period 1st April 2004 to 31st March 2005)

D =	4,400,000 kWh (period 1st April 2005 to 15th May 2005#)

P =	4,000,000 kWh (period 1st April 2004 to 15th May 2004)

# Latest date for which settlement data is available

Where forecasting before the relevant Financial Year concerned, The Company would in the above example use values for E and P from Financial Year 2003/04 and D from Financial Year 2004/05.

## iii)	Half-Hourly (HH) Metered Demand Forecast – New User

F =	M * T/W

where:

F = 	Forecast of User’s HH metered gross demand and embedded export at Triad for the Financial Year

M =	User’s HH average weekday period 35 demand for the last complete month for which settlement data is available

T =	Total system HH gross demand and embedded export at Triad in the preceding Financial Year

W =	Total system HH average weekday Settlement Period 35 metered demand for the corresponding period to M for the preceding year

Example:

The Company calculates a HH demand forecast on the above methodology at 10th September 2005 for a new User registered from 10th June 2005 for the period 10th June 2004 to 31st March 2006.

Gross demand:

F =	1,000 * 17,000,000 / 18,888,888

F =	900 kW

where:

M =	1,000 kW (period 1st July 2005 to 31st July 2005)

T =	17,000,000 kW (period November 2004 to February 2005)

W =	18,888,888 kW (period 1st July 2004 to 31st July 2004)

Embedded export:

F =	-150 * -7,200,000 /- 6,000,000

F =	-180 kW

where:

M =	-150 kW (period 1st July 2005 to 31st July 2005)

T =	-7,200,000 kW (period November 2004 to February 2005)

W =	-6,000,000 kW (period 1st July 2004 to 31st July 2004)

## iv)	Non Half Hourly (NHH) Metered Energy Consumption Forecast – New User

F =	J + (M * R/W)

where:

F =	Forecast of User’s NHH metered energy consumption for the Financial Year

J =	Residual part month summed NHH metered energy consumption for the hours 16:00 to 19:00 for each day where new User registration takes place other than on the first of a month

M =	User’s summed NHH metered energy consumption for the hours 16:00 to 19:00 for each day for the last complete month for which settlement data is available

R =	Total system summed NHH metered energy consumption for the hours 16:00 to 19:00 for each day for the period from the start of that defined under M but for the preceding year and until the end of that preceding Financial Year

W =	Total system summed NHH metered energy consumption for the hours 16:00 to 19:00 for each day for the period identified in M but for the preceding Financial Year

Example:

The Company calculates a NHH energy consumption forecast on the above methodology at 10th September 2005 for a new User registered from 10th June 2005 for the period 10th June 2005 to 31st March 2006.

F =	500 + (1,000 * 20,000,000,000 / 2,000,000,000)

F = 	10,500 kWh

where:

J =	500 kWh (period 10th June 2005 to 30th June 2005)

M =	1,000 kWh (period 1st July 2005 to 31st July 2005)

R =	20,000,000,000 kWh (period 1st July 2004 to 31st March 2005)

W = 	2,000,000,000 kWh (period 1st July 2004 to 31st July 2004)

# 14.29 Stability & Predictability of TNUoS tariffs

## Stability of tariffs

The Transmission Network Use of System Charging Methodology has a number of elements to enhance the stability of the tariffs, which is an important aspect of facilitating competition in the generation and supply of electricity.  This appendix seeks to highlight those elements.

Each node of the transmission network is assigned to a zone, these zones are themselves fixed.  The result of this is to dampen fluctuations that would otherwise be observed at a given node caused by changes in generation, demand, and network parameters.  The criteria used to establish generation zones are part of the methodology and are described in Paragraph 14.15.42.

In addition to fixing zones, other key parameters within the methodology are also fixed for the duration of the price control period or annual changes restricted in some way.  Specifically:

- the expansion constant, which reflects the annuitised value of capital investment required to transport 1MW over 1km by a 400kV over-head line, changes annually according to TOPI. The other elements used to derive the expansion constant are only reviewed at the beginning of a price control period to ensure that it remains cost-reflective.  This review will consider those components outlined in Paragraph 14.15.59 to Paragraph 14.15.69.

- the expansion factors, which are set on the same basis of the expansion constant and used to reflect the relative investment costs in each TO region of circuits at different transmission voltages and types, are fixed for the duration price control.  These factors are reviewed at the beginning of a price control period and will take account of the same factors considered in the review of the expansion constant.

- the locational security factor, which reflects the transmission security provided under the NETS Security and Quality of Supply Standard, is fixed for the duration of the price control period and reviewed at the beginning of a price control period.

- the Transmission Demand Residual Charging Bands which are used in setting Transmission Demand Residual Tariffs are fixed for the duration of the Onshore Transmission Owner price control period and reviewed at the beginning of a price control period.

## Predictability of tariffs

The Company revises TNUoS tariffs each year to ensure that these remain cost-reflective and take into account changes to allowable income under the price control and TOPI.  There are a number of provisions within the ESO Licence and the CUSC designed to promote the predictability of annually varying charges.  Specifically, The Company is required to give the Authority 150 days notice of its intention to change use of system charges together with a reasonable assessment of the proposals on those charges; and to give Users 2 months written notice of any revised charges.  The Company typically provides an additional months notice of revised charges through the publication of “indicative” tariffs.  Shorter notice periods are permitted by the framework but only following consent from the Authority.

These features require formal proposals to change the Transmission Use of System Charging Methodology to be initiated in October to provide sufficient time for a formal consultation and the Authority’s veto period before charges are indicated to Users.

More fundamentally, The Company also provides Users with the tool used by The Company to calculate tariffs. This allows Users to make their own predictions on how future changes in the generation and supply sectors will influence tariffs. Along with the price control information, the data from the Electricity Ten Year Statement (ETYS), and Users own prediction of market activity, Users are able to make a reasonable estimate of future tariffs and perform sensitivity analysis.

To supplement this, The Company also prepares an annual information paper that provides an indication of the future path of the locational element of tariffs over the next five years.  This analysis is based on data included within the  Electricity Ten Year Statement (ETYS).  This report typically includes:

- an explanation of the events that have caused tariffs to change;

- sensitivity analysis to indicate how generation and demand tariffs would change as a result of changes in generation and demand at certain points on the network that are not included within the SYS;

- an assessment of the compliance with the zoning criteria throughout the five year period to indicate how generation zones might need to change in the future, with a view to minimising such changes and giving as much notice of the need, or potential need, to change generation zones; and

- a complete dataset for the DCLF Transport Model developed for each future year, to  allow Users to undertake their own sensitivity analysis for specific scenarios that they may wish to model.

The first year of tariffs forecasted in the annual information paper are updated twice throughout the proceeding Financial Year as the various Transport and Tariff model inputs are received or amended.  These updates are in addition to the Authority 150 days notice and publication of “indicative” tariffs.

The parameters used in the calculation of generation cap (in paragraph 14.15.5) will be published along with the forecast and confirmed values in the Tariff Information Paper which is produced in compliance with Condition 5 (of the NGC’s proposed GB electricity transmission use of system charging methodology - the Authority’s decisions document March 2005 80/5).

In addition, The Company will, when revising generation charging zones prior to a new price control period, undertake a zoning consultation that uses data from the latest information paper.  The purpose of this consultation will be to ensure tariff zones are robust to contracted changes in generation and supply, which could be expected to reduce the need for re-zoning exercises within a price control period.

The calculation, as undertaken by The Company, of the Charges for Physical Assets required for Connection when setting TNUoS Charges for a Charging Year.

To aid in the transparency and understanding of the setting of TNUoS Tariffs, at the same time as The Company publishes the draft and final TNUoS Charges for a Charging Year, The Company shall publish the details and components applied in the above calculation, the figures attributed to these and the output of the calculations as provided for in the proforma calculation schedule attached at Schedule 1 to this CUSC Section 14. The output shall be published in the form as set out in Schedule 1 to this CUSC Section 14.

Guidance on the Calculation of the Charges for Physical Assets required for Connection when setting TNUoS Charges for a Charging Year.

To aid in the transparency and understanding of the setting of TNUoS Tariffs in each Charging Year, and in any event no later than the date The Company publishes the draft TNUoS Charges for the following Charging Year, The  Company shall publish guidance on how it will undertake the calculation to set TNUoS tariffs in compliance with the Limiting Regulation for that following Charging Year and when assessing compliance following the conclusion of that Charging Year.

# Section 2 – The Statement of the Balancing Services Use of System Charging Methodology

# 14.30 Principles

- The ESO Licence allows The Company to derive revenue through the Balancing Services Use of System (BSUoS) Charges.  This statement explains the methodology used in order to calculate the BSUoS Charges.

- The Company keeps the electricity system in balance (energy balancing) and maintains the quality and security of supply (system balancing).  Users pay for the cost of these through BSUoS Charges.

- All Users excluding those Users with valid Declarations, Interconnectors and Virtual Lead Parties are liable for Balancing Services Use of System Charges based on their Final Demand in each Settlement Day.

- BSUoS Charges are calculated on a fixed price basis as described in Section 14.31.

- BSUoS Charges comprise the following costs:

- The Total Costs of the Balancing Mechanism

- Total Balancing Services Contract costs

- Internal Expenditure (as defined in the ESO Licence)

- RAV Returns (as defined in the ESO Licence)

- Costs associated with contracting for and developing Balancing Services

- Adjustments

- Costs invoiced to The Company associated with Manifest Errors and Special Provisions.

- All costs under Relevant Contracts awarded through the Onshore Tender Process.

# 14.31 Calculation of the Daily Balancing Services Use of System Charges

## Calculation of the Daily Balancing Services Use of System Charges

- BSUoS Charges are calculated and levied for each Settlement Day based on the Fixed BSUoS Price.

- The Fixed BSUoS Price will be the same for all Settlement Days within the same Fixed Price Period, unless a revised Fixed BSUoS Price comes into effect as outlined in Paragraph 14.31.15.

- The Fixed BSUoS Price is calculated using forecast average Total BSUoS Costs.

Calculation of forecast Total BSUoS Costs

- The forecast average Total BSUoS Costs for a Fixed Price Period t (fBSUoSTOTt) are calculated using the following formula:

fBSUoSTOTt= fBSUoSEXTt+ fBSUoSINTt + kbt

Where:

fBSUoSTOTt = forecast average Total BSUoS Costs associated with BSUoS for the Fixed Price Period t

fBSUoSEXTt = forecast External BSUoS Costs. The terms which make up External BSUoS Costs are set out as term EXTt in part E of condition F1 of the ESO Licence

fBSUoSINTt = forecast Internal BSUoS Costs. The terms which make up Internal BSUoS Costs are set out  as term INTt in Part D

of condition F1 of the ESO Licence.

kbt = the amount included in the Fixed BSUoS Price for Fixed Price Period t, to account for any over or under recovery from previous Fixed Price Periods. For the first Fixed Price Period (t=0) this amount is zero.

- The amount included to account for over or under recovery, described in Paragraph 14.31.4, is calculated by subtracting the forecast revenue collected via the Fixed BSUoS Price allocated to Total BSUoS Costs in Fixed Price Periods prior to Fixed Price Period t, from the latest forecast of Total BSUoS Costs, as calculated by applying the latest values to the formula in Paragraph 14.31.4, for those same Fixed Price Periods prior to Fixed Price Period t. This is inclusive of any revenue collected from Final Reconciliation (RF) BSUoS Charges, pursuant to Paragraph 14.32.4.

- The process described in Paragraph 14.31.5 does not apply to Final Reconciliation (RF) BSUoS Charges for Settlement Days which occur before the start of the first Fixed Price Period, on 1st April 2023.

Calculating the Fixed BSUoS Price

- The Fixed BSUoS Price for each Fixed Price Period t is calculated by the following formula:

FixedBSUoSPt= fBSUoSTOTtfTQMt+fSGQMt

Where:

FixedBSUoSPt = the Fixed BSUoS Price for the Fixed Price Period t

fBSUoSTOTt = forecast average Total BSUoS Costs for the Fixed Price Period t

fTQMt = forecast of the total Transmission Connected Site BM Unit Metered Volume during the Fixed Price Period t

fSGQMt = forecast of the total Gross Demand BM Unit Volume during the Fixed Price Period t

Notice of Fixed BSUoS Price

- Users are given a notice period of at least three months of the Fixed BSUoS Price, for the next Fixed Price Period.

Calculation of BSUoS Charges for BM Units

- The Total BSUoS Charges for an individual BM Unit are calculated for each Settlement Day, denoted by d, based on the BM Unit’s metered volumes for each Settlement Period, denoted by j, and based on the Fixed BSUoS Price.

- Supplier BM Units and Exempt Export BM Units, prefixed by i, have their Total BSUoS Charges calculated by the following formula:

BSUoSTOTid=FixedBSUoSPd xj∈dSGQMij

Where:

BSUoSTOTid = the Total BSUoS Charges for a Supplier or Exempt Export BM Unit i, on Settlement Day d

FixedBSUoSPd = the Fixed BSUoS Price on Settlement Day d.

SGQMij = the Gross Demand Supplier or Exempt Export BM Unit Volume for a Supplier or Exempt Export BM Unit i for Settlement Period j

- All Transmission Connected BM Units, prefixed by m, have their Total BSUoS Charges calculated by the following formula:

BSUoSTOTmd=FixedBSUoSPd xj∈dTQMmj

Where:

BSUoSTOTmd = the Total BSUoS Charges for a Transmission Connected BM Unit m on Settlement Day d

FixedBSUoSPd = the Fixed BSUoS Price on Settlement Day d.

TQMmj = the total Transmission Connected Site BM Unit Metered Volume for a Transmission Connected BM Unit m for Settlement Period j

Calculation of BSUoS Charges for a BSUoS liable customer

- The BSUoS Charges for a BSUoS liable customer c on a Settlement Day d will be calculated by the following formula:

BSUoSTOTcd= i,m∈c(BSUoSTOTid+BSUoSTOTmd)

Where:

BSUoSTOTcd = the Total BSUoS Charges for a BSUoS liable customer c on Settlement Day d

BSUoSTOTid = the Total BSUoS Charges for a Supplier or Exempt Export BM Unit i on Settlement Day d

BSUoSTOTmd = the Total BSUoS Charges for a Transmission Connected BM Unit m on Settlement Day d

Definition of the BSUoS charging base

- BSUoS liability is based on a User’s Final Demand.

## Interconnector BM Units

- BM Unit and Trading Units associated with Interconnectors, including those associated with the Interconnector Error Administrator, are not liable for BSUoS Charges. BM Units, including Secondary BM Units, which are associated with Virtual Lead Parties are not liable for BSUoS Charges.

Issuing a revised Fixed BSUoS Price

- If before or during a Fixed Price Period, The Company forecasts that it will neither recover sufficient funds through BSUoS Charges nor will it hold sufficient funds in the Industry BSUoS Fund and the BSUoS Working Capital Facility to meet balancing costs during that Fixed Price Period, The Company has the right to set a revised Fixed BSUoS Price for the entirety of or remainder of that Fixed Price Period.

- In the case that The Company needs to issue a revised Fixed BSUoS Price, then The Company will use reasonable endeavours to consult on the revised Fixed BSUoS Price prior to its application. The Company will provide a minimum notice of 5 Business Days before the commencement of a revised Fixed BSUoS Price.

- The revised Fixed BSUoS Price, described in Paragraph 14.31.15, is determined by the following formula:

FixedBSUoSPrevs=fBSUoSTOTsfTQMs+fSGQMs

Where:

FixedBSUoSPrevs = The revised Fixed BSUoS Price for the number of Settlement Days s remaining in the Fixed Price Period

fBSUoSTOTs = 	forecast of remaining balancing costs for the number of Settlement Days s remaining in the Fixed Price Period. The Company would use reasonable endeavours to consult on this value, pursuant to Paragraph 14.31.16.

fTQMs = 	forecast of the total Transmission Connected Site BM Unit Metered Volume for the number of Settlement Days s remaining in the Fixed Price Period

fSGQMs = 	forecast of the total Gross Demand BM Unit Volume for the number of Settlement Days s remaining in the Fixed Price Period

- Pursuant to Paragraph 14.3 1.17, the revised Fixed BSUoS Price only covers costs for the remainder of the Fixed Price Period. The BSUoS Working Capital Facility would be built back up in subsequent Fixed Price Periods, via the Fixed BSUoS Prices calculated for those Fixed Price Periods.

# 14.32 Settlement of BSUoS

## Settlement and Reconciliation of BSUoS Charges

- There are two stages of the reconciliation of BSUoS Charges described below:

- Initial Settlement (SF)

- Final Reconciliation (RF)

## Initial Settlement of BSUoS

- The Company will calculate initial settlement (SF) BSUoS Charges in accordance with the methodology set out in section 14.31 above, using the latest available data, including data from the Initial Settlement Run and the Initial Volume Allocation Run.

## Reconciliation of BSUoS Charges

- Final Reconciliation will result in the calculation of a reconciled charge for each Settlement Day in the Financial Year.  The Company will calculate Final Reconciliation (RF) BSUoS Charges (with the inclusion of interest as defined in the CUSC) in accordance with the methodology set out in section 14.31 above, using the latest available data, including data from the Final Reconciliation Settlement Run and the Final Reconciliation Volume Allocation Run.

- Pursuant to Paragraph 14.31.5, any revenue collected or paid out by The Company as part of Final Reconciliation (RF) BSUoS Charges during a Fixed Price Period t will be considered as part of the revenue associated with that Fixed Price Period t, for the purposes of calculating the over or under recovery factor for subsequent Fixed Price Periods.

## Unavailability of Data

- If any of the elements required to calculate the BSUoS Charges in respect of any Settlement Day have not been notified to The Company in time for it to do the calculations then The Company will use data for the corresponding Settlement Day in the previous week.  If no such values for the previous week are available to The Company then The Company will substitute such variables as it shall, at its reasonable discretion, think fit and calculate Balancing Services Use of System Charges on the basis of these values. When the actual data becomes available a reconciliation run will be undertaken.

## Disputes

- If The Company or any customer identifies any error which would affect the total Balancing Services Use of System Charges on a Settlement Day then The Company will recalculate the charges following resolution of the error.  Revised invoices and/or credit notes will be issued for the change in charges, plus interest as set out in the CUSC.  The charge recalculation and issuing of revised invoices and/or credit notes will not take place for any day where the total change in the Balancing Services Use of System Charges are less than £2000.

## Relationship between the Statement of the Use of System Charging Methodology and the ESO Licence

- BSUoS Charges are made on a daily basis and as such  this Statement sets out the details of the calculation of such charges on a daily basis. Customers may, when verifying Balancing Services Use of System Charges refer to the ESO Licence which sets out the maximum allowed revenue that The Company may recover .

- The Company has, where possible and appropriate, attempted to ensure that acronyms allocated to variables within the Balancing Services charging software, and associated reporting, match with the acronyms given to those variables used within this statement.

## Balancing Services Use of System Acronym Definitions

For the avoidance of doubt “as defined in the BSC” relates to the Balancing and Settlement Code as published from time to time.

| Expression | Acronym | Unit | Definition |
| --- | --- | --- | --- |
| Balancing Mechanism Unit | BM Unit or BMU |  | As defined in the BSC |
| System Restoration Costs | SRCd | £ | The total costs associated with the provision of System Restoration (as defined in the Grid Code), including procuring, testing, warming, utilisation, capital contributions and payments for the cost of feasibility studies. This item also includes the costs of any validated claims made for costs of supporting the Electricity System Restoration Standard at Users’ sites, in accordance with CMP398. |
| External Balancing Services Use of System Costs | BSUoSEXTjd | £ | External System Operator (SO) Balancing Services Use of System Costs applicable to Settlement Period j for Settlement Day d |
| Internal Balancing Services Use of System Costs | BSUoSINTd | £ | Internal System Operator (SO) Balancing Services Use of System Costs applicable to Settlement Period j for Settlement Day d |
| Balancing Services Use of System tariff | BSUoStariffj | £/MWh | Tariff for the Balancing Services Use of System Charge applicable to Settlement Period j |
| Total Balancing Services Use of System Costs | BSUoSTOT | £ | Total Balancing Services Use of System Charges |
| BSUoS Working Capital Facility | BWCF | £ | As defined in Section 11 |
| Fixed BSUoS Price | FixedBSUoSPt | £/MWh | The Fixed BSUoS Price for the Fixed Price Period t, as defined in Section 11 |
| Revised Fixed BSUoS Price | FixedBSUoSPrevs | £/MWh | The revised Fixed BSUoS Price which applies for a number of Settlement Days s within a Fixed Price Period, as described in Paragraph 14.31.15 |
| Over/under recovery factor | kbt | £ | The factor included in the Fixed BSUoS Price for Fixed Price Period t, to account for any over or under recovery from previous Fixed Price Periods. For the first Fixed Price Period (t=0) this factor is zero. |
| BM Unit Metered Volume | QMij | MWh | As defined in the BSC |
| Gross Final Demand BM Unit Volume | SGQM | MWh | The Import data as at the Transmission System Boundary by Settlement Period for Supplier BM Units in respect to gross Final Demand volume (exclusive of all export volumes and import volumes from BM Units or Single Sites with a valid Declaration), multiplied by the applicable TLM |
| Supplier BM Unit |  |  | As defined in the BSC |
| Transmission Loss Multiplier | TLMij |  | As defined in the BSC |
| Transmission Connected Final Demand BM Unit Metered Volume | TQM | MWh | The BM Unit Metered Volume for Final Demand with a Bilateral Agreement with The Company, exclusive of export volumes, which is multiplied by the TLM |
| Transmission System Boundary |  |  | As defined in the Balancing and Settlement Code |
| Total System Energy Imbalance Volume | TQEIj | MWh | As defined in the Balancing and Settlement Code in force immediately prior to 1 April 2001 |
| Final Reconciliation Settlement Run |  |  | As defined in the BSC |
| Final Reconciliation Volume Allocation Run |  |  | As defined in the BSC |
| Initial Settlement Run |  |  | As defined in the BSC |
| Initial Volume Allocation Run |  |  | As defined in the BSC |
| Lead Party |  |  | As defined in the BSC |

CUSC Section 14 Schedule 1

Calculation of charges that fall within the Connection Exclusion

EU Regulation 838/2010

Schedule 1

The proforma of the form and content to be published for the purposes of the calculation in accordance with Paragraph 14.29.

| Project Name | Transmission Asset name | PARC/Non PARC | Annual Local Charge for company Transmission Asset | TEC | Tariff |
| --- | --- | --- | --- | --- | --- |

where an LDTEC Block Offer has been accepted (Profiled Block LDTEC) and a firm profile of capacity has been purchased.

where an LDTEC Indicative Block Offer has been accepted (Indicative Profiled Block LDTEC) and a right to future additional capacity up to a requested level has been purchased, the availability of which will be notified on a weekly basis in accordance with the CUSC.

† For simplicity, fixed reference node has been used instead of a distributed reference node.

http://www.nationalgrid.com/uk/Electricity/Charges/gbchargingapprovalconditions/5/